3GPP TSG-SA WG2 Meeting #152-e	S2-220XXXX

# Electronic meeting, 17 – 26 August 2022

Source:	Interdigital

Title:	KI #1: Evaluation on Layer2 UE to UE Relay

Document for:	Approval

Agenda item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract:  This PCR analyzed supported functions of each solution on layer2 UE to UE relay for key issue #1.

# 1.	Introduction

This PCR propose evaluation on Layer2 UE-to-UE Relay in following aspects of

- Service Authorization

- Relay Discovery and Selection

- Unicast Connection Setup

- QoS control

- Relay reselection

- PC5 link management

# 2.	Proposal

It is proposed to adopt the following changes into TR 23.700-33.

* * * First Change * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.X	Key Issue #1: Support of UE-to-UE Relay

## 7.X.1 	Evaluation on Layer2 UE-to-UE Relay

1.  Service Authorization

- For service authorization and policy provisioning, sol#8 proposes that PCF based service authorization and provisioning for U2U relay.

2. Relay Discovery and Selection

- For relay discovery and selection, three different kinds of solutions are included.

  - Model A discovery

  - Model B discovery

  - Integrated discovery with unicast connection setup procedure.

- In Model A discovery, solution #34 propose that relay UE forward announcement message from announcing UE with RSC specifying R-UE. Another solution for model A discovery, which are proposed in sol#3, sol#10, and sol#33, is that relay UE send its own announcement message with list of UEs in proximity and it requires periodic UE and Relay UE’s announcement to maintain up to date list. Sol #33, sol #10 and sol #3 include the L2 ID of the UEs in proximity in the announcement message. The Announcement message size varies depending on the number of UEs in proximity.

- In Model B discovery, which are proposed in sol#1, sol#10, sol#30, and sol#34, relay UE forwards solicitation (and response) messages from source UE to target UE (and vice-versa). Sol#33 propose that relay UE responds to the solicitation message from source UE when target UE is in the list of UE of proximity managed by relay UE.

  - For Model B, Sol#1, sol#10, sol #33 and Sol#34 propose that source UE handles the relay selection. For sol#1, sol#10, and sol#34, this implies that the target UE replies to all solicitation messages that it received while sol#30 propose that target UE handles the relay selection. For sol#1, sol#10, and sol#34, doing the relay selection at the source UE implies more PC5 signaling messages exchanged between the UEs and for sol#33, all relays receiving the solicitation message and having this UE in their list of proximity UEs send a response message.

- As another option, sol#1 and sol#13 support integrated discovery into PC5 unicast link establishment for the target UE with relay indication.

- In TS23.303 and TS23.304, Model B discovery can be used for discovering other UEs supporting a specific service, but current proposed solutions of sol#1, sol#33, sol#34 only support discover a UE with target UE ID.

3. For unicast connection setup

- In TS23.304, UE oriented L2 link establishment and ProSe service oriented L2 link establishment are defined.

- For UE oriented L2 link establishment, Sol#13, sol#30, sol#33, and sol#34 are proposed. They propose E2E DCR message can be transmitted in unicast manner over established PC5 link between source UE and relay and target UE and relay with assumption that discovery procedure among source UE, relay UE, and target UE performed before. In addition, Sol#1 and Sol#13 propose also that E2E DCR is sent in broadcast manner.

  - Before E2E unicast connection setup procedure, Sol#30, sol#33, and sol#34 assumed that PC5 link between source UE and relay UE shall be established and also propose that PC5 link between target UE and relay UE shall be triggered by relay UE based on the information of target UE included in DCR message. This requires that the Relay parses the DCR message.

  - Sol#13 propose that PC5 link between target UE and relay UE is triggered by target UE.

  - Sol #13 additionally supports the integrated UE discovery with user-oriented method, i.e. source UE sends a broadcast DCR message including the target UE user info.

- For ProSe service oriented L2 link establishment, sol#13 is proposed. Sol#13 propose E2E DCR message with integrated discovery with relay indication. Source UE send DCR to Relay UE and Relay UE will forward DCR in broadcast manner. After receiving E2E DCR, a UE which want to establish E2E link with the source UE for the indicated ProSe service will trigger PC5 link establishment with relay UE and respond DCA to source UE.

  - Sol#13 also supports ProSe service oriented L2 link establishment after discovery procedure run. In this case, the source UE sends the DCR message with the destination L2 ID set to the discovered relay L2 ID.

- For E2E link establishment, sol#13, 30, 33, and #34 assume that end to end security established and handled by PDCP layer before DCA from target UE to source UE and the details shall be scope of SA3.

- After successful E2E link establishment, sol#13, sol#30, and sol#33 assumed that adaptation layer will be used for routing messages between source UE and target UE via Relay UE. sol#3 proposed detail frame format for L2 U2U relay without adaptation layer.

NOTE: Adaptation layer support: RAN has defined architecture for L2 U2U Relay and introduced adaptation layer containing IDs used at Relay for packet forwarding. Details on adaption layer are to be defined by RAN.

4. QoS control,

- E2E PC5 QoS flows are setup between S-UE and T-UE. sol#4 propose that QoS control mechanism with E2E QoS parameter division by the UE-to-UE Relay. After the PC5 QoS parameters splitting for two PC5 links, how to handle the data in AS layer at each PC5 link is scope of RAN.

5. Relay reselection

- For Relay reselection, sol#7 propose that S-UE and T-UE negotiate relay reselection using link modification procedure through existing relay connections which allows fast E2E PC5 link establishment via the new Relay and reduce communication disruption as reselection, path switch, and security establishment procedure can be performed in existing link or skipped by reusing one in existing link.

6. PC5 link management

- For PC5 link management, in TS23.304, Link ID update, L2 link release, L2 link modification, and Keep alive procedures are defined.

- Sol#32 proposes to run the LIU procedure between the UE and the Relay, with the Adaptation layer ID added to the procedure. In the case where the application layer ID and/or IP address are changed, the Relay informs the end-to-end peer UE using the Link Modification procedure. Running the LIU procedure between the UE and the Relay, without involving the end-to-end peer UEs is possible at the condition that other IDs are used when a message is forwarded onto another per-hop link. (i.e. IDs sent in cleartext in the messages header are only sent on 1 PC5 link between the UE and the relay).

- L2 link release, L2 link modification, and Keep alive procedures are assumed to be reused in E2E PC5 link.

* * * End of Changes * * *