3GPP TSG-SA WG2 Meeting #152-e	S2-220XXXX

# Electronic meeting, 17 – 26 August 2022

Source:	Interdigital

Title:	KI #1: Evaluation on Layer3 UE to UE Relay

Document for:	Approval

Agenda item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract:  This PCR analyzed supported functions of each solution on layer3 UE to UE relay for key issue #1.

# 1.	Introduction

This PCR propose evaluation on Layer3 UE-to-UE Relay in following aspects of

- Service Authorization

- Relay Discovery and Selection

- Connection management using Layer3 UE-to-UE Relay for IP traffic

- Connection management using Layer3 UE-to-UE Relay for non-IP traffic

- PC5 connection setup with Relay UE

- Relay reselection

- PC5 link management

# 2.	Proposal

It is proposed to adopt the following changes into TR 23.700-33.

* * * First Change * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.X	Key Issue #1: Support of UE-to-UE Relay

## 7.X.2 	Evaluation on Layer3 UE-to-UE Relay

- Service Authorization

- For service authorization and policy provisioning, sol#8 propose that PCF based service authorization and provisioning for U2U relay.

- Relay Discovery and Selection

- For relay discovery and selection, three different kinds of solutions are included.

  - Model A discovery

  - Model B discovery

  - Integrated discovery with unicast connection setup procedure.

- In Model A discovery, solution #34 propose that relay UE forward announcement message from announcing UE with RSC specifying R-UE. Another solution for model A discovery, which are proposed in sol#3, sol#9, sol#10, sol#12, and sol#33, is that relay UE send its own announcement message with list of UEs in proximity and it requires periodic UE and Relay UE’s announcement to maintain up to date list. Sol #33, sol #10, sol#12, and sol #3 include the L2 ID of the UEs in proximity in the announcement message. The Announcement message size varies depending on the number of UEs in proximity.

- In Model B discovery, which are proposed in sol#1, sol#10, sol#30, and sol#34, relay UE forwards solicitation (and response) messages from source UE to target UE (and vice-versa). Sol#12 and Sol#33 propose that relay UE responds to the solicitation message from source UE when target UE is in the list of UE of proximity managed by relay UE.

- For Model B, Sol#1, sol#10, sol#12, sol #33 and Sol#34 propose that source UE handles the relay selection. For sol#1, sol#10, and sol#34, this implies that the target UE replies to all solicitation messages that it received while sol#30 propose that target UE handles the relay selection. For sol#1, sol#10, and sol#34, doing the relay selection at the source UE implies more PC5 signaling messages exchanged between the UEs and for sol#12 and sol#33, all relays receiving the solicitation message and having this UE in their list of proximity UEs send a response message.

- In TS23.303 and TS23.304, Model B discovery can be used for discovering other UEs supporting a specific service, but current proposed solutions of sol#1, sol#33, sol#34 only support discover a UE with target UE ID.

- As another option, sol#1 and sol#13 support integrated discovery into PC5 unicast link establishment for the target UE with relay indication.

- Connection management using L3 U2U Relay for IP traffic

- Sol#2, sol#5, sol#9, sol#11, and sol#12 propose that L3 U2U Relay will work as IP router and DNS server.

- Sol#2 specifies that Relay allocates IP address/prefix, sol#5 proposes to use link local IPv6 address of each UE and inform the IP address to the selected Relay, and sol#12 propose to allow both options.

- Sol#2, sol#9, and sol#12 propose to use DNS query to acquire IP address of peer UE and sol#5 propose to use ProSe layer signaling to acquire IP address of peer UE. Sol#2, sol#5, sol#9 and sol#12 propose to share IP address in protection via relay UE after security establishment.

- Sol#11 does not require extra procedure to share IP address between source UE and target UE but it allows sharing of IP addresses unprotected in DCR from U2U Relay to target UE.

- Sol#31 propose to add authorization of UE using security credential to share IP address only to authorized UE. Source UE may allow sharing IP address only to the authorized UE by sharing token. The detail procedure needs to be coordinated with SA3.

- Connection management using L3 U2U Relay for non-IP traffic

- Sol#6 propose that Relay UE manage a mapping between End-to-End connection and dedicated L2 ID of Relay. Dedicated L2 ID of Relay is used for nonIP PC5 link between Relay and source UE and between Relay and target UE

- PC5 connection setup with Relay UE

- Sol#2, sol#5 and sol#9 propose that each UE to setup PC5 connection and security establishment with L3 U2U relay after discovery before IP address retrieval for peer UE via relay UE.

- Sol#11 proposes to combine PC5 connection setup with relay UE and End to End connection setup for IP traffic or non-IP traffic.

- Sol#12 propose to setup PC5 connection and security establishment with relay UE when source UE is triggered for End-to-End connection setup and relay UE will initiate PC5 connection setup and security establishment with target UE.

- Sol#11 and sol#12 do not require for the UE to maintain PC5 connection with Relay before being triggered for E2E connection.

- Relay Reselection

- Sol#7 propose that S-UE and T-UE negotiate relay reselection using link modification procedure through existing relay connections. Sol#7 enables UEs to exchange IP address used at new Relay so that UE may resume end to end data traffic without further signaling to acquire IP address via new Relay.

- PC5 link management

- For PC5 link management, in TS23.304, Link ID update, L2 link release, L2 link modification, and Keep alive procedures are defined.

- Sol#32 proposes to run the LIU procedure between the UEs and the Relay when new IP address needed. Using LIU procedure enable sharing new IP address between source UE and target UE in protection and it support link local IPv6 address and IP address assignment by relay UE.

- Sol#6 proposes to run the LIU procedure to update Layer 2 ID of source UE and target UE and dedicated L2 ID of Relay for the new end to end connection.

- L2 link release, L2 link modification, and Keep alive procedures are assumed to be reused in E2E PC5 link.

* * * End of Changes * * *