3GPP TSG-SA WG2 Meeting #152-e	S2-220XXXX

Electronic meeting, XX – XX August 2022						 				(rev of S2-220XXXX)

Source:	Interdigital

Title:	Solution Update on Sol#22

Document for:	Approval

Agenda item:	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract of the contribution:  This PCR update solution #22.

1. Reason for Change

This PCR updates solution #22…

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-33.

* * * First Change - All text is new * * *

## 6.22	Solution #22: Direct Communication Path switching between PC5 and Uu

### 6.22.1	Description

This solution resolves Key Issue #3 for direct communication path switching between PC5 and Uu.

### 6.22.2	Procedures

#### 6.22.2.1	Direct Communication Path switching from PC5 to Uu

Figure 6.22.2.1-1: Path switching from PC5 to Uu

0.	UE1 and UE2 are provisioned with path switching policies (e.g. enabled/disabled, preferred interface, radio quality thresholds) by using the procedure as defined in clause 6.2 of TS 23.304 [3]. For each ProSe service which is allowed for path switching between PC5 and Uu, the UEs are provided with PC5 QoS parameters which will be allowed for path switching to Uu connection.

1.	UE1 and UE2 establish a PC5 connection. During this procedure, the Path switching parameters (e.g. path switching enabled/disabled) are exchanged. During PC5 connection setup, for ProSe service allowed path switching, UE1 and UE2 negotiate QoS within the provisioned PC5 QoS parameters to be allowed for path switching.

2.	UE1 informs UE2 about its PDU session status (e.g. available for PDU session establishment) by using the PC5 Keepalive procedure.

3.	UE2 informs UE1 about its PDU session status (e.g. available for PDU session establishment) by using the PC5 Keepalive procedure.

Editor's note:	Whether other ProSe signalling can be used for PDU session status exchange is FFS.

4.	If UE1 determines to switch path from PC5 to Uu e.g. based on PC5 signal level and UE2's PDU session status (i.e. PDU session establishment is available), UE1 sends a PC5 Path Switching Request message to UE2, including the path switching direction (i.e. PC5-to-Uu).

5.	UE2 may establish a PDU session or modify an existing PDU session for switched traffic. In the PDU session establishment request or modification request, UE2 include indication on path switch from PC5 to Uu in order to ensure 5GCN to assign QoS flows for the PC5 QoS flows of ProSe service to be path switched. Based on Path Switch indication, requested DNN, 5GCN may assign QoS flows for the applications allowing path switch.

NOTE. It is assumed that 5GCN has PCC rules for the applications to be supported Uu connection and PC5 connection and allowing path switch between Uu and PC5 connection for each UE.

6.	UE2 sends a PC5 Path Switching Response message to UE1, which includes the IP address associated to UE2's PDU session.

7.	UE1 may establish a PDU session or modify an existing PDU session for switched traffic. In the PDU session establishment request or modification request, UE2 include indication on path switch from PC5 to Uu in order to ensure 5GCN to assign QoS flows for the PC5 QoS flows of ProSe service to be path switched. Based on Path Switch indication, requested DNN, 5GCN may assign QoS flows for the applications allowing path switch.

8.	UE1 sends a PC5 Path Switching Ack message to UE2, which includes the IP address associated to UE1's PDU session. UE1 may send a PC5 Path Switching Abort message to abort the path switching procedure e.g. if the PDU session establishment/modification failed.

9.	After the step 8, UE1 and UE2 may release the PC5 connection and use Uu path for communication.

Editor's note:	How to support Ethernet traffic and Unstructured traffic is FFS.

#### 6.22.2.2	Direct Communication Path switching from Uu to PC5

Figure 6.22.2.2-1: Path switching from Uu to PC5

0.	UE1 and UE2 are provisioned with path switching policies (e.g. enabled/disabled, preferred interface, radio quality thresholds) by using the procedure as defined in clause 6.2 of TS 23.304 [3]. UE1 and UE2 use Uu path for communication.

1.	UE1 and UE2 establish a PC5 connection. In order to discover each other in PC5 discovery phase, discovery parameters e.g. Application ID, User Info ID,  may be assigned from application or derived in UE1 and UE2 using the information e.g. application ID, User ID of UE1, User ID of UE2, which are used during step 0. During PC5 connection setup, for ProSe service allowed path switching, UE1 and UE2 negotiate QoS within the provisioned PC5 QoS parameters to be allowed for path switching.

2.	UE1 sends a PC5 Path Switching Request message, including the path switching direction (i.e. Uu-to-PC5) and the IP address associated to UE1's PDU session.

3.	UE2 sends a PC5 Path Switching Response message, including the IP address associated to UE2's PDU session.

4.	UE1 and UE2 use PC5 path for communication.

5.	UE1 and UE2 may release/modify their PDU session.

Editor's note:	How to support Ethernet traffic and Unstructured traffic is FFS.

### 6.22.3	Impacts on services, entities and interfaces

UE:

-	Support path switch procedure.

PCF:

-	Support path switch policy provisioning.

* * * End of Changes * * *