3GPP WG SA WG2 Meeting #152E e-meeting	S2-2206156r04

Elbonia, Aug 17th – 26th, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI # 7, Solution #43: Update on FL member selection

Document for:	Discussion / Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes a update of procedures in Solution #43 for KI#7.

- Discussion

This contribution proposes a update procedure for Key Issue #7: 5GS Assistance to Federated Learning Operation in the FS_AIMLsys.

It proposes to update the Solution #43 for KI#7 to clarify that the "the FL member subscription data, indicating whether the network is allowed to expose UE related 5GC information" of step 2 of clause 6.43.2 means the user consent subscription information of FL member. Similar to the user consent checking before the data collection by NWDAF as defined in clause 6.2.9 of TS 23.288 [6], considering data privacy and/or security issues, the user consent checking is also required before the UE related network information or data analytics are exposed to the 3rd party. Therefore, before the AF subscribes to the UE related network information or data analytics from 5GC to determine the UE groups for FL operation, the AF should first retrieve the user consent subsciption information of the UE from UDM to determine whether the UE authorizes the network to expose the UE related network information or data analytics to the 3rd party. If the UE does not authorize the network to expose the UE related network information or data analytics to the 3rd party, the AF will not subscribe to the 5GC information of the UE, and the AF may not select the UE to join the FL operation.

2.	Proposal

It is proposed to adopt the following change in the TR 23.700-80.

* * * * First Change * * * *

6.43	Solution #43: FL member selection

6.43.1	Description

This solution is proposed to address Key Issue #7: 5GS Assistance to Federated Learning Operation.

This solution proposed that application server determine the UE groups for FL operation based on the user consent subscription information, UE's location, end to end delay and packet drop for the UE.

6.43.2	Procedures

6.43.2.1	Procedure for AF directly collection information for FL member selection

Figure 6.43.2-1: Procedure for FL member selection

1.	The AF retrieves the user consent subscription data, indicating whether the network is allowed to expose UE related 5GC information, for the UEs in the candidate list from the UDM (via NEF if AF is untrusted) by invoking Nudm_SDM_Get service operation.

2.[Optional]The AF requests the location reporting of the UEs from the AMF (via NEF if AF is untrusted) by invoking Namf_EventExposure_Subscribe (Location Reporting).

The AF should maintain the candidate UE list itself before FL member selection, and include the Target UE Identifier(s) for the UEs in the candidate list in the request as described in clause 5.6.7 of TS 23.501 [3], which is same for step 3 and step 4.

3.	The AF initiates direct notification of Qos Monitoring procedure for delay information for the UEs in the candidate list, as defined in steps 1a-5 of clause 6.4.2.1 of TS 23.548 [10].

4.	The AF requests user plane information, e.g. Throughput UL/DL, Packet transmission, Packet retransmission, for the UEs in the candidate list from UPF.

NOTE 1:	How to collect data from UPF will depend on the outcome of FS-UPEAS SID.

5.	The AF selects FL members based on user consent subscription data from UDM, UE location from AMF, and delay and packet drop information reporting from UPF.

NOTE 2:	AF may keep monitoring the information from AMF/UPF/UDM to update FL members.

6.43.3	Impacts on services, entities and interfaces

AF:

-	Supporting user consent subscription data collection from UDM.

-	Supporting FL member selection based on the information from AMF, UDM and UPF.

UDM:

-	Supporting user consent information of whether the network is allowed to expose UE related 5GC information.

UPF:

-	Supporting to report user plane information to AF.

* * * * End of change * * * *