SA WG2 Meeting #152e	S2-2205480r03

# August 17th – 26th, 2022; Elbonia          	  		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell	, Qualcomm, Huawei, HiSilicon, China Telecom, LG Electronics

Title: 	Conclusions for KI2: baseline

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution: Conclusions for KI2: baseline

# 1	Discussion

(Rapporteur’s input) Content empty on purpose to start from an unbiased (empty) content

# 2 Proposal

It is proposed to update TR 23700-17 as follows

# 8	Conclusions

## 8.1	Key Issue #1: Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities. This clause is FFS, waiting for BBF/CableLabs feedback to a LS

## 8.2	Key Issue #2: How to select a TNGF/N3IWF that supports the S-NSSAI(s) needed by the UE

### 8.2.1	How to select an N3IWF that supports the S-NSSAI(s) needed by the UE

Selection of N3IWF that supports the S-NSSAIs needed by the UE is enabled based on extended ANDSP configuration and NAS-based redirection (using Registration Reject) as defined by Solution 15 (as updated by S2-2205515).

### 8.2.2	How to select a TNGF that supports the S-NSSAI(s) needed by the UE

Selection of TNGF that supports the S-NSSAIs needed by the UE is enabled based on:

- UE based solutions where UE(s) to use WLAN SP policy to select  a SSID allowing to access to a TNGF that supports the slices the UE are willing to use and then leverage this information to try registering onto 5GC via TNGF

- The WLAN SP policy is extended with the indication of the set of slices associated to a SSID.

- AMF may trigger the UE Policy Association Establishment procedure to provide the UE with updated WLANSP if the selected TNGF does not support the slices requested by UE.

Editor’s note: Whether an AMF based approach where the AMF may redirect (via a registration reject) a UE towards a another SSID  is to be supported is FFS

No solution using the AMF redirection of the UE with a registration accept will be pursued in normative specifications