3GPP TSG-WG SA2 Meeting #152E e-meeting 	S2-2206392r062

Elbonia, August 17 – 26, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Nokia, China Mobile, China Telecom, Xiaomi, Tencent, Samsung

Title:	KI #1/#2, New Sol: QoS enhancement to support synchronized delivery of multiple QoS flows

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Propose a solution for QoS enhancement to support synchronized delivery of multiple QoS flows of the same application related to KI#1 and KI#2.

1. Introduction/Discussion

In SA#96, WT1 for FS_XRM was revised as following in SP-220705:

“WT#1: Enhancements for supporting multi-modality service:

-	Study whether and how to enable delivery of related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) with an application to the user at a similar time, using only control plane enhancements (i.e. QoS policy coordination, PCC enhancement for the 5GS). No N3/N9 user plane impact.

NOTE 1: Any parameters provided by 5GC to NG-RAN via the Control plane need to have a clear explanation about how they are intended to be used by NG-RAN.”

With this agreed clarification of the scope, this contribution resubmits the postponed 2204390r09 and provides further details on intended usage of the proposed parameters in NG-RAN.

SA#149e agreed to study Key Issue #1: Policy control enhancements to support multi-modality flows coordinated transmission for single UE. and Key Issue #2: Support the Application Synchronization and QoS Policy Coordination for Multi-modal Traffic among Multiple UEs. Both key issues require to study Potential enhancements to policy control to support coordination handling at the application and Whether and how interaction between an AF and the 5GS is needed for QoS policy coordination. Both key issues require delivering related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) for an application to the user at a similar time.

Observation 1: The QoS policy coordination is among multiple QoS flows of the same application in both single UE case and multiple UEs case,

Observation 2: Related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) for an application needs to be delivered to the user at a similar time in both single UE case and multiple UEs case. The exact synchronous delivery needs to be taken care by the end points and the application.

Observation 3: Related tactile and multi-modal data (e.g., audio, video and haptic data) for an application have also other dependencies with each other than just synchronous delivery. For instance, some flows may be critical to the usability of the whole application and if they fail successful delivery of other flows will not help. For example, a VR service has the basic video flow, enhancement video flow, audio flow and haptic data flow. Once the basic video flow cannot be delivered to the receiver, the basic experience of users already cannot be satisfied, so it is useless to deliver the related enhancement video flow, audio flow and haptic data flow to the receiver.

Which flows are the critical ones and which flows require synchronous delivery is only known to the application itself. However, the application would be able to express that information to the network (5GS) where the information could be translated to polices for admission control, QoS alignment and QoS fulfilment.

For multi-modality case, it is assumed that application itself knows the network delay budget for each service flow so that the flows do not need to wait for each other for playback or rendering and is in charge of synchronization between service flows. To make sure of the application synchronization, 5GS should provide required PDB for each service flow under the requirements from application.

AF could also provide alternative PDB(s) for each service flow, and association relationship between alternative PDB(s) of different service flows, so that each set of alternative PDBs also leads to desirable synchronous delivery time between the flows.In case the 5GS has to downgrade to an alternative PDB for one service flows, 5GS should choose  a corresponding alternative PDB also for all of the other service flows .

Step1: AF provides 5GS with alternative PDB(s) for each service flow, and association relationship between alternative PDB(s) of different service flows. The association relationship indicates for the chosen PDB for service flow#A, the PDBs that should be chosen for other service flows.

Figure 1 Association relationship between Alt. PDB of different service flows, provided by AF

Step 2：5GS binds service flow to QoS flow and creates Alternative QoS Profile for each QoS flow according to alternative PDB provided by AF.

Figure 2 Association relationship between AQPs of different QoS flow

Step 3：In case the PDB of one QoS flow changes, RAN chooses AQPs coordinately among QoS flows, e.g. in case AF asks for synchronized delivery between service flow#A and service flow#B, if AQP#A1 is used for QoS flow#A, AQP#B1 is chosen for QoS flow#B.

This contribution proposes a solution for the enhanced interaction between AF and the 5GS and also QoS policy enhancement to support joint admission control, joint QoS fulfilment and the delivery of data from multiple QoS flows of an application in a similar time. The solution applies to both KI#1 and KI#2.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change(all new) * * * *

## 6.X	Solution #X: QoS enhancement to support the synchronized delivery of multiple QoS flows

6.X.1	Key Issue mapping

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> | <Key Issue #4> | <Key Issue #5> | <Key Issue #6> | <Key Issue #7> | <Key Issue #8> | <Key Issue #9> |
| #1 |  |  |  |  |  |  |  |  |  |
| #2 |  |  |  |  |  |  |  |  |  |
| #x | X | X |  |  |  |  |  |  |  |

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

6.X.2	Description

This solution proposes enhanced interaction between the AF and the 5GS and also QoS policy enhancement to support the delivery of data from multiple service flows (mapped to QoS flows) of an application in a similar or compatible delay (PDB of the QoS flow). Similar or compatible here mean that when each flow is delivered with its delay requirement, the receiving application can play out or render the data from each flow with minimum buffering or waiting time between the flows. The exact synchronous playout or rendering is handled by the application itself using, e.g., RTP time stamps. The compatible delays are kept even if the 5GS has to downgrade any of the flows to alternate QoS profile with a longer PDB by downgrading also the other flows’s corresponding alterative QoS PDBs.

The solution also proposes enhanced interaction between the AF and the 5GS that allows AF to provide the 5GS policy information about the service flows with respect to which flows are critical to the usability of the application and if they fail either at admission phase or while being delivered (e.g., even downgrade to a lower alternative QoS profile is no longer possible), successful admission or delivery of any of the other flows will not help. The 5GS can use this information for its admission control and QoS fulfilment decisions at both CN and RAN level.

For an application, service flows requiring coordinated delivery are grouped into a service flow group. Each service flow group has group level treatment requirements used for associated control of the service flows in the group, e.g., delivery of data to the receiver at a similar time. Each service flow can also be declared as critical within the group meaning its failure will render the application unusable. Each flow in the service flow group still has its own QoS requirements which is mapped to a QoS profile such as 5QI, ARP, PDB, PER, etc, and optionally a set of ordered alternative QoS profiles (AQPs).

For each group of service flows of an application, AF provides service flow association requirements to 5GS via interactions between AF and PCF. The service flow association requirements indicate association among a group of service flows of the application, and associated control between these service flows. The service flow association requirements consist of a service flow coordination group ID and group level treatment requirements for a group of service flows. The service flow coordination group ID is for identifying a group of service flows, and the group level treatment requirements include information for associated control of service flows in the group, e.g., information for for flow criticality forjoint fulfillment of admission control/resource allocation purposes and information for indicating synchronized delivery of service flows to the receiver side.

PCF maps the service flow coordination group to QoS flow coordination group in the 5GS and derives associated group level treatment policy in the network based on service flow association requirements.

PCF can provide PCC rules with group level treatment policy for service flows in the group, where the group level treatment policy/requirement may include the following information:

1. Synchronized delivery and joint QoS fulfilment information, including an ordered list of PDB values per service flow in the coordination group. SMF derives the QoS profile and AQP(s) of each QoS flow based on ordered list of PDB values. The AQPs of the QoS flows in the group with the same order number are mapped AQPs, which means they are associated with each other. When PDB values have been chosen in such a way that when the PDB in the same position is chosen for each flow, the flows are delivered to the recipient in a synchronized manner, i.e., with a similar end-to-end delay. In case of one of the QoS flows in the group is upgraded/downgraded to the QoS profile or an alternative QoS profile, RAN should upgrade/downgrade the other QoS flows in the group to the QoS profile or mapped AQP accordingly. Finally, if QoS of a service flow or a certain percentage of QoS flows in a service flow group cannot be fulfilled and downgrade to a lower AQP is no longer possible, RAN could abandon all flows of the group. Such joint QoS fulfilment could be limited to a certain period of the lifetime of the flow.

Editor's Note: for KI#2 multiple UEs scenario, it is FFS how to support Synchronized delivery and joint QoS fulfilment.

2. Joint admission control and resource allocation information, indicating associated resource allocation in 5GS (e.g., RAN) among service flows. For example, once a service flow or a certain percentage of the service flows in a group cannot be allocated with the network resource due to poor network condition, the others do not need to be allocated to save the network resource and thus they should be treated jointly.

Each service flow in a service flow group is treated as individual QoS flow without aggregation, and based on the PCC rule, SMF could derive QoS profile and AQP(s) for each QoS flow.

The network fulfills the QoS alignment requirement in the QoS flow coordination group based on the group level treatment policy in addition to the fulfillment of the QoS requirements of each individual QoS flow. RAN may use mapped AQP in case the QoS Profile of some QoS flows in the group cannot be fulfilled. RAN may use the group level treatment policy also for admission control or handover decisions. E.g., perform a handover of all the flows in the group to target RAN together to facilitate the synchronized delivery, admit all QoS flows in the QoS flow coordination group as a whole. Alternatively, SMF/PCF may use such group level treatment policy for PDU session management decisions. E.g., release the other QoS flows in the coordination group in case admission control/QoS fulfillment of one of the QoS flow fails.

6.X.3	Procedures

6.X.3.1	Enhanced interaction between AF and 5GS for QoS policy coordination among multiple QoS flows

AF provides service flow coordination group ID together with group level treatment requirements to PCF via NEF using the procedure of setting up an AF session with required QoS. The AF can provide the requirements and information of all of the service flows in a group with a single request or using multiple requests. If there are service flows related to multiple UEs, the request contains flow information for each UE.

The AF can provide updates to the service flow information or group policy in update requests. If the AF provides or adds service flows in separate requests, joint admission control/resource allocation can be performed not at one specific point of time but still in a joint manner in that failed admission of one service flow can affect the corresponding actions on already established and to-be-established service flows in the same service flow group.

Figure 6.X.3-1: Setting up an AF session with required QoS procedure

1. The AF sends single (1a) or separate (1b) requests to reserve resources for an AF session using an extended Nnef_AFsessionWithQoS_Create request message (AF Identifier, one or multiple UE identifiers, Flow description(s), QoS reference, (optional) Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order and the order of QoS reference parameters is used as association relationship between QoS reference parameters of different service flow), service flow coordination group ID, group level treatment requirements to the NEF.

NOTE: The UE identifier could be UE IP address or GPSI.

The group level treatment requirements may include joint admission and joint QoS fulfilment policy for the flows in the group. The policy can indicate that the admission and/or QoS fulfilment is to be failed if it fails for any or certain percentage of the flows in the group or any of the flows in the group indicated as critical or choosing AQP for each QoS flow coordinately based on the order of AQPs of different QoS flows.

The group level treatment requirements may include synchronized delivery requirement for flows in the group.

2. NEF performs authorization of the single or separate AF requests. It continues with step 3 if the request is authorized and continues with step 5 is the request is not authorized.

3. The NEF discovers the PCF and interacts with the PCF by triggering single or separate Npcf_PolicyAuthorization_Create requests for the UE(s) included in the AF’s request. In the request to the PCF the NEF provides AF Identifier, UE address(es), Flow description(s), the QoS reference, and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order and the order of QoS reference parameters is used as association relationship between QoS reference parameters of different service flow), service flow coordination group ID, and group level treatment requirements. PCF uses this information to issue policy rules for the service flows in the request together. PCF also uses this information to check if there is already established service flows using the same service flow coordination group ID, in which case PCF treats the received request together with the existing policy rules for the service flow coordination group.

4. PCF responses to NEF whether the group level policy is authorized or not.

5. NEF responses to AF whether the request is granted or not.

6.X.3.2	Procedure to distribute coordination group ID and group level treatment policy in 5GS

PCF derives the group level treatment policy for each service flow coordination group based on the group level treatment requirements from the application, and SMF maps the service flow coordination group to QoS flow coordination group in the 5GS. Each service flow in a service flow group should be treated as individual QoS flow without aggregation.

PCF distributes the service flow coordination group ID and group level treatment policy in 5GS using the following procedure. SMF is responsible for providing the information to individual RAN Nodes with the PDU Session modification procedure, which is separate for each UE or PDU Session involved.

Figure 6.X.3-3:  coordinated delivery of multiple QoS flows

1. PCF provides service flow coordination group ID and group level treatment policy to SMF using the existing session management policy association procedure (e.g., SMF policy association modification procedure). This happens with a separate procedure for each PDU session using the flow coordination group ID as a correlator that allows SMF to combine all the policies.

2. If the PCC rule contains a service flow coordination group ID, SMF binds the PCC rule to separate QoS Flows and no other PCC rule is bound to this QoS Flow.

3.-4. SMF provides QoS flow coordination group ID, QoS profile, Alternative QoS Profile (including association relationship information with AQPs of other QoS flows in the group) and group level treatment policy further to RAN via AMF as part of session management information per UE using the existing single PDU session modification procedure.

RAN identifies the QoS flows in the same QoS flow coordination group and applies the group level treatment policy on the service flows in the service flow coordination group accordingly from this step onwards (e.g., AQP level alignment, joint QoS fulfilment/admission control at RAN).

For QoS alignment and joint QoS fulfilment policy, it is enough if RAN gets the information of each QoS flow in the group independently as it can link them to each other with the common flow coordination group ID.

5. RAN uses the information provided by SMF to apply joint admission policy. The related actions taken by RAN are communicated with SMF/PCF. More detailed procedure for this part is provided in Section 6.X.3.3.

6-7. RAN sets up the AN resource and update the session context accordingly.

8. RAN uses the information provided by SMF to monitor QoS and apply AQP level alignment, joint QoS fulfilment. More detailed procedure for this part is provided in Section 6.X.3.3.

9. RAN sends notification to AF via QoS Parameter Notification Control.

For multiple UE case: Based on the newly selected QoS parameter from received from QoS Parameter Notification Control, AF adjusting QoS parameters or release service flows for other UEs via the procedure defined in 6.X.3.1.

According to the above procedures, the RAN can identify the QoS flows in the same QoS flow coordination group and apply the group level treatment policy, e.g., AQP level alignment to ensure synchronized delivery, joint QoS fulfilment, joint admission control/resource allocation among the QoS flows.

6.X.3.3	Procedure to handle group level treatment requirements in RAN

Group level treatment requirements (joint admission control, joint QoS fulfilment, AQP level alignment) can be performed at RAN and/or at SMF/PCF for a single or multiple UEs. In both cases, RAN notifies the group level treatment result along with the QoS flow group ID to SMF/PCF via AMF. In the former case, PCF can notify AF on the received result whereas in the latter case, SMF can make PDU session management decisions according to the notification from RAN and PCF can decide on group level treatment policy update to be applied by the RAN in addition to notifying AF.

RAN performs admission control for each received PDU session modification request following the current procedure. In case of failure of admission control for one of the QoS flow in a coordination group, RAN releases the already reserved resource for QoS flows in the same coordination group if defined in the policy.

In case one or more of the QoS flows cannot fulfil its QoS requirements, AQP level alignment can be performed or based on communication with SMF/PCF/AF, PDU session modification with QoS flow modification or termination can be performed.

6.X.3.4	Procedure to enable atomic handling of joint admission in 5GS

In case AF provides the information about all service flows associated with each other by a coordination flow group identifier in a single request or in a correlated batch of individual requests, the 5GS is able to make an proactive admission control decision across all of the service flows. This has the benefit that the first flows would not first be admitted and then released if a later critical flow could not be admitted.

For the procedure in 6.X.3.1 this requires for the multiple request option between AF and NEF and NEF and PCF an additional batch ID and counter across requests which indicates to the receiving NF which requests belong to the same batch and the amount of the requests in the batch. The receiving NF, i.e., NEF or PCF, respectively, does not proceed until it has received all the requests in the batch, or fails the requests if it does not receive all of them within specific time. This means AF is required to provide the full batch of requests in close succession.

For the procedure in 6.X.3.2 this requires a similar batch ID and counter for requests between the PCF and SMF. SMF does not proceed with PDU Session modifications until it has received the full batch.

After receiving the full batch, the SMF determines how many QoS flow additions or modifications per each RAN Node the batch contains. When issuing the PDU Session modification requests towards each RAN Node, the SMF includes in each request in addition to the flow group correlation ID and the flow group correlation policy (which indicates if a certain flow is critical for admission) and the total number of critical QoS flows as a new parameter.

NOTE: Separate batch IDs and counters between AF and NEF, NEF and PCF, and PCF and SMF are needed in the general case that multiple PCFs and SMFs may be supported for a flow coordination group, otherwise the same batch ID and information can be carried even all the way from AF to SMF. Also, only if it is assumed that UEs involved in the flow coordination group can be served by different RAN Nodes, the SMF needs to determine the number of critical QoS flows per RAN Node, otherwise the information provided by PCF (or even AF) can be directly used.

When receiving the first PDU session modification request with a given flow group coordination ID, the RAN uses the “total number of critical QoS flows” to check if it can admit all the QoS flows and start reserving resources for all of them at once:

- In case RAN can admit all the critical QoS flows, RAN configures the QoS flows information provided in the request and reserves resources also for the rest of the QoS flows that are not part of the request. The resource reservation can be done for a certain period of time, which is implementation specific and out of scope of SA2. The admission of the additional flows is still finally confirmed only when the RAN responds to the individual PDU Session Modification requests.

- In case RAN cannot admit all the QoS flows, RAN notifies SMF/PCF that the admission of all the QoS flows failed with a proper cause code. This allows SMF/PCF to stop proceeding with the rest of the PDU Session Modifications. If RAN still receives them it will reject their admission.

Editor’s Note: It is for FFS if other group admission policies than admitting all or none of the critical flows should be supported or if RAN needs to be given more information about the QoS flows in a batch than just the total number of the critical QoS flows.

If a new critical service flow is added to the flow coordination group by the AF at a later stage, it would not share the same batch ID and would be handled according to the default procedure in 6.X.3.2.

6.X.4	Impacts on services, entities and interfaces

AF

- Uses extended Nnef_AFSessionWithQoS API to provide group level treatment requirements to a group of flows. The flow grouping is indicated by a unique flow group coordination ID. The extended API allows AF to request a QoS reservation for a group of service data flows across multiple UEs and ask group level treatment policies such as joint admission and joint QoS fulfilment for them. AF is provided results of the requests on the level of the whole group.

- Atomic handling of joint admission requires AF to provide all requests to NEF with service flows subject to joint admission in a single request or in a batch of correlated requests.

NEF

- Supports extended Nnef_AFSessionWithQoS API to provide group level treatment requirements to a group of flows. The flow grouping is indicated by a unique QoS flow group coordination ID. Provides the group level treatment policy and the QoS flow group coordination ID to PCF.

- Atomic handling of joint admission requires NEF to provide all requests with service flows subject to joint admission to PCF in a single request or in a batch of correlated requests.

PCF

- Supports and enforces group level treatment policy for the group of flows sharing the same QoS flow group coordination ID. Provides information about the group level treatment policy to SMF, and via SMF to RAN

- Atomic handling of joint admission requires PCF to provide all policy updates with service flows subject to joint admission to SMF in a in a batch of correlated updates.

SMF

- Provides the QoS flow group coordination ID , group level treatment policy to RAN.

- Atomic handling of joint admission requires SMF to determine the total number of critical QoS flows within QoS flow coordination group per RAN Node and provide the total number critical QoS flows to RAN

RAN

- Enhanced enforcement of joint admission, joint QoS fulfilment or synchronous delivery policy, i.e., AQP level alignment, for the flow group.

No impacts to UE, UPF.

# * * * * End of changes * * * *