SA WG2 Meeting #152e	S2-2206739r02

# August 17th – 26th, 2022; Elbonia          	  		(revision of S2-220)

Source: 	Lenovo, Huawei? HiSilicon? NTT DOCOMO? CATT? Vivo?

Title: 	Conclusions for KI#4: KI#4: Influencing UPF and EAS (re)location for collections of UEs

Document for: 	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_eEDGE_Ph2 / Rel-18

Abstract of the contribution: Conclusions for KI#4: Influencing UPF and EAS (re)location for collections of UEs

- Discussion

Void.

2 Proposal

It is proposed to update TR 23700-48 as follows

Start change

7	Evaluation

Editor's note:	This clause will capture the evaluations related to the solutions per KI.

7.x	Evaluation for KI#4: Influencing UPF and EAS (re)location for collections of UEs

For KI#4, the key points to address the issues include:

- 1) How to define and identify a collection of UEs forming a dynamic ad-hoc group that should use the same EAS and/or same local part of DN and/or same DNAI. The dynamic ad-hoc group, agreed between the 5GC and the AF (e.g. as part of a SLA) can be managed only by AF or be synchronized by both AF and 5GC using agreed targeting information.

- 2) How to inform the SMF the information of the same EAS and/or same local part of DN and/or same DNAI to be used. The information of the same EAS and/or same local part of DN and/or same DNAI to be used can be determined by the AF and sent to the 5GC for maintaining and using (corresponding to AF selection of common EAS/DNAI), or can be determined by the 5GC and maintained within 5GC (corresponding to 5GC selection of common EAS/DNAI). For the AF-determined DNAI/EAS, it can be sent to the UE via application layer which is out of scope of 3GPP SA2, it can also be sent to the 5GC for the routing decisions for traffic of PDU Session and the DNS handling rule decision.

In 3GPP TS 23.548, the mechanism to support “Edge Relocation Triggered by AF” is defined. The AF may invoke the AF request targeting an individual UE address procedure as described in TS 23.502 [3] clause 4.3.6.4 (the AF influence on traffic routing procedure), due to EAS relocation.

In 3GPP TS 23.548 and 23.502, The solution for “EAS (Re-)discovery over Session Breakout Connectivity Model ” is described. The SMF interacts with EASDF and provides the ECS option or the Local DNS server address, related to candidate DNAI(s) for that FQDN for the UE location, as part of the rules to handle DNS queries from the UE to the EASDF, the EASDF sends DNS message to the local DNS server or sends DNS message to the centralized DNS server with ECS option. The rules to handle DNS queries from the UE can be updated based on the notified DNS Query with certain FQDN, UE mobility or PSA relocation. For the selection of the candidate DNAI for a FQDN for the UE, the SMF may consider the UE location, network topology, EAS Deployment Information and related policy information for the PDU Session provided as defined in TS 23.503 [4] clause 6.4 or be preconfigured into the SMF. The EAS Deployment Information used to construct the rule can be set per group.

Solutions of KI#4 address one or more of the following aspects: UE collection definition, 5GC selection of common EAS/DNAI, AF selection of common EAS/DNAI.

| Solution | UE collection definition | 5GC selects common EAS/DNAI | AF selects common EAS/DNAI |
| --- | --- | --- | --- |
| #14: Group Management | X |  |  |
| #15: Selection of common DNAI |  | X | X |
| #16: Selecting the same EAS/DNAI for collection of UEs | X | X | X |
| #17: Application layer EAS selection for collections of UEs |  |  | X |
| #18: Discovery of the same EAS for collections of UEs | X | X |  |
| #19: Influencing UPF and EAS (re)location for collections of UEs | X |  |  |
| #34: Selecting the same EAS/DNAI for collection of UEs | X | X |  |
| #35: Providing dedicated (re)location information as traffic routing information | X |  | X |
| #36: Providing dedicated (re)location information as EAS Deployment information | X |  | X |
| #37: (Re)location of same EAS and coordination across UEs |  | X |  |

Thus, based on the issues for KI#4 and to reuse the existing mechanism as much as possible and solve the issue for KI#4, the solutions are analysed with focus on the above three aspects.

Aspect 1: UE collection definition.

Solution #14 performs UE collection definition by group management. It utilizes Nnef_ParameterProvision (or new defined) service for AF to send GPSI list and external group ID. UDM aligns the group data and UE subscription data. UDR generates internal group ID, and stores SUPI list, internal group ID and external group ID.

If Nnef_ParameterProvision service is used, it shall be enhanced to support dynamic UE group management, which only supports 5G VN group or MBS group management in current specification.

Solution #19 performs UE collection definition by group management. AF uses Nnef_ParameterProvision (or new defined) service to send group management information, including group member management parameters and group data:

-  Group member management parameters: GPSI list, external group ID.

-  Group data: DNN/S-NSSAI, Group Type (i.e. ad-hoc group for gaming/platooning/…), EAS ID, Group Attributes (e.g. same EAS/same PSA is needed/…).

NEF translates external group ID into internal group ID, and stores these information in UDM.

Solution #16, #18 and #34 use UE list, group ID or any UE, and Spatial Validity Condition to define the UE collection of specific application, which is identified by FQDN or Application ID.

Solution #35 and #36, the collection of UEs can be identified using explicit group management extending the 5GVN group management which is similar as that described for Solution #14 and #19. The collection of UEs can also be identified with target UEs with service information. The PCF or SMF associates the UE with the collection based on the group ID or the service information via traffic routing information or EDI, and the specific service information for each PDU session received via the procedure of Service specific parameter provisioning.

Aspect 2: 5GC selection of common EAS/DNAI.

Solution #16 proposes that the SMF determines that UE belongs to the dynamic UE group when steps 1-9 of clause 6.2.3.2.2 of TS 23.548 [3] are performed, and the SMF obtains common EAS/DNAI of the UE collection, locally or retrieving from UDR. If the common EAS/DNAI has not been determined for UE collection, the SMF determines the common EAS/DNAI via the Rel-17 EAS discovery procedure, and synchronizes to UDR with the common EAS/DNAI.

Solution #34 EDI is used for maintaining the common EAS/DNAI. SMF synchronizes common EAS/DNAI by updating EDI to NEF/UDR.

Solution #37 UDM/UDR is used for maintaining the common DNAI/EAS as part of group information. The difference is, when common EAS/DNAI is synchronized, the UDM/UDR shall send notification to the SMF(s) that subscribed the group data (see solution #19), including the common EAS/DNAI. Solution #37 only applies to the case of pre-defined group.

Solution #15 introduces a new network function SCMF to manage the common DNAI. SMF may obtain the common DNAI either when PDU Session is established (pre-invoke of SCMF) or when reported DNS Query by EASDF (dynamic invoke of SCMF). The SMF sends EASDF information to build ECS option or L-DNS server address according to the common DNAI. Then rest of procedure in clause 6.2.3.2.2 of TS 23.548 [3] are performed. Solution #15 only deals with common DNAI case.

Solution #18 assumes that the UE group is served by a single SMF. When reported DNS Query, the SMF respond to the EASDF with either the common EAS, or the same information to build ECS option or L-DNS server address as other member(s) of the dynamic UE group.

Aspect 3: AF selection of common EAS/DNAI.

Solution #15 proposes both 5GC and AF selection ways, and only AF selection procedure is evaluated in this aspect.

SMF selects UPF according to common DNAI obtained from SCMF, and notifies the common DNAI to AF as target DNAI via UP path change event. Then AF sends “group FQDN” corresponding to the common DNAI to UE via application layer. UE solves common EAS via DNS. If common DNAI is updated, SMF gets notified by SCMF, and above is repeated.

Solution #17 proposes that AF determines members of the dynamic UE group. For each group member UE, AF sends request to the SMF for candidate DNAI(s) via PCF. SMF notifies AF the candidate DNAI(s). AF selects common DNAI and common EAS according to the candidate DNAI(s) from all the SMF(s). Then AF sends the common DNAI to each SMF via AF influence traffic procedure, and sends the address of the common EAS to each UE via application layer.

Solution #35 and #36 propose solution to support the case that AF determines common EAS/DNAI for certain application for dynamic UE group. The common EAS/DNAI is sent to the SMF via pcc rule or EAS Deployment Information, and the SMF use the common DNAI/EAS for the routing decisions for traffic of PDU Session and the DNS handling rule decision. In Solution #35, the common EAS/DNAI for the collection of UEs is received via traffic routing information, and the PCF associates the UE accessing the application with the collection of UEs using the same DNAI/EAS, and the PCF send the pcc rule with the common EAS/DNAI to the SMF. In Solution #36, the common EAS/DNAI for the collection of UEs is received via EAS Deployment Information, and the SMF associates the UE accessing the application with the collection of UEs using the same DNAI/EAS.

2nd change

8	Conclusions

Editor's note:	This clause will capture the conclusions of the study.

8.x	Conclusions for KI#4: Influencing UPF and EAS (re)location for collections of UEs

8.x	Conclusions of Key Issue #4

KI#4 shall be concluded by the three aspects: UE collection definition, 5GC selection of common EAS/DNAI, AF selection of common EAS/DNAI.

UE collection definition.

PDU Sessions that belong to the UE collection shall use the same DNN/S-NSSAI. The UEs in the UE collection are identified by a UE list, group ID or any UE may be used. Optionally, Spatial Validity Condition, may be used to limit the UEs in the UE collection to a specific area.

UEs in the UE collection shall use the same application. 5GC identifies the application based on the FQDN(s) used in the DNS query.

For group ID case, the dynamic group management is used to identify the UEs in the UE collection. For dynamic group management to the existing NEF service is enhanced or a NEF new service is defined. UDR is used to store the group id and the UE identities in the group.

Editor’s Note: It is FFS what NEF service is user for dynamic group management.

Nnef_ParameterProvision service is used for 5G VN/MBS UE group management, and the service is enhanced for dynamic 5G VN/MBS group management

5GC selection of common EAS/DNAI.

All the solutions above are based on Rel-17 EAS discovery procedure, and use a 5GC NF to manage the common EAS/DNAI. This is the consensus of the solutions for 5GC selection of common EAS/DNAI.

For this aspect, it is proposed that:

-	5GC selection of the common EAS/DNAI for UE collection shall be supported.

-	Each SMF managing a PDU Session in the UE collection invokes the SCMF for selection of common DNAI for an application. The SMF indicates the application and the list of candidate DNAIs to the SCMF. The SCMF can use the existing procedures to subscribe for UE mobility events to retrieve the location of the UE.

-	For the first PDU Session in the UE collection, the SCMF selects the common DNAI and indicates it to the SMF. SMF that receives the common DNAI from SCMF stores the common DNAI to UDM/UDR.

-	The procedures in TS 23.548 [x] are used to perform the DNS Query to the application identified by the FQDN(s). If common EAS IP address is required by the application, the SMF that invoked the SCMF stores the resulted EAS IP address to the UDM/UDR.

-	UDR is used for storing the common DNAI for UE collection.

-	Each SMF managing a PDU Session in the UE collection subscribes for notifications for the common DNAI from UDR. The SMF uses the DNAI for the PDU Session as received from the UDR.

-	In the event of mobility of the UE belonging to the UE collection, the SCMF may determine a new common DNAI, and indicate it to the SMF. The SMF stores it to the UDR, and the UDR notifies rest of the SMFs in the UDR collection.

-	In the event of a new PDU Session, if the PDU Session belongs to a UE collection for an application, the SMF invokes the SCMF and the SCMF may determine a new common DNAI, and indicate it to the SMF. The SMF stores it to the UDR, and the UDR notifies rest of the SMFs in the UDR collection.

-	In the event of a PDU Session release, if the PDU Session belongs to a UE collection for an application, the SMF invokes the SCMF and the SCMF may SCMF may determine a new common DNAI, and indicate it to the SMF. The SMF stores it to the UDR, and the UDR notifies rest of the SMFs in the UDR collection.

-	The parameter "Traffic correlation" in Traffic Influence (defined in clause 5.6.7 of TS 23.501 [x]) is used to indicate that a common DNAI shall be selected for the UE collection.

Editor's Note: It is FFS how to indicate that a common EAS IP address is required by the application.

-	The procedures in TS 23.548 are used to configure the same DNS handling rules (for example, the same DNS server) to resolve the FQDN in DNS query for collection of UEs.

-	If a common EAS IP address is required by the application, the EASDF responses directly with the IP address to the DNS Query.

AF selection of common EAS/DNAI.

As an option, AF could determine the common EAS/DNAI and provide the common EAS/DNAI to 5GS.

For this aspect, it is proposed that:

-	AF selection of the common EAS/DNAI for UE collection shall be supported.

-	AF determined DNAI/EAS for UE collection shall be sent to the 5GC for the routing decisions for traffic of PDU Session and the DNS handling rule decision.

End of change