3GPP TSG-WG SA2 Meeting #153E	S2-2208475

e-meeting, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#5, conclusions

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_MBS2 / Rel-18

Abstract of the contribution: This paper proposes updates to the interim conclusions for key issue 2

# 1	Introduction

Conclusions for key issue 5 were postponed at the last SA2 meeting.

One point of discussion was whether and how to support scenarios where the AF does not know the transmission times when it sends service announcements.

It was suggested that the capability limited devices could in such cases be informed via service announcements. However, delivering such updated service announcements is not straight forward:

- Requiring UEs to periodically wake up and read service announcements at a server means substantial extra power consumption and network load.

- Sending separate service announcements to all involved UEs (which would be buffered according to existing procedures until UEs in power saving mode are reachable) also requires substantial network load and goes against the intention to apply MBS for a data transmission towards such UEs. In addition, the AF needs to be aware of all UEs.

Thus a procedure where the network determines the involved UEs based on the information that the joined the MBS session and informs them about a scheduled transmission time appears preferable. It is suggested to keep the support of that procedure optional.

# 2.	Proposal

It is proposed to include the following changes in TR 23.700-47.

*** 1st Change all new***

## 8.X	Key issue 5: Coexistence with existing power saving mechanisms for capability-limited devices

The following principles are applied for normative work:

-	The UE is configured by the AF with scheduled transmission times when MBS data transmission is scheduled to occur

-	For data transmission at times known when the service announcement is performed (e.g. periodic times), the AF shall inform UE about the scheduled time of the MBS service (e.g., start time, end time, repetition period) in the service announcement message.

-	For data transmissions at times not known when the service announcement is performed, the AF uses a deferred activation request for an MBS session indicating a scheduled transmission time which is more than a wakeup period of the UEs in the future. This information is propagated through MB-SMF and SMFs to AMFs serving UEs within the MBS session. When the UEs wake up, they obtain a notification of the scheduled data transmission time from the AMF. Support of the deferred activation procedure is optional

-	The AF sends MBS data at the scheduled transmission times.

-	At the scheduled data transmission times, the UEs apply the procedures of Solution 14 to receive MBS data.