SA WG2 Meeting #S2-153E	S2-2209107

10 - 17 October, 2022, Electronic meeting

Title:	[DRAFT] Reply LS on XR-awareness in RAN

Response to:	S2-2208144 / R2- 2209215

Release:	Release 18

Work Item:	FS_XRM, FS_NR_XR_enh

Source:	SA2

To:	RAN2

Cc:	RAN1

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks RAN2 on their LS (S2-2208144 / R2-2209215) on XR-awarenss in RAN.

Regarding PDU Set awareness in NG-RAN, SA2 has reache the following agreements:

-	PDU Sets with different PDU Set Importance belonging to the same media stream are bound by the UPF on the same QoS Flow.

-	PDU Set Importance, together with other information listed in TR 23.700-60 clause 8.X.3.1, is conveyed from UPF to NG-RAN via the GTP-U header of user plane packets.

-	While NG-RAN handling of PDU Sets with different PDU Set importance is in the scope of RAN2, the SA2 assumption is that the PDU Sets of the same media stream are delivered in order over the radio.

-	From SA2 perspective the PDU Set Importance is used by NG-RAN primarily for PDU Set-level packet dropping in presence of downlink congestion.

RAN2 may identify additional use of PDU Set Importance.

2. Actions:

To RAN2 group.

ACTION: 	SA2 respectfully asks RAN2 to take into account the feedback above.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#154			14 - 18 November 2022			Toulouse, FR