3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2209676

Elbonia, October 10 – 17, 2022

Source:	Huawei, HiSilicon, Sony

Title:	KI#3: Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: Evaluation and Conclusion for KI#3.

1. Discussion

This contribution proposes the evaluation and conclusion for KI#3.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-05 V1.0.0.

# * * * * First change * * * *

7.3	Evaluations for KI#3

There are six solutions addressing Key Issue#3, Efficient mobility and service continuity when served by mobile base station relay, i.e. solution#4, solution#5, solution#12, solution#13, solution #16, and solution #17:

-	Solution #4 and #5 addresses both KI#2 and KI#3. Solution #4 assumes that the TACs potentially will be broadcasted by the MBSR are provided to the UE by the AMF of the UE served by the MBSR, so that the UEs do not need to perform mobility registration when the MBSR changes TACs. In order to support UE mobility management without the mobility registration, some special contexts are created on NG-RAN associated with the MBSR, and in the AMF serving the UEs. These contexts are connected using a new identifier "Link ID". With solution#4, the donor gNB is able to support AMF-UE relocation for all the UEs served by this AMF with one N2 signalling. i.e., when UE moves with MBSR, the NG-RAN proxies the UE idle mobility and NG-RAN triggers the UE handover due to MBSR mobility. Solution#4 works with legacy UEs when the UE is served by the MBSR, no mobility registration is performed and therefore 5G-GUTI is not updated even if the UE context transfer is triggered among AMFs. CT WGs and SA3 evaluation is required to confirm that the UE context management at AMF is not broken. Support of the new context at donor gNB and AMF and their maintenance during MBSR mobility requires verification by CT WGs and RAN3.

-	Solution #5 is based on solution #4 to enhance idle mobility and handover to support mobile terminated services. Solution#5 further creates a special context in AMF serving the MBSR, and associate it with the context in AMF-UE using the Link ID. Interactions between AMF-UE and AMF-MBSR are introduced to maintain the binding of the Link ID. and AMFs are enhanced to redirect the UE paging message from AMF-UE to AMF-MBSR. The AMF-MBSR forwards the paging message to NG-RAN serving the MBSR so that it can be broadcasted by MBSR. The paging forwarding by AMFs needs verification by CT WGs and RAN3.

-	Solution #12 address KI#3. It proposes to use a full migration approach with a 2-step handover procedure for reducing the service interruption, firstly MBSR handover is performed without F1 anchor change, then UE handover is triggered.  With this solution, the UEs served by MBSR would still experience mobility event even if they remain connected to the same MBSR. Other the other hand, Solution#12 does enhance the existing Xn and N2 handover procedures to support group UE handover and therefore reduces corresponding signalling over Xn and N2. Overall, the principle of this solution is to aggregate the information of group UEs in a single message to support bulk handling in 5GC. When group UEs are accessing the same IAB-node, the commonality is that group UEs share the same user location and AMF. The support of group mobility procedure requires verification of RAN WG3. The solution#12 does not address the IDLE mode mobility aspect when the UEs move together with the MBSR. Additionally, delivery of the RRC messages to the connected UEs requires F1-C connectivity between the source IAB-donor-CU and the migrating MBSR. This F1-C connectivity may have to be carried via the CN if the source IAB-donor-CU and the target IAB-donor do not have Xn and/or IP connectivity. This should be coordinated with RAN WGs.

-	Solution #13 address KI#3. It proposes to adopt a partial migration approach that besides the CU of gNB handles the MBSR mobility, a central CU that acts as F1 anchor for as long as practically possible to shield the MBSR mobility from the UEs it serves, which saves the AN signalling for mobility. This requires some discussion and adoption in RAN WGs. An interaction with RAN3 is required to consolidate this approach. The feasibility to support IAB mobility based on the Rel-17 IAB partial migration should be discussed in the RAN WGs. In addition, R18 NR_mobile_IAB in RAN WGs only focuses on full migration (RP-213601). Given that the m-CU cannot cover all possible MBSR operation locations, additional handling from other solutions may needed to complement this solution.

-	Solution #16 addressed KI#3. It proposes that the MBSR is configured by the donor gNB to broadcast the TAC of its serving cell. For a UE served by the MBSR, it would observe the TAC change when it moves together with the MBSR across different serving cells, even if it camps on the same MBSR cell. On the other hand, a UE would not be triggered to perform mobility registration when it camps on or leaves the MBSR cell temporarily.

-	Solution #17 addressed KI#3. It proposes two options. Option#1 is that the MBSR uses a dedicated TAC, and the TAC remains the same when MBSR moves across different donor gNBs. The donor gNB will add or remove the TAC of the MBSR to UE's AMF(s) when the MBSR connects to leaves it. With this solution, the UE served by MBSR would not perform mobility registration, even when the MBSR changes donor gNBs. Option#2 is that MBSR broadcasts a TAC which maintains unchanged within a certain service area. Once there is no N2 reference point between new IAB-donor gNB and AMF-UEs, the MBSR will change the TAC to broadcast during the full migration phase. This triggers the UEs camped on the MBSR to perform mobility registration.

The solutions have RAN impact as following and need RAN coordination:

1)	Solution #4 and #5 require NG-RAN to manage MBSR context with Link IDs sent by AMFs via NGAP messages, one Link ID identifies one AMF that serves a group of UEs camp on the MBSR, as well as require NG-RAN to initiate an NGAP message to new AMF with Link ID(s) for AMF relocation when needed.

2)	Solution #12 requires NG-RAN to support full migration approach with group handover for UEs. The potential F1-C connectivity via CN needs coordination with RAN WGs.

3)	Solution #13 requires NG-RAN to support a partial migration approach with a central CU (m-CU) together with a local CU (gNB-CU).

4) Solution #16 and #17 need RAN confirmation of the feasibility of updating the UE-AMF regarding the TAC supported by the MBSR via NGAP.

# * * * * Second change * * * *

8.3	Conclusions for KI#3

Editor's note: This clause includes interim conclusions, and needs further study.

For KI#3, the interim conclusions are as follows:

- During MBSR’s mobility, the TAC broadcasted by the MBSR is the same as the TAC of the cell where the IAB-UE is located. When the IAB-UE enters to a new TA, the cell broadcasting information is updated accordingly.

- The UE’s mobility management is performed using the legacy mechanism as defined in the TS 23.501 [2] and TS 23.502 [5]. The UE in CM-Idle shall follow legacy procedure when detecting a TAC which is not in the TA list.

- Each UE connected via the MBSR may have different serving AMFs e.g., due to slicing and individual PDU sessions/QoS service flows configured. UE context handling and path switching would be handled per each individual UE.

Editor's note: The above interim conclusion and support of group mobility are subject to feedback of RAN WGs and will be revisited as per the feedback.

Editor's note: Regarding the alternatives assuming MBSR operates with dedicated TAC(s), it is subject to the feedback of RAN WGs.

Editor's note: For UEs in RRC-Connected/CM-Connected state via an MBSR, whether and how to guarantee UE’s service continuity is subject to the IAB full migration/mobility in the RAN Rel-18 study.

Editor's note: The alternatives assuming MBSR operates with dedicated TAC(s) or TAC from Donor are subject to the feedback of RAN WGs.

# * * * * End of changes * * * *