3GPP SA WG2 Meeting #153E	S2-2209872

e-meeting, 10-17 October 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Annex to capture methods to notify UE regarding RAN time sync for KI#1

Document for:	Approval

Agenda Item:	9.7.1

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes an annex to capture the methods to notify UE regarding RAN time sync for KI#1 for TR 23.700-25.

# Discussion

In order to reach consensus regarding KI#1 conclusion, we need to capture methods to notify UE regarding RAN time sync for KI#1.

# Proposal

It is proposed to add the following text to TR 23.700-25.

*** Start of changes (all new text) ***

Annex A:

## Annex A.1 Methods to notify RAN Time Synchronization Status towards the UE

### A.1.1 Alternative 1: gNB provides a reference report ID within SIB

In this alternative when there is a new RAN timing synchronization status report available at the gNB, the gNB includes in the SIB a status report ID as a notification for the UEs reading the SIB. The report ID can be an optional integer information element. This report ID enables the UE to know there is new information available. There are two options for the UE to determine RAN timing synchronization status information with the Report ID:

- The UE can actively retrieve the RAN timing synchronization status information from the network by entering RRC_Connected..

- Report ID is an index that maps to a pre-defined and/or standardized time synchronization characteristics thus the UE can automatically determine this without having to move to RRC_CONNECTED state.

An overall procedure for SIB including a reference report ID is illustrated in Figure A.1.1-1.

Figure A.1.1-1: Procedure for gNB provisioning status report ID in SIB

1.	The UE has received reference time information using unicast RRC or via SIB9. The RAN releases the UE to RRC Inactive or RRC Idle state.

2.	The NG-RAN node detects a primary source event (e.g., degradation, failure, recovery).

3.	The NG-RAN generates a RAN timing synchronization status report and an associated status report ID.

4-5.	The NG-RAN node broadcasts a status report ID in the cell using SIB to notify the primary source event to the UEs camping in the cell.

6.	The UE reads SIB and the status report ID and:

- 	Alternative a), the UE determines there is a new RAN timing synchronization status report available at the NG-RAN which should be retrieved following steps 7-9.; or

-	Alternative b), the UE uses the status report ID as an index to map to the pre-defined and/or standardized characteristics. Steps 7-9 are skipped.

7.	In case of alternative a), the UE enters RRC_Connected..

8.	In case of alternative a) after UE moves to RRC_CONNECTED mode, the NG-RAN determines the UE is subscribed to RAN timing synchronization status (e.g., based on configuration provided by the TSCTSF via AMF).

9.	In case of alternative a) the NG-RAN node sends the last available RAN timing synchronization status report with its associated status report ID to the UE via dedicated RRC signaling.

10.	In case of alternative b) the UE uses the report ID as an index to map to the pre-defined and/or standardized characteristics that describe the RAN timing synchronization status.

### A.1.2 Alternative 2:

xxxx

### A.1.3 Alternative 3:

xxxx

### A.1.4 Alternative 4:

xxxx

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | SA2#149E | S2-2201055 | - | - | - | Proposed skeleton agreed at S2#149E | 0.0.0 |
| 2022-02 | SA2#149E | - | - | - | - | Implemented agreed P-CRs at S2#149E – S2-2201759, S2-2201760, S2-2201761, S2-2201762, S2-2201763, S2-2201764, S2-2201765, S2-2201766, S2-2201768, S2-2201769, S2-2201770. | 0.1.0 |
| 2022-04 | SA2#150E | - | - | - | - | Implemented agreed P-CRs at S2#150E - S2-2203462, S2-2203463, S2-2203464, S2-2203465, S2-2203466, S2-2203467, S2-2203468, S2-2203469, S2-2203470, S2-2203471, S2-2203472, S2-2203473, S2-2203474, S2-2203475, S2-2203476, S2-2203477, S2-2203478. | 0.2.0 |
| 2022-05 | SA2#151E | - | - | - | - | Implemented agreed P-CRs at S2#151E - S2-2205159, S2-2205160, S2-2205161, S2-2205162, S2-2205163, S2-2205164, S2-2205165, S2-2205166, S2-2205167, S2-2205168, S2-2205169, S2-2205170, S2-2204421, S2-2205171, S2-2205172, S2-2205173, S2-2204591, S2-2205174, S2-2205175, S2-2205176, S2-2205177, S2-2205178, S2-2205179, S2-2205180, S2-2205181, S2-2205182, S2-2205183 | 0.3.0 |
| 2022-08 | SA2#152E | - | - | - | - | Implemented agreed P-CRs at SA2#152E - S2-2207723, S2-2207724, S2-2207725, S2-2207726, S2-2207727, S2-2207728, S2-2207729, S2-2207730, S2-2207731, S2-2207732, S2-2207733, S2-2207734, S2-2205934, S2-2206610, S2-2207735, S2-2207736, S2-2207737, S2-2206593, S2-2205933 | 0.4.0 |
| 2022-09 | SA#97-e | SP-220818 | - | - | - | MCC editorial update for presentation to TSG SA for information` | 1.0.0 |