SA WG2 Meeting ##153-e	S2-2208135

10-17 October 2022, Electronic Meeting

3GPP TSG-CT WG4 Meeting #111-e	C4-224409

# E-Meeting, 18th – 26th August 2022

# Title:	Reply LS on Nudm_UEContextManagement service for satellite NG-RAN

# Response to:	LS (C1-222786) on Nudm_UEContextManagement service for satellite NG-RAN from CT1

# Release:	Rel-17

# Work Item:	5GSAT_ARCH-CT

Source:	CT4

To:	CT1

Cc:	SA2

Contact Person:

Name:	Ulrich Wiehe

E-mail Address:	ulrich.wiehe@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

CT4 thanks CT1 for their LS on Nudm_UEContextManagement service for satellite NG-RAN, which asks for action:

ACTION: 	CT1 respectfully requests CT4 to update Nudm_UEContextManagement service so that the AMF can be made aware of whether each of the TAIs of the current cell should be forbidden or not.

Answer:

Taking into account the
 approved CR #4131 to TS 24.501 in CP-220147 (which was attached to CT1's LS) and also the
 approved CR #4305 to TS 24.501 in C1-223556 (which adds additional modifications to clause 5.1.1.4),

CT4 respectfully ask CT1 to note that the AMF is made aware of the complete list of the UE's forbidden TAIs in the Serving PLMN as part of the Nudm_SDM Get service operation. This allows the AMF to determine whether each of the TAIs of the current cell should be forbidden or not.

However, the list of forbidden TAIs sent from UDM to AMF as part of the UE's subscription information does not indicate which TAIs are forbidden for roaming and which TAIs are forbidden for regional provision of service. Consequently the AMF cannot determine based on the UE's subscription information received from the UDM whether a given TAI of the current cell should be included in

a) the Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" IE; or

b) the Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" IE; or

c)	both;

in the REGISTRATION ACCEPT message.

CT4 could not reach consensus whether the changes in C1-223556 actually require updates to the Nudm_SDM Get service operation, allowing to indicate per forbidden TAI whether the TAI is forbidden for roaming or forbidden for regional provision of services, based on UE subscription information.

2. Actions:

To CT1

ACTION: 	CT4 ask CT1 to clarify whether the requirement exist for the UDM to make the AMF aware whether (based on subscription information) a given forbidden TAI is forbidden for roaming or forbidden for regional provision of services.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#112e			10/2022	E-Meeting