3GPP TSG-SA WG2 Meeting #153E	S2-2208295

Elbonia, Oct 10 - 17, 2022	(revision of S2-220xxxx)

# Title:	[Draft] Reply LS on UAV authorization container

# Response to:	(S2-2208104/C4-223513) LS on UAV authorization container

# Release:	Rel-17

# Work Item:	ID_UAS

Source:	[Nokia will be] SA2

To:	CT4

Cc:	CT1, CT3

Contact Person:

Name:	Pallab Gupta

Tel. Number:

E-mail Address:	pallab.gupta@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: TS 23.256 CR xxxx.

1. Overall Description:

SA2 thanks CT4 for their LS on UAV authorization container.

SA2 would ask CT4 to note the following answers for the corresponding questions raised by CT4 and would request CT4 to define/update stage 3 specification accordingly.

- Does the SMF need to identify the type of payload (C2- or UUAA-Aviation Payload) to perform the procedures from and towards the UE?

Answer: No. The SMF does not need to identify the payload type. From an SMF pov, it is transparent whether the authorization is for UAV or for C2 connectivity or for both. In all the cases, the SMF invokes Nnef_Authentication_AuthenticateAuthorize service operation including the Service Level Device Identity and the UAS container provided by the UE. The UAS NF may need to identify the payload type, as the UAS NF checks the stored UUAA context before forwarding the request from SMF to the USS/UTM.

The USS/UTM sends the authentication/authorization result to the UE as part of the Authorization Data container which is again transparent to the 3GPP. The SMF does not need to identify if the authorization data sent from USS/UTM to the UE contains C2 related data or UAV authentication/authorization related data or both. The SMF decides to perform either the step 3 or step 7 in Figure 5.2.3.2 -1 of TS 23.256, based on the presence of authorization result as below:

i) If the Nnef_Authentication_AuthenticateAuthorize contains no Success/Failure indication then the SMF executes steps 3c to 3f in Figure 5.2.3.2 -1.

ii) If the Nnef_Authentication_AuthenticateAuthorize contains a Success/Failure indication, then the SMF executes step 7 in Figure 5.2.3.2 -1

In case i) above an AA container is expected.

- If yes, how is this identification performed, and what are the impacted procedures?

Answer: See answer for question 1.

2. Actions:

To CT4

ACTION: 	SA2 kindly asks CT4 to take the above into account and update stage 3 specification accordingly if necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #154		Nov 14 - 18, 2022				Toulouse, France