3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208318

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#5, Update conclusion on KI#5

Document for:	Approval

Agenda Item:	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: The conclusion on KI#5 of FS_eNPN_Ph2 about how to enable NPN as hosting network for providing access to localized services.

1. Introduction/Discussion

The contribution would like to further update the conclusion about the architecture used by the UE to access both home network services and localized services.

Since the localized service is accesses via hosting network, which may have location condition. UE shall access to hosting network directly to obtain the localized services. If the localized service can be obtained via overlay network, it means that the UE can be anywhere as long as it can establish an IPSec with the N3IWF in the hosting network via a PLMN. However, this is not aligned with the definition of localized services. Therefore, the option of accessing home network services via underlay network, while accessing localized services via overlay network is not appropriate.

In summary, the contribution proposes that when the hosting network is a SNPN which provides localized services, the existing overlay/underlay architecture is reused for the UE to access both home network services via overlay network via N3IWF and localized services via underlay network.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-08.

# * * * * First change * * * *

8.5	Key Issue #5: Enabling access to localized services via a specific hosting network

The following conclusions are agreed for normative work:

-	The existing SoR procedure is enhanced as follows:

-	The SoR contains a prioritized list of hosting networks. The list is determined by the home network and may be associated with validity conditions, including time and location conditions.

-	When the hosting network is a SNPN which provides localized services, the existing overlay/underlay architecture is reused for the UE to access both home network services via overlay network via N3IWF and localized services via underlay network.

Editor's note:	The scenario of accessing to localized services via N3GPP network is for further consideration.

# * * * * End of changes * * * *