3GPP TSG-SA WG2 Meeting #153E (e-meeting)		S2-2208331

# Oct 10 – 17, 2022, Elbonia																			 (revision of )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0136 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MBS session activation and deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In CT4 LS S2-2208136/C4-224417, there are some questions related with N3mb and N19mb handling in multicast MBS session activation (clause 7.2.5.2) and multicast MBS session deactivation (clause 7.2.5.3): Q1: During the MBS session deactivation procedure, does the MB-SMF instruct the MB-UPF to stop the forwarding of DL MBS data towards the N3mb DL F-TEIDs configured in the MB-UPF for the MBS session? Q2: If the answer to Q1 is yes, when the multicast traffic resumes (i.e. first packet arrives at MB-UPF), does the forwarding of MBS data towards the N3mb DL F-TEIDs resume only after the MB-SMF instructs the MB-UPF to forward the packets at step 15 of the MBS session activation call flow (Figure 7.2.5.2-1), i.e. after the MB-SMF activates the multicast MBS session towards the AMF(s) (and NG-RAN nodes) and receives a successful response from the AMF (and a first NG-RAN node)? Q3: Can SA2 confirm that during an MBS session deactivation procedure, the SMF need not and shall not request the UPF(s) to maintain the N19mb tunnels and that, after an N19m tunnel (of the MBS session) is removed in the UPF, the SMF shall not send a ContextUpdate request message towards the MB-SMF to report the removal of the N19mb tunnel in the UPF? Clarification is required on N3mb and N19mb handling when multicast MBS session is activated and deactivated. Besides, in clause 7.2.5.2 and clause 7.2.5.3, the interaction between MB-SMF and MB-UPF refers to an incorrect clause 4.4 of TS 23.502 which describes interaction between SMF and UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 7.2.5.2, in step 14, clarify AMF sends response to MB-SMF after receiving the first successful NGAP activation response from NG-RAN. - In clause 7.2.5.3, in step 2, clarify MB-SMF requests MB-UPF to remove N19mb tunnels and there are no needs for the SMF(s) to send ContextStatusUpdate to remove N19mb tunnel again. - Regarding N3mb tunnel handling, which hasn’t been specified clearly, there are two options: - Option-1 (keep current solution): In MBS session deactivation, MB-SMF requests MB-UPF to keep the N3mb tunnels. In MBS session activation, there is no need for MB-SMF to provision the available N3mb tunnel back to MB-UPF. In this way, if NG-RAN establishes or releases shared delivery when the MBS session is inactive, the MB-SMF needs to interact with the MB-UPF. - Option-2 (optimize current solution): In MBS session deactivation, MB-SMF requests MB-UPF to remove the N3mb tunnels as well. In MBS session activation, the MB-SMF provisions the available N3mb tunnels back to MB-UPF. In this way, an N4 interaction can be saved, if the shared delivery is established or released when the MBS session is inactive. - Clarify that the handling of interaction between MB-SMF and MB-UPF is the same as the handling of interaction between SMF and UPF, which can be referred to clause 4.4 of TS 23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear information which caused the misunderstanding in CT4. Incorrect description regarding the reference to clause 4.4 of TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.4 (Option-2), 7.2.2.4 (Option-2), 7.2.5.2, 7.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Option-1: Start of Change * * * *

#### 7.2.5.2	MBS session activation procedure

The following can trigger the MBS session activation procedure:

-	AF requests MB-SMF to activate the MBS session;

-	MB-UPF receives the multicast data and notifies MB-SMF.

Figure 7.2.5.2-1: MBS session activation procedure

In this procedure, steps 11 to 15 are executed if the MB-SMF finds out there are shared tunnel established. Steps 11 to 15, if needed, are executed in parallel with steps 2 to 10.

1.	The procedure may be triggered by the following events:

-	When the MB-UPF receives downlink data for a multicast MBS session, based on the instruction from the MB-SMF (as described in clause 7.2.5.3), the MB-UPF sends N4mb Notification (N4 Session ID) to the MB-SMF for indicating the arrival of DL MBS data.

-	The AF sends MBS Activation request (TMGI) to the MB-SMF directly or via NEF.

2.	MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify (MBS Session ID, multicast session state = Active) to SMF(s).

The SMF sets the related multicast MBS session state to Active and finds out the list of UEs that joined the multicast MBS session identified by the related TMGI. If the SMF determines the user plane of the associated PDU session(s) of the UE(s) with respect to the TMGI are activated already, steps 3-8a will be skipped for those UE(s), i.e. executed from step 8b.

3.	The SMF invokes Namf_MT_EnableGroupReachability Request (List of UEs, [PDU Session ID of the associated PDU Sessions], TMGI, [UE reachability Notification Address], [most demanding ARP, 5QI of all MBS QoS Flow within MBS session])) to AMF(s). When later UE is reachable, the UE reachability Notification Address is used by the AMF to identify and notify the related SMF.

After receiving the request, for each UE in the list, the AMF determines CM state of the UE: see steps 4 - 7.

4a.	If there are UEs involved in the multicast MBS Session and in CM-CONNECTED state, the AMF indicates those UEs to the SMF, using Namf_MT_EnableGroupReachability Response (UE list). Otherwise, the response does not include UE list.

4b.	For each UE in the UE list included in step 4a, if the QoS profile(s) for associated PDU Session has not yet been provided, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information (PDU Session ID, MBS Session ID, [QoS profile(s) for associated QoS flow(s)], [mapping information between the unicast QoS flow and multicast QoS flow])) to the AMF for the UE which is identified in step 4a. The associated unicast QoS Flow(s) as well as the mapping information between the unicast QoS Flow(s) and multicast QoS Flow(s) are included to support the 5GC Individual MBS traffic delivery.

The procedure continues at step 9.

5.	[Conditional] If AMF determines that there are UEs in CM-IDLE state and involved in the multicast MBS Session, the AMF figures out the paging area covering all the registration areas of those UE(s), which need to be paged. The AMF may apply paging differentiation as specified in clause 6.12. The AMF sends a Multicast Group paging request message to the NG-RAN node(s) belonging to this Multicast Paging Area with the involved UE list and TMGI as the identifier to be paged if the related NG-RAN node(s) support MBS. If the NG-RAN node(s) do not support MBS, the AMF sends Paging message(s) to the NG-RAN node(s) per UE as described in step 4b in clause 4.2.3.3 of TS 23.502 [6].

NOTE 1:	In addition to the paging in clause 6.12, other paging strategies is up to AMF implementation.

NOTE 2:	The details of the paging are specified by the RAN WGs.

6.	Receiving the paging, the UE(s) in CM-IDLE state sends Service Request message to the AMF, see clause 4.2.3 of TS 23.502 [6].

NOTE 3:	Step 6 for a UE can be parallel to step 5 for another UE(s), which has not received any paging yet.

7a/7b.	After receiving the Service Request sent by the UE(s),

-	Either based on the received PDU Session ID in step 3, the AMF identifies the related SMF and invokes Nsmf_PDUSession_UpdateSMContext request. The procedure continues at step 9; or:

-	Based on the received UE reachability Notification Address in step 3, the AMF identifies and notifies the related SMF of the UE(s), which are reachable now and the Location Information, by using the Namf_MT_UEReachabilityInfoNotify message. In this case, it can be a separated notification or combined with step 8.

8a.	For UE(s) that do not respond to paging, the AMF informs the SMF of the paging failure in Namf_MT_UEReachabilityInfoNotify.

8b.	For UE(s) that is indicated as reachable via the Namf_MT_UEReachabilityInfoNotify message, or user plane of the associated PDU session is activated already but the QoS profile(s) for associated QoS flow(s) needs to be provided for the PDU session, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information ()) to the AMF same as described in step 4b.

9.	The AMF sends N2 request message (N2 SM information ()) to the RAN node.

NOTE 4:	A joined UE is not able to receive MBS data if NG-RAN rejects the PDU Session Resource setup request (due to implementation specific reasons, e.g. activation of user plane fails due to the number of UEs reaching a limit).

10a.	If the shared tunnel has not been established before, the shared tunnel is established at this step, as defined in clause 7.2.1.4. The NG-RAN configures UE with RRC messages if needed.

10b.	Steps 8 to 12 defined in clause 7.2.1.3 are performed. If 5GC Individual MBS traffic delivery is used, the SMF configures the UPF for individual delivery and if necessary, requests the MB-SMF to configure the MB-UPF to send multicast data to the UPF.

11.	If the MB-SMF finds out there are shared tunnel established, step11-15 are performed. The MB-SMF invokes Namf_MBSCommunication_N2MessageTransfer Request (TMGI, N2 SM Information (Activation, TMGI)) to the AMF for those NG-RAN nodes, which have shared tunnel with MB-UPF. This step may be performed in parallel with step 2.

NOTE 5:	The messages in steps 10a, 11 and 12 are MBS-specific and it is possible that the AMF(s) in steps 10a, 11 and 12 are not associate to any UEs involved in the multicast MBS Session.

12.	The AMF sends NGAP activation request message (N2 SM Information ()) to the NG-RAN nodes. For those UEs that have joined in the MBS Session and are in RRC_INACTIVE state, the RAN nodes perform RAN paging as specified in TS 38.300 [9].

13.	The NG-RAN nodes responses to AMF by NGAP activation response message. The NG-RAN nodes establish radio resources to transmit multicast MBS session data to the UE(s). The NG-RAN shall not release the radio connection of a UE that has joined into the Multicast MBS session only because no unicast traffic is received for the UE.

14.	After receiving the first successful NGAP activation response message from the NG-RAN, the AMF sends Namf_MBSCommunication_N2MessageTransfer Response () to the MB-SMF.

15.	The MB-SMF sends N4mb Session Modification Request to the MB-UPF to forward the receiving packet. The MB-UPF responds to the MB-SMF with N4mb Session Modification Response acknowledging the MB-SMF request. The N4mb interactions between MB-SMF and MB-UPF are the same as the ones between SMF and UPF, which are specified in clause 4.4 of TS 23.502 [6].

* * * * Option-1：Next Changes * * * *

#### 7.2.5.3	MBS session deactivation procedure

Figure 7.2.5.3-1: MBS session deactivation procedure

In this procedure, steps 3 to 4 and steps 5 to 9 are executed in parallel.

1.	The procedure may be triggered by the following events:

-	When MB-UPF detects there is no data receives for the MBS Session, MB-UPF sends MB-N4 Notification (N4 Session ID) to the MB-SMF for deactivating the MBS session.

-	AF sends MBS Deactivation request (TMGI) to the MB-SMF directly or via NEF.

2.	The MB-SMF send N4mb Session Modification Request (TMGI, Buffered Downlink Traffic detection) to the MB-UPF. The N4mb interactions between MB-SMF and MB-UPF are the same as the ones between SMF and UPF, which are specified in clause 4.4 of TS 23.502 [6]. The Buffered Downlink Traffic detection is requested by MB-SMF for next time MBS session activation. If the MBS session is to be activated via the AF request directly, this indication is not needed. The MB-SMF also indicates the MB-UPF to remove the shared tunnel(s) that are used for Individual MBS traffic delivery over N19mb interface, so that the SMF(s) do not need to invoke Nmbsmf_MBSSession_ContextUpdate to the MB-SMF after the UPF(s) release the N19mb DL tunnel(s).

MB-UPF to MB-SMF: N4mb Session Modification Response acknowledging the MB-SMF request.

3.	The MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify request (MBS Session ID) to the SMFs.

Based on the received MBS Session ID, the SMF sets the indicated multicast MBS session state to Inactive:

‐	If the SMF finds out there are UE(s) that joined the indicated multicast MBS session and use 5GC Individual MBS traffic delivery, step 4 is performed for those UE(s).

‐	If SMF find there are no UE(s) that joined the indicated MBS session and use 5GC Individual MBS traffic delivery, no further operation for SMF is required.

4.	[Conditional] For those UE(s) that the 5GC individual delivery is used, step 3b and steps 4-8 in clause 4.3.3.2 of TS 23.502 [6] are performed to remove the associated QoS flow(s) related to the multicast MBS session.

NOTE 1:	Whether the associated QoS Flow(s) are removed from UE, NG-RAN, or only resource in NG-RAN is removed is up to implementation.

5.	If the MB-SMF finds out there are shared tunnel established over N3mb interface, the MB-SMF sends Namf_MBSCommunication_N2MessageTransfer Request (TMGI, N2 SM information (Deactivation, TMGI)) to the AMFs.

6.	The AMF sends NGAP deactivation request message (N2 SM information ()) to the NG-RAN nodes.

7.	The NG-RAN node keeps the multicast MBS Session Context and N3mb shared tunnel for the multicast MBS session.

If the MBS Session Context indicates no UE for the multicast MBS session (e.g. due to UE becomes CM-IDLE state), the NG-RAN triggers release of the shared delivery as described in clause 7.2.2.4.

8.	NG-RAN acknowledges the NGAP deactivation Response message.

9.	The AMF invokes Namf_MBSCommunication_N2MessageTransfer Response to acknowledge the service for MB-SMF.

When the MBS session is in "Inactive" state and handover procedure is triggered, it is defined in clause 7.2.3.6.

NOTE 2:	There is no explicit "deactivation" indication to the UE, how the UE is changed to IDLE state is defined in TS 38.300 [9].

* * * * Option-2: Start of Change * * * *

#### 7.2.1.4	Establishment of shared delivery toward RAN node

In the following cases, the shared tunnel for shared delivery is established between the NG-RAN and MB-UPF:

-	The first UE is included in the context of the MBS session in the NG-RAN.

NOTE 1:	When the multicast MBS session is inactive, if there is at least one UE joining the multicast MBS session is in RRC-CONNECTED state in the NG-RAN, the shared delivery is not released.

NOTE 2:	Share delivery establishment procedures are used when MBS supporting NG-RAN node(s) get involved in the multicast MBS session regardless of the state of the multicast MBS session.

-	Handover to the target NG-RAN when the shared delivery tunnel is not established in the target RAN node for this multicast MBS session.

Figure 7.2.1.4-1: Establishment of shared delivery toward NG-RAN node

1.	A NG-RAN node decides to establish shared delivery for a multicast MBS session when it serves at least one UE within the multicast MBS session. For location dependent services, the NG-RAN node needs to establish shared delivery for the location dependent contents of a multicast MBS session if it serves at least one UE assigned to an MBS Session ID and Area Session ID.

2.	The NG-RAN sends an N2 MBS Session request message (MBS Session ID, [Area Session ID], N2 SM information ([unicast DL tunnel Info])) towards the AMF.

If the NG-RAN node is configured to use unicast transport for the shared delivery, it allocates a GTP tunnel endpoint and provides the unicast DL tunnel Info in the request, which includes the GTP tunnel endpoint and NG-RAN node address. For location dependent MBS services, the NG-RAN node also provides the Area Session ID.

3.	The AMF selects the MB-SMF serving the multicast MBS session, e.g. using the NRF discovery service or locally stored information. It invokes Nmbsmf_MBSSession_ContextUpdate request (MBS Session ID, [Area Session ID], N2 SM information) to the MB-SMF.

The AMF stores the information of the NG-RAN nodes (e.g. NG-RAN node ID) for the subsequent signaling related to the multicast MBS Session.

4.	[Conditional] If the MB-SMF received unicast DL tunnel Info in step 3 and if the MBS session is active, it configures the MB-UPF to send multicast data for the multicast MBS session (or location dependent content of the multicast MBS session if an Area Session ID was received) towards that GTP tunnel endpoint via unicast transport.

5.	The MB-SMF stores the information of the AMF (e.g. AMF ID) in the MBS Multicast MBS session context (or location dependent part of the Multicast MBS Session Context if an Area Session ID was received) to enable subsequent signalling towards that AMF.

6.	The MB-SMF sends Nmbsmf_MBSSession_ContextUpdate response (MBS Session ID, [Area Session ID], N2 SM information ([TMGI], multicast QoS flow information, session state (Active/Inactive), [multicast DL tunnel Info], [MBS service areas])) to the AMF. If the MB-SMF did not receive unicast DL tunnel Info in step 3, it provides the multicast DL tunnel info that includes transport multicast address (e.g. a LL SSM) and a GTP tunnel endpoint for multicast transport of the shared delivery.

7.	The AMF sends an N2 MBS Session response message (MBS Session ID, [Area Session ID], N2 SM information) to the NG-RAN node. If the NG-RAN node receives the multicast DL tunnel Info of the shared delivery, it uses the transport multicast address included in the multicast DL tunnel info to join the multicast transport distribution.

* * * * Option-2：Next Changes * * * *

#### 7.2.2.4	Release of shared delivery toward RAN node

In the following case, the shared delivery tunnel may be released between NG-RAN and MB-UPF:

-	The last UE is excluded from the context of the multicast MBS session in the NG-RAN node;

-	Handover to the target NG-RAN when the UE is the last UE for this multicast MBS session in the source NG-RAN node during handover preparation phase known by the source NG-RAN node;

-	Handover to the target E-UTRAN when the UE is the last UE for this multicast MBS session in the source NG-RAN node;

-	MBS session deletion.

NOTE:	When the multicast MBS session is inactive, the shared delivery is not released if there is at least one UE is in RRC-CONNECTED state for this multicast MBS session.

Figure 7.2.2.4-1: Release of shared delivery toward RAN node

1.	A RAN node decides to release shared delivery for a multicast MBS session, e.g. because it no longer serves at least one UE within the multicast MBS session. For location dependent services, the NG-RAN node may release shared delivery for the location dependent contents of a multicast MBS session if it no longer serves at least one UE assigned to an MBS Session ID and Area Session ID.

2.	The NG-RAN node sends N2 MBS Session release request (MBS Session ID, [Area Session ID], [N2 SM information ([GTP tunnel info], release indication)])]) to the AMF. For location dependent services, the NG-RAN node also provides the Area Session ID. The RAN node includes the unicast DL tunnel info if unicast transport is used for the shared delivery. If the NG-RAN node was configured to use multicast transport for the 5GC Shared MBS traffic delivery, the NG-RAN node does not include the N2 SM information in the message.

3.	[Conditional] If the N2 SM information is received or it is the last RAN node controlled by the AMF serving the multicast MBS session, the AMF invokes Nmbsmf_MBSSession_ContextUpdate request (MBS Session ID [Area Session ID], [leave indication], [N2 SM information]) to the MB-SMF corresponding to the MB-SMF ID stored in the AMF for the MBS Session ID. If it is the last RAN node controlled by the AMF serving the multicast MBS session identified by the MBS Session ID or both the MBS Session ID and Area Session ID (if exists), the leave indication is included.

4.	[Conditional] If unicast transport was used towards the NG-RAN node, the MB-SMF determines whether the context update is for tunnel release or create based on the release indication in the N2 container. If the MB-SMF determines the context update is for tunnel release and if the MBS session is active (the N3mb tunnel is provisioned in the MB-UPF), the MB-SMF sends N4mb Session Modification to the MB-UPF to release the N3mb tunnel used for the multicast MBS session (or location dependent content of the multicast MBS session if an Area Session ID was received) towards that RAN node using the received GTP tunnel info.

5.	[Conditional] The MB-SMF responses to the AMF with MBS Session ID and Area Session ID if received. If leave indication is received, the MB-SMF also removes the information of the AMF from the context of the multicast MBS session.

6.	The AMF removes the information of the RAN node from the context of the multicast MBS session (or location dependent part of the multicast MBS session if an Area Session ID was received).

7.	The AMF sends an N2 MBS Session release response (MBS Session ID, [Area Session ID]) to the RAN node. The NG-RAN node deletes the GTP tunnel info (if unicast transport is used for the shared delivery) or sends IGMP/MLD leave message to leave the multicast distribution tree (if multicast transport is used for the shared delivery). The NG-RAN node releases local resources for the multicast MBS session.

* * * * Option-2：Next Changes * * * *

#### 7.2.5.2	MBS session activation procedure

The following can trigger the MBS session activation procedure:

-	AF requests MB-SMF to activate the MBS session;

-	MB-UPF receives the multicast data and notifies MB-SMF.

Figure 7.2.5.2-1: MBS session activation procedure

In this procedure, steps 11 to 15 are executed if the MB-SMF finds out there are shared tunnel established. Steps 11 to 15, if needed, are executed in parallel with steps 2 to 10.

1.	The procedure may be triggered by the following events:

-	When the MB-UPF receives downlink data for a multicast MBS session, based on the instruction from the MB-SMF (as described in clause 7.2.5.3), the MB-UPF sends N4mb Notification (N4 Session ID) to the MB-SMF for indicating the arrival of DL MBS data.

-	The AF sends MBS Activation request (TMGI) to the MB-SMF directly or via NEF.

2.	MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify (MBS Session ID, multicast session state = Active) to SMF(s).

The SMF sets the related multicast MBS session state to Active and finds out the list of UEs that joined the multicast MBS session identified by the related TMGI. If the SMF determines the user plane of the associated PDU session(s) of the UE(s) with respect to the TMGI are activated already, steps 3-8a will be skipped for those UE(s), i.e. executed from step 8b.

3.	The SMF invokes Namf_MT_EnableGroupReachability Request (List of UEs, [PDU Session ID of the associated PDU Sessions], TMGI, [UE reachability Notification Address], [most demanding ARP, 5QI of all MBS QoS Flow within MBS session])) to AMF(s). When later UE is reachable, the UE reachability Notification Address is used by the AMF to identify and notify the related SMF.

After receiving the request, for each UE in the list, the AMF determines CM state of the UE: see steps 4 - 7.

4a.	If there are UEs involved in the multicast MBS Session and in CM-CONNECTED state, the AMF indicates those UEs to the SMF, using Namf_MT_EnableGroupReachability Response (UE list). Otherwise, the response does not include UE list.

4b.	For each UE in the UE list included in step 4a, if the QoS profile(s) for associated PDU Session has not yet been provided, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information (PDU Session ID, MBS Session ID, [QoS profile(s) for associated QoS flow(s)], [mapping information between the unicast QoS flow and multicast QoS flow])) to the AMF for the UE which is identified in step 4a. The associated unicast QoS Flow(s) as well as the mapping information between the unicast QoS Flow(s) and multicast QoS Flow(s) are included to support the 5GC Individual MBS traffic delivery.

The procedure continues at step 9.

5.	[Conditional] If AMF determines that there are UEs in CM-IDLE state and involved in the multicast MBS Session, the AMF figures out the paging area covering all the registration areas of those UE(s), which need to be paged. The AMF may apply paging differentiation as specified in clause 6.12. The AMF sends a Multicast Group paging request message to the NG-RAN node(s) belonging to this Multicast Paging Area with the involved UE list and TMGI as the identifier to be paged if the related NG-RAN node(s) support MBS. If the NG-RAN node(s) do not support MBS, the AMF sends Paging message(s) to the NG-RAN node(s) per UE as described in step 4b in clause 4.2.3.3 of TS 23.502 [6].

NOTE 1:	In addition to the paging in clause 6.12, other paging strategies is up to AMF implementation.

NOTE 2:	The details of the paging are specified by the RAN WGs.

6.	Receiving the paging, the UE(s) in CM-IDLE state sends Service Request message to the AMF, see clause 4.2.3 of TS 23.502 [6].

NOTE 3:	Step 6 for a UE can be parallel to step 5 for another UE(s), which has not received any paging yet.

7a/7b.	After receiving the Service Request sent by the UE(s),

-	Either based on the received PDU Session ID in step 3, the AMF identifies the related SMF and invokes Nsmf_PDUSession_UpdateSMContext request. The procedure continues at step 9; or:

-	Based on the received UE reachability Notification Address in step 3, the AMF identifies and notifies the related SMF of the UE(s), which are reachable now and the Location Information, by using the Namf_MT_UEReachabilityInfoNotify message. In this case, it can be a separated notification or combined with step 8.

8a.	For UE(s) that do not respond to paging, the AMF informs the SMF of the paging failure in Namf_MT_UEReachabilityInfoNotify.

8b.	For UE(s) that is indicated as reachable via the Namf_MT_UEReachabilityInfoNotify message, or user plane of the associated PDU session is activated already but the QoS profile(s) for associated QoS flow(s) needs to be provided for the PDU session, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information ()) to the AMF same as described in step 4b.

9.	The AMF sends N2 request message (N2 SM information ()) to the RAN node.

NOTE 4:	A joined UE is not able to receive MBS data if NG-RAN rejects the PDU Session Resource setup request (due to implementation specific reasons, e.g. activation of user plane fails due to the number of UEs reaching a limit).

10a.	If the shared tunnel has not been established before, the shared tunnel is established at this step, as defined in clause 7.2.1.4. The NG-RAN configures UE with RRC messages if needed.

10b.	Steps 8 to 12 defined in clause 7.2.1.3 are performed. If 5GC Individual MBS traffic delivery is used, the SMF configures the UPF for individual delivery and if necessary, requests the MB-SMF to configure the MB-UPF to send multicast data to the UPF.

11.	If the MB-SMF finds out there are shared tunnel established, step11-15 are performed. The MB-SMF invokes Namf_MBSCommunication_N2MessageTransfer Request (TMGI, N2 SM Information (Activation, TMGI)) to the AMF for those NG-RAN nodes, which have shared tunnel with MB-UPF. This step may be performed in parallel with step 2.

NOTE 5:	The messages in steps 10a, 11 and 12 are MBS-specific and it is possible that the AMF(s) in steps 10a, 11 and 12 are not associate to any UEs involved in the multicast MBS Session.

12.	The AMF sends NGAP activation request message (N2 SM Information ()) to the NG-RAN nodes. For those UEs that have joined in the MBS Session and are in RRC_INACTIVE state, the RAN nodes perform RAN paging as specified in TS 38.300 [9].

13.	The NG-RAN nodes responses to AMF by NGAP activation response message. The NG-RAN nodes establish radio resources to transmit multicast MBS session data to the UE(s). The NG-RAN shall not release the radio connection of a UE that has joined into the Multicast MBS session only because no unicast traffic is received for the UE.

14.	After receiving the first successful NGAP activation response message from the NG-RAN, the AMF sends Namf_MBSCommunication_N2MessageTransfer Response () to the MB-SMF.

15.	The MB-SMF sends N4mb Session Modification Request to provision N3mb DL tunnel(s) to the MB-UPF and request the MB-UPF to forward the receiving packet. The MB-UPF responds to the MB-SMF with N4mb Session Modification Response acknowledging the MB-SMF request. The N4mb interactions between MB-SMF and MB-UPF are the same as the ones between SMF and UPF, which are specified in clause 4.4 of TS 23.502 [6].

* * * * Option-2：Next Changes * * * *

#### 7.2.5.3	MBS session deactivation procedure

Figure 7.2.5.3-1: MBS session deactivation procedure

In this procedure, steps 3 to 4 and steps 5 to 9 are executed in parallel.

1.	The procedure may be triggered by the following events:

-	When MB-UPF detects there is no data receives for the MBS Session, MB-UPF sends MB-N4 Notification (N4 Session ID) to the MB-SMF for deactivating the MBS session.

-	AF sends MBS Deactivation request (TMGI) to the MB-SMF directly or via NEF.

2.	The MB-SMF send N4mb Session Modification Request (TMGI, Buffered Downlink Traffic detection) to the MB-UPF. The N4mb interactions between MB-SMF and MB-UPF are the same as the ones between SMF and UPF, which are specified in clause 4.4 of TS 23.502 [6]. The Buffered Downlink Traffic detection is requested by MB-SMF for next time MBS session activation. If the MBS session is to be activated via the AF request directly, this indication is not needed. The MB-SMF maintains the N3mb DL tunnels, and indicates the MB-UPF to remove all shared tunnel(s). For Individual MBS traffic delivery over N19mb interface, there are no needs for the SMF(s) to invoke Nmbsmf_MBSSession_ContextUpdate to the MB-SMF after the UPF(s) release the N19mb DL tunnel(s). For Shared MBS traffic delivery over N3mb interface, the MB-SMF will provision active N3mb DL tunnels(s) to the MB-UPF when MBS session is activated, as specified in clause 7.2.5.2.

MB-UPF to MB-SMF: N4mb Session Modification Response acknowledging the MB-SMF request.

3.	The MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify request (MBS Session ID) to the SMFs.

Based on the received MBS Session ID, the SMF sets the indicated multicast MBS session state to Inactive:

‐	If the SMF finds out there are UE(s) that joined the indicated multicast MBS session and use 5GC Individual MBS traffic delivery, step 4 is performed for those UE(s).

‐	If SMF find there are no UE(s) that joined the indicated MBS session and use 5GC Individual MBS traffic delivery, no further operation for SMF is required.

4.	[Conditional] For those UE(s) that the 5GC individual delivery is used, step 3b and steps 4-8 in clause 4.3.3.2 of TS 23.502 [6] are performed to remove the associated QoS flow(s) related to the multicast MBS session.

NOTE 1:	Whether the associated QoS Flow(s) are removed from UE, NG-RAN, or only resource in NG-RAN is removed is up to implementation.

5.	If the MB-SMF finds out there are shared tunnel established over N3mb interface, the MB-SMF sends Namf_MBSCommunication_N2MessageTransfer Request (TMGI, N2 SM information (Deactivation, TMGI)) to the AMFs.

6.	The AMF sends NGAP deactivation request message (N2 SM information ()) to the NG-RAN nodes.

7.	The NG-RAN node keeps the multicast MBS Session Context and N3mb shared tunnel for the multicast MBS session.

If the MBS Session Context indicates no UE for the multicast MBS session (e.g. due to UE becomes CM-IDLE state), the NG-RAN triggers release of the shared delivery as described in clause 7.2.2.4.

8.	NG-RAN acknowledges the NGAP deactivation Response message.

9.	The AMF invokes Namf_MBSCommunication_N2MessageTransfer Response to acknowledge the service for MB-SMF.

When the MBS session is in "Inactive" state and handover procedure is triggered, it is defined in clause 7.2.3.6.

NOTE 2:	There is no explicit "deactivation" indication to the UE, how the UE is changed to IDLE state is defined in TS 38.300 [9].

* * * * End of Changes * * * *