SA WG2 Meeting #153e	S2-2208376

# October 10th – 14th, 2022; Elbonia            		  		(revision of S2-220xxxx)

Title:	[draft] Reply LS on handling of PDU sessions for emergency services when registering via both 3GPP and non-3GPP accesses

Response to:	C1-225230 / S2-2208123

Release:	Rel-18

Work Item:	5GProtoc18-non3GPP, TEI17

Source:	Nokia, Will be SA2

To:	CT1, CT4

Cc:

Contact Person:

#### Name:	 Hannu Hietalahti

E-mail Address:	 Hannu.Hietalahti@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR 3711, TS 23.502 CR 3561

1. Overall Description:

SA2 thanks CT1 for their LS. SA2 have discussed it and has agreed the attached CR to address the problem that was identified by CT1.

SA2 has already clarified earlier in TS 23.501 CR 3659 that the UE shall have at most one emergency PDU session, but the existing emergency PDU session can be transferred from one Access Type to another. The SMF requirement in TS 23.501 clause 5.16.4.9 is intended to prevent the UE from establishing a parallel emergency PDU session. As it was discovered that the SA2 specification is in conflict with the CT1 and CT4 specifications in the same area, SA2 has aligned the stage 2 with the stage 3 requirements in the attached CR.

SA2 would like to re-use the already specified error handling by which the SMF shall consider a new PDU Session establishment request as a request to remove the existing PDU Session and to establish a new one as requested by the UE. If the SMF receives PDU Session establishment request from the same UE on the same PDU Session ID, according to TS 29.502, the SMF shall already locally delete the old PDU Session and proceed with the new request.

The attached CR on TS 23.501 re-uses the same already existing mechanism to enforce that the maximum number of one emergency PDU Session per UE is not exceeded with one exception. The difference from the normal error handling case is that establishment of new emergency PDU session overrides possibly existing old PDU Session irrespective of the PDU Session ID.

2. Actions:

To CT1 and CT4:

ACTION: 	SA2 kindly asks CT1 and CT4 to take the above information into account and to align their specifications with it as appropriate.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#154 | November, 14-18th | Toulouse | France |
| SA2#154AH | January, 16-20th | Any (e-meeting) | Elbonia |