SA WG2 Meeting #153e	S2-2208383

October 10th – 14th, 2022; Elbonia               	(revision of S2-220xxxx)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI#1: resolve EN in conclusions

Document for: 	Agreement (P-CR)

Agenda Item: 	9.26

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract of the contribution: This contribution proposes to update conclusions for KI#1 by resolving ENs.

# 1 Discussion

KI#1 conclusions contains an editor's note on the target UE performing UE-to-UE Relay reselection for Model B discovery.

Possible collision situations should be taken into account when considering which discovery model to use for UE-to-UE Relay reselection. Any possible conflicts are removed, if the end-point UEs negotiate the discovery model using discovery message.

UE-to-UE Relay discovery integrated to the PC5 unicast link establishment can be used for UE-to-UE Relay selection but as the discovery can be performed for different reasons, it cannot be used for UE-to-UE Relay discovery.

The conditions to use Model A and Model B discovery are added.

The condition to use the negotiated UE-to-UE Relay reselection between the Source UE and the Target UE Since the UE-to-UE discovery integrated with PC5 unicast link establishment can be used already for the UE-to-UE Relay selection.

# 2 Proposal

It is proposed to update the Key Issue #1 conclusion in TR 23.700-33 as follows.

*** First Change ***

## 8.1	Key Issue #1: Support of UE-to-UE Relay

For Key Issue #1 (Support of UE-to-UE Relay), the followings are taken as conclusions:

The following conclusions are common for both Layer-3 UE-to-UE Relay and Layer-2 UE-to-UE Relay:

-	For UE-to-UE Relay discovery, both Model A and Model B discovery are supported.

Editor's note:	It is FFS the exact information to be included in discovery messages.

-

-	UE-to-UE relay discovery integrated into PC5 unicast link establishment procedure is supported for UE-to-UE Relay selection but it is not supported for UE-to-UE Relay discovery.

-	For UE-to-UE Relay selection, the Source UE performs the UE-to-UE Relay selection for both Model A and Model B discovery.

-	For UE-to-UE Relay selection, the Target UE does not perform the UE-to-UE Relay selection for Model B discovery. The Target UE performs the UE-to-UE Relay selection in case the UE-to-UE relay discovery is integrated into PC5 unicast link establishment procedure, i.e., upon receiving a Direct Communication Request from the Source UE via one or more UE-to-UE Relay UEs.

-	The negotiated UE-to-UE Relay selection procedure and the reselection between Source UE and Target UE can be used for selection and reselection of the UE-to-UE Relay under different conditions.

NOTE 1:	UE-to-UE Relay selection/reselection requires cooperation with RAN WGs during normative work.

-	IP, Ethernet and Unstructured traffic types are supported.

NOTE 2:	Ethernet and Unstructured traffic types can be encapsulated in IP traffic type if supported by source and target UE.

NOTE 3:	Conclusion on Service Authorization information related to UE-to-UE Relay operation sent by AMF to NG-RAN can be finalized based on feedback from RAN WGs.

The following conclusions are specific for Layer-3 UE-to-UE Relay:

-	For UE-to-UE Relay Per-hop links setup (i.e. PC5 link establishment between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE), Source UE initiates the PC5 link setup with UE-to-UE Relay, and UE-to-UE Relay initiates the PC5 link setup with the target UE.

-	For QoS control of Layer-3 UE-to-UE Relay, the UE-to-UE Relay receives E2E QoS from Source UE and determines the per-hop QoS parameters to satisfy the E2E QoS.

The following conclusions are specific for Layer-2 UE-to-UE Relay:

-	Per-hop links (i.e. PC5 link between Source UE and UE-to-UE Relay, as well as between UE-to-UE Relay and Target UE) needs to be established before E2E PC5 link establishment is performed.

NOTE 4:	How the E2E PC5-S messages are forwarded by the UE-to-UE Relay is to be determined by RAN WGs.

NOTE 5:	For Layer-2 UE-to-UE Relay, RAN WGs will define how the E2E QoS will be handled and split over the PC5 links.