3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208386

Elbonia, October 10 – 17, 2022

Source:	Huawei, HiSilicon

Title:	KI#7: update of Solution#20

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: Updating Solution#20: CAG based UE access control via MBSR.

1. Discussion

Solution#20 proposes to use the existing CAG, Closed Access Group, concept as a base to manage the UE access control via MBSR. There is an EN about the CAG identifier of MBSR.

Editor's Note: whether this is a CAG or whether this will have to be an independent identifier supported from rel-18 or both the CAG and a new identifier could be considered is FFS.

Considering the legacy UE(s) shall support to access the MBSR cell, it is not necessary to indicate UE whether a CAG cell is MBSR cell or normal cell. Therefore it is proposed to remove the EN and updates the solution accordingly.

In addition, when the MBSR is roaming and allowed to operate as an IAB node of a VPLMN, the VPLMN OAM shall configure a CAG Identifier valid in the VPLMN. Therefore, the CAG ID broadcasted by MBSR depends on the PLMNs in which the MBSR is allowed to operate as as an IAB node.

Figure 1: Broadcasting different CAG Identifiers for different PLMN

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-05 V1.0.0.

# * * * * First change * * * *

6.20	Solution #20: CAG based UE access control via MBSR

6.20.1	Introduction

This solution relates to KI#7 "Control of UE’s access to 5GS via a mobile base station relay". The solution proposes to use the existing CAG, Closed Access Group, concept as a base to manage the UE access control via MBSR.

There are certain simplifications on the existing CAG logic to fit into the IAB architecture.

6.20.2	Functional Description

When a UE tries to select or connect to 5GS via a MBSR (a mobile IAB-node), the access control of the UE is managed by CN, RAN and UE. During the CM-CONNECTED mobility, the access control via MBSR (mobile IAB-node) is managed by CN and RAN.

6.20.2.1  CAG identifier of MBSR

A CAG identifier shall be assigned for the MBSR node to use and it is unique within the scope of a PLMN ID. The CAG identifier is announced by the cells covered by the MBSR. The CAG identifier of MBSR cell may be different from the CAG identifier of normal cell. If the MBSR is pre-configured with the PLMN list in which the MBSR is allowed to operate as an IAB node, the corresponding CAG Identifier per PLMN is also configured in the MBSR.

6.20.2.2  UE CAG configuration

To access the CAG cell of MBSR, the UE, that supports CAG as indicated as part of the UE 5GMM Core Network Capability, is configured with the CAG information as described in the clause 5.30.3.3 of TS 23.501[2].

Considering the roaming scenario, the CAG information per PLMN ID shall be configured to UE.

6.20.2.2  Access control

RAN and CN supports the UE access control based on the CAG identifier associated with the MBSR cell and the allowed CAG identifiers for the UE.

For UEs that support CAG, they may select the cells covered by a MBSR based on the configured MBSR CAG identifier using the existing CAG control logic. The legacy UE(s) that support CAG shall also support to access the MBSR cell based on the CAG identifier.

For the UEs that do not support CAG (i.e. the UE is not able to understand the broadcasted CAG information by the cell) but supports the use of IAB architecture, it may select the cell as normal and rely on RAN and CN to control the IAB-node access using network side CAG control logic.

See more details of the CAG concept in clause 5.30.3 of TS 23.501 [2].

6.20.3	Procedures

6.20.3.1	General

The CAG control is well described in different procedures in TS 23.502 [5]. Procedure details may be updated to support the additional control in case of MBSR CAG Identifier.

The CAG specific procedures, involving RAN/AS, are defined in TS 38.300 [20] clause 16.7. These procedures allow the control of:

-	whether new UEs (Rel-18 onward), or also legacy UEs, can access to a mobile-IAB CAG cell;

-	whether non-CAG capable UEs can access, or not, to a mobile-IAB CAG cell (as in TS 38.304 [21]).

A mobile-IAB CAG cell may also be configured with a human-readable network name, to be signalled to the UE for presentation to the user.

6.20.4	Impacts on services, entities and interfaces

AMF:

-	Supports baseline CAG control based on MBSR CAG identifier.

UDM:

-	Supports storing CAG information of MBSR as UE’s Subscription data. .

gNB/IAB-node:

-	Supports CAG function.

UE:

-	Supports CAG function.

NOTE:	Some of the potential CAG extensions and additional control mechanisms may have RAN dependencies.

# * * * * End of changes * * * *