3GPP TSG SA WG 2 Meeting 153E		S2-2208392

Elbonia 10 – 17 October 2022

Title:	[Draft] LS on XR and Media Services

Response to:	LS on UE Power Saving for XR and Media Services (R1-2205531/S2-2205421)

Release:	Rel-18

Work Item:	FS_XRM, FS_NR_XR_enh

Source:	SA WG2

To:	RAN WG1, RAN WG2, RAN WG3

Cc:

Contact person:	Paul Schliwa-Bertling

Paul.Schliwa-Bertling(at)Ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	DocNumber(s) [Description e.g.. Draft TS 29.414 v0.1.0]. 
!! WARNING !! Do not insert the file directly as an object in this Word document.

# 1	Overall description

SA2 thanks RAN1 for the LS on UE Power Saving for XR and Media Services (R1-2205531/S2-2205421) that SA2 considers in context of the Rel-18 study on XR and Media services (FS_XRM).

SA2 would like to inform RAN1, RAN3 and RAN2 that the FS_XRM study for Rel-18 has produced TR 23.700-60. Some conclusions that SA2 arrived at in this study have RAN dependencies for which SA2 seeks RAN WGs feedback.

Concerning KI#3: 5GS information exposure for XR/media enhancements:

SA2 concluded to introduce support for L4S using ECN bits marking in the inner user plane IP header for both, UL and DL on a given QoS Flow. The purpose is to signal to the application layer (data source) that, to maintain certain latency, application layer rate adaptation is to be applied. The criteria for NG-RAN to determine the event when to perform the L4S marking is up to NG-RAN implementation. Two variants of L4S marking are concluded to be introduced, (1) L4S marking in the NG-RAN node and (2) L4S marking by the PSA UPF based on information provided by NG-RAN. SA2 would like to ask RAN2 and RAN3 following questions:

Q1: For the case of variant (1), is it feasible for NG-RAN to perform L4S marking on a given QoS Flow?

Q2: For the case of variant (2) where the PSA UPF performs L4S marking, is it feasible for NG-RAN to indicate in GTP-U to UPF the event for L4S marking on a given QoS Flow?

SA2 would also like RAN WGs´ feedback regarding the support for API based exposure via N3 and N2 of following information:

- Congestion level information

- Round-trip delay for a given QoS Flow and combination of QoS Flows

- Data rate

Concerning KI#x [TBD, Questions according to the conclusions]

# 2	Actions

To RAN WG1, RAN WG2, RAN WG3

ACTION: 	SA2 kindly ask RANx and [TBD] to provide feedback as indicated.

# 3	Dates of next TSG SA WG2 meetings

TSG-SA2 Meeting #154	14-20 November 2022	Toulouse, France

TSG-SA2-Ad hoc Meeting 16-20 January 2023	TBD