3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2206106

Elbonia, Oct10th – 17th, 2022	(revision of S2-220xxxx)

Source:	China Telecom

Title:	Update of Solution #21 for FS_GMEC KI#1

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing an update of Maximum Bit Rate for a 5G VN group.

1. Introduction/Discussion

It is proposed to update the solution #21 to solve EN

1. The first EN in 6.21.2.1

Editor's note: It is FFS whether the GSMF functionality is better handled as part of an existing NF.

The EN is regarding the text here: “A Group Session Management Function (GSMF) is introduced in 5GC control plane. GSMF is responsible for storing the status of the activated 5G VN group members and their maximum bit rate.”

Option 2： Monitor and limit the bit rate of each VN group through PCF and UDR.

-	The 5G VN group data is extended to include Group-MBR. Group-MBR limits the total bit rate that can be expected to be provided across all sessions of a 5G VN group.

-	The existing UDM parameter provisioning service for 5G VN group data provisioning is re-used, with the difference that the 5G VN group data may contain a Group-MBR as a new input parameter. The Group-MBR is also stored as part of the 5G VN group data in UDR.

-	The existing NEF parameter provisioning service for 5G VN group data provisioning is re-used, with the difference that the 5G VN group data may contain a Group-MBR as a new input parameter.

-  If GSMF functionality is handled as part of PCF, PCF should supports monitoring and limitation of the data rate per VN Group. The VN Group policy control information is per VN Group information stored by the UDR and updated by the PCF during PDU Session Establishment or Modification procedure using Nudr service. The VN Group policy control information includes Group MBR and Remaining Group MBR.

-  At the establishment of a SM Policy Association, the PCF shall check whether the Remaining Group MBR is higher than the Authorized Session-AMBR. If so, the PCF shall deduct the value of the Authorized Session-AMBR allocated to the PDU Session from the Remaining Group MBR for that VN Group in the UDR. If the Remaining Group MBR is not sufficient, the PCF shall reject the establishment of the SM Policy Association.

-  If the Remaining Group MBR for that VN Group reaches a (operator defined) threshold that indicates that it is closer to zero, the PCF may apply a policy decision to strengthen the traffic restrictions for individual PDU Sessions or PCC rules and interact with the SMF accordingly. When the Remaining Group MBR for that Group increases again, the PCF may relax the traffic restrictions for individual PDU Sessions or PCC rules.

-  At the time a change of Authorized Session-AMBR occurs, the PCF shall update the Remaining Group MBR for that VN Group in the UDR accordingly.

-  At the time the SM Policy Association for an VN Group subject to data rate limitation is terminated, the PCF shall add the value of the Authorized Session-AMBR allocated to the PDU Session to the Remaining Group MBR for that VN Group in the UDR.

Proposal: Add the above texts to the TR.

2. The secound EN in 6.21.3.1

Editor's note: This solution supports policy control when members in VN group belong to different PCF.

Both options 1 and option 2 support multiple PCFs for the same VN group.

Proposal: Delete this EN.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

6.21	Solution #21: Support for attribute of Maximum Bit Rate for a 5G VN group

6.21.1	Introduction

This solution aims to address the KI#1: Enhance group attribute management

6.21.2	Functional Description

6.21.2.1	Attribute of Maximum Bit Rate for a 5G VN group

This solution proposes a method for management and control of maximum bit rate for a 5G VN group, with following principles:

Option 1： Add a new network function GSMF

-	A Group Session Management Function (GSMF) is introduced in 5GC control plane. GSMF is responsible for storing the status of the activated 5G VN group members and their maximum bit rate.

-	The 5G VN group data is extended to include Group-MBR. Group-MBR limits the total bit rate that can be expected to be provided across all sessions of a 5G VN group.

-	The existing UDM parameter provisioning service for 5G VN group data provisioning is re-used, with the difference that the 5G VN group data may contain a Group-MBR as a new input parameter. The Group-MBR is also stored as part of the 5G VN group data in UDR. GSMF can obtain Group-MBR from UDM.

-	The existing NEF parameter provisioning service for 5G VN group data provisioning is re-used, with the difference that the 5G VN group data may contain a Group-MBR as a new input parameter.

-	When a SMF completes the process of a 5G LAN PDU session establishment.

-	SMF sends a group update request to GSMF. The request message includes UE ID, 5G VN group ID, Session-AMBR for Non-GBR QoS flow and MFBR for GBR QoS flow, etc.

-	GSMF updates and stores the sum of the Session-AMBR and MFBR for GBR QoS Flows of all PDU Sessions in the VN Group.

-	The SM Policy Association establishment/modification message sent by SMF to PCF shall include the 5G VN group ID.

-	When a PCF receives SM Policy Association establishment/modification message from SMF:

-	PCF query GSMF to retrieve the group information.

-	The group information provided by GSMF includes:

-	The sum of the Session-AMBR and MFBR for GBR QoS Flows of all PDU Sessions in the VN Group.

-	Group-MBR.

-	Based on the group information provided by GSMF, PCF makes corresponding decisions.

-	When a 5G LAN PDU session is released:

-	SMF sends a group update request message to GSMF.

-	GSMF updates the sum of the Session-AMBR and MFBR for GBR QoS Flows of all PDU Sessions in the VN Group.

Figure 6.21.2.1-1 depicts the architecture :

Figure 6.21.2.1-1: Architecture for option 1

Option 2： Monitor and limit the bit rate of each VN group through PCF and UDR

-	The 5G VN group data is extended to include Group-MBR. Group-MBR limits the total bit rate that can be expected to be provided across all sessions of a 5G VN group.

-	The existing UDM parameter provisioning service for 5G VN group data provisioning is re-used, with the difference that the 5G VN group data may contain a Group-MBR as a new input parameter. The Group-MBR is also stored as part of the 5G VN group data in UDR.

-	The existing NEF parameter provisioning service for 5G VN group data provisioning is re-used, with the difference that the 5G VN group data may contain a Group-MBR as a new input parameter.

-  PCF should supports monitoring and limitation of the data rate per VN Group. The VN Group policy control information is per VN Group information stored by the UDR and updated by the PCF during PDU Session Establishment or Modification procedure using Nudr service. The VN Group policy control information includes Group MBR and Remaining Group MBR.

-  At the establishment of a SM Policy Association, the PCF shall check whether the Remaining Group MBR is higher than the Authorized Session-AMBR. If so, the PCF shall deduct the value of the Authorized Session-AMBR allocated to the PDU Session from the Remaining Group MBR for that VN Group in the UDR. If the Remaining Group MBR is not sufficient, the PCF shall reject the establishment of the SM Policy Association.

-	At the time a PCC Rule of a GBR service data flow is installed, modified or activated in the SMF, the PCF requests reporting "Successful resource allocation" with the corresponding Policy Control Request Trigger. When the SMF reports that the resources are successfully allocated, the PCF shall deduct the value of the MBR authorized for the GBR service data flow from the Remaining Group MBR for that VN Group in the UDR.

-  If the Remaining Group MBR for that VN Group reaches a (operator defined) threshold that indicates that it is closer to zero, the PCF may apply a policy decision to strengthen the traffic restrictions for individual PDU Sessions or PCC rules and interact with the SMF accordingly. When the Remaining Group MBR for that Group increases again, the PCF may relax the traffic restrictions for individual PDU Sessions or PCC rules.

-  At the time a change of Authorized Session-AMBR occurs, the PCF shall update the Remaining Group MBR for that VN Group in the UDR accordingly.

-  At the time the SM Policy Association for an VN Group subject to data rate limitation is terminated, the PCF shall add the value of the Authorized Session-AMBR allocated to the PDU Session to the Remaining Group MBR for that VN Group in the UDR.

6.21.3	Procedures

6.21.3.1	Procedure for option 1 when a UE initiates a 5G LAN PDU session establishment request

Figure 6.21.3.1-1 outlines the procedure for support the attribute of Group-MBR for a VN group when a UE initiates a 5G LAN PDU session establishment request.

Figure 6.21.3.1-1: Procedures for support of the attribute of maximum number of users for a VN group when a UE initiates a 5G LAN PDU session establishment request

1. Based on local configuration or signal trigger, GSMF subscribe to 5G VN Group data from UDM.

2. UE sends PDU Session Request to the SMF.

3. SMF sends SM Policy Association Establishment Request. The message shall include the 5G VN group ID.

4. PCF performs GSMF selection through local configuration or NRF.

Editor's note:	It is FFS what parameters GSMF carries when registering to NRF. This may be further analysed when the functionality of the GSMF is clearer.

5. PCF query GSMF to retrieve the group information.

6. GSMF provide group information. The Group information include Group-MBR and the sum of the Session-AMBR and MFBR for GBR QoS Flows of all PDU Sessions in the VN Group.

7. PCF makes policy decisions based on message received from GSMF.

If the sum of bit rate allocated for the PDU sessions currently active UEs of a 5G VN group reached Group-MBR , PCF can make the following decisions:

a)	PCF reject the establishment of the SM Policy Association and provides an indication to the SMF indicating the Group-MBR is overflown.

b)	Other decisions

Editor's note:	It is FFS whether PCF needs to make other decisions.

8. PCF sends SM Policy Association Establishment Response (clause 4.16.4 of TS 23502).9. SMF sends PDU session Setup Response to UE.

If the PCF provides an indication to SMF indicating the Group-MBR is overflown, SMF shall reject the PDU Session with a suitable cause code.

10. If the new PDU session is accepted, SMF sends group information update request to GSMF. The request message includes UE ID, 5G VN group identifier, Session-AMBR for Non-GBR QoS flow and MFBR for GBR QoS flow.

11. GSMF updates and stores the group information.

12. GSMF sends Response to SMF with successful update indication.

6.21.3.2	Procedure for option 1 when a UE or network initiates a 5G LAN PDU session release request

Figure 6.21.3.2-1 outlines the procedure for support of the attribute of Group-MBR for a VN group when a UE or network initiates a 5G LAN PDU session release request.

Figure 6.21.3.2-1: Procedures for support of the attribute of Group-MBR for a VN group when a UE or network initiates a 5G LAN PDU session release request

1.	Step 1 follows the UE or network initiated PDU session release procedure already defined in TS 23.502 [3].

2.	SMF completes the 5G LAN PDU session release produce, then it sends a group information update request message to GSMF, which contains VN Group ID, UE ID, etc.

3.	GSMF updates the sum of the Session-AMBR and MFBR for GBR QoS Flows of all PDU Sessions in the VN Group.

4.	GSMF sends a Response to the SMF with successful update indication.

6.21.3.3 Procedure for option 2

When option 2 is selected，this solution does not need to add new network function and interfaces.

The procedure is the same as PCF monitoring and limiting the data rate of each network slice.

6.21.4  Impacts on existing entities and interfaces

Editor's note:	This clause lists impacts to existing entities and interfaces.

# * * * * End of changes * * * *