3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208412

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion of KI#2

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_ph3 / Rel-18

Abstract: It is proposed to have evaluation and conclusion of KI#2.

1. Introduction/Discussion

There are 6 solutions for KI#2. The details are listed in the following table:

| Solutions | Extended SoR | Legacy SoR generated based on slice info | URSP |
| --- | --- | --- | --- |
| Solution #6: Extended SoR VPLMN Slice Information transfer to UEs | X |  |  |
| Solution #7: Enabling awareness of Network Slice availability in VPLMNs | X |  |  |
| Solution #16: UE assisted slice based VPLMN prioritization for Extended SoR | X |  |  |
| Solution #17: Slice based VPLMN Selection Policy |  |  | X |
| Solution #18: Sending rejected NSSAI to the UDM to assist the UDM to steer the UE to the PLMN supporting rejected NSSAI |  | X |  |
| Solution #19: configuring the UE with network slice aware preferred PLMNs lists | X |  |  |
| Solution #20: VPLMN Selection following existing SoR information |  | X |  |

The solutions can be divided into three categories.

- Extended SoR based solution

This kind of solutions requires UE to be aware of new types of SoR. And then UE could selece VPLMN accordingly. It gives more freedom to the UE but this solutions cannot be used by legacy UEs. Besides, there can be security issues to send the supporting S-NSSAIs of a PLMN before the UE access to it, especially for the S-NSSAIs requiring NSSAA.

- Legacy SoR generated based on slice info

UE will still use the legacy SoR but the SoR is generated based on the slices that the UE may want to access. This kind of solutions could be used for legacy UEs. Solution #18 will expose potentially parts or all of the S-NSSAIs that the VPLMN supports.

- URSP based solution

UE will determine whether to check the SoR info in sequence based on the URSP. The UE will know whether an application requires a specific S-NSSAI. This kind of solution also requires UE to understand the new URSP. This kind of solution also requires UE to understand the new URSP. The registered VPLMN can inspect the new URSP.

Considering there are solutions that could be used for legacy UEs, it is propose to adopt such kind of solutions in normative phase.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change * * * *

7.x	Evaluation for KI#2

The solutions can be divided into three categories.

- Extended SoR based solution (Solution #6, #7, #16 and #19)

This kind of solutions requires UE to be aware of new types of SoR. And then UE could select VPLMN accordingly. It gives more freedom to the UE but this solutions cannot be used by legacy UEs. Besides, there can be security issues to send the supporting S-NSSAIs of a PLMN before the UE access to it, especially for the S-NSSAIs requiring NSSAA.

- Legacy SoR generated based on slice info (Solution #18 and #20)

UE will still use the legacy SoR but the SoR is generated based on the slices that the UE may want to access. This kind of solutions could be used for legacy UEs. Solution #18 will expose potentially parts or all of the S-NSSAIs that the VPLMN supports.

- URSP based solution (Solution #17)

UE will determine whether to check the SoR info in sequence based on the URSP. The UE will know whether an application requires a specific S-NSSAI. This kind of solution also requires UE to understand the new URSP. The registered VPLMN can inspect the new URSP.

Editor’s note: Whether the above solutions have security issue is subject to the feedback of SA3.

# * * * * Second change * * * *

8.X	Conclusions for Key Issue #2

It is agreed to adopt the Solutions based legacy SoR as defined in current TS 23.122 [7] in normative phase with following principles.

-  The SoR-AF can generate SoR based on Subscribed S-NSSAIs, Requested NSSAI and/or Expected NSSAI.

-  The UE can send Requested NSSAI and/or Expected NSSAI to the UDM for SoR generation. UDM can further send them to the SOR-AF for SoR generation.

-  The UDM can provide Subscribed S-NSSAIs to the SoR-AF for SoR generation.

-  There is no update on the SoR information.

# * * * * End of changes * * * *