3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208413

Elbonia, October 10 – 17, 2022	(revision of S2-2205713)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion of KI#3

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: It is proposed to have evaluation and conclusion of KI#3.

1. Introduction/Discussion

The evaluation of KI#3 categorizes the solutions and 5 categories:

There are 10 solutions for KI#3. The details are shown in Table 1-1

| Solutions | Temporary slice | Smaller Applicable area of service | RAN impact | UE impact | Remark |
| --- | --- | --- | --- | --- | --- |
| Solution #8: Gracefully network slice termination | X |  |  |  |  |
| Solution #9: Support of a Network Slice with an AoS not matching existing TA boundaries |  | X | X | X |  |
| Solution #10: Associating a validity timer with a temporary slice | X |  |  | X |  |
| Solution #11: Enabling UEs to Request S-NSSAIs not uniformly available | X | X | Maybe | X |  |
| Solution #21: Temporary slice based on URSP | X | X |  |  |  |
| Solution #22: Enabling graceful slice termination with support of UE policies | X | X |  | X |  |
| Solution #23: UE registration for conditional network slices |  | X |  | X | Not sure how it resolve KI#3 |
| Solution #24: On the handling temporary network slices | X |  | X | X |  |
| Solution #26: Multiple areas and resource partitioning |  | X | X | X |  |
| Solution #29: On handling S-NSSAIs not supported in certain TAs of a RA during a registration |  | X | X | X | Part of solution #9 |
| Solution #44: Controlling UE access to the network per Network Slice on a per cell level granularity |  | X | X | X |  |
| Solution #45: Constrained Service Area for the Network Slice |  | X | X |  |  |

The solutions can be divided into 3 categories:

- Solutions only for temporary slice (Solution #8, #10 and #24)

Solution #8 proposes UDM will indicate validity timer to the AMF, then AMF could release the PDU Session gracefully. However, there is no difference from UE perspective if OAM has removed the slice. (The AMF also releases the PDU Session accordingly.) So, it cannot release the PDU Session gracefully as there can be an potential traffic initiated by the UE.

Solution #10 proposes to further send the validity timer to the UE. Then the UE could finish its service gracefully. But the UE needs to understand the new parameter.

Solution #24 is similar as Solution #10 but further proposes periodically available S-NSSAI. But not sure it is the scenario mentioned by the KI.

- Solutions only for smaller applicable area of service  (Solution #9/29, #23, #26, #44 and #45)

Solution #9 proposes multiple-level TA, which does not break the unified TA assumption but has big impact on the RAN and UE.  The smaller areas of service can be implicitly implemented by the Secondary TAs, but there is no detail about the procedures of service request. Solution #29 is part of Solution #9 on the aspect of RA determination/handling. But the detailed relationship between them needs to be further clarified.

Solution #23 claims it could resolve K#3 but there is no description on a finer applicable area of the service.

Solution #26 supports reducing resources for cells outside service area. For the cells outside the service area, but within the TA, the network slice is available but cannot support the KPIs as agreed to be available within the service area.

Solution #44 supports extension with a per cell level granularity per network slice for Service Area Restrictions. The NG-RAN and UE support extension of Service Area Restrictions per cell level granularity per network slice. It breaks the unified TA assumption.

Solution #45 proposes constrained service area with cell granularity per S-NSSAI is configured in the RAN to improve radio resources usage and avoid TA reallocation. In the condition that other services are unaffected, the RAN prevents the UE from entering into the constrained service area (or leaving service area) when any PDU session related to the S-NSSAI has an active user plane. It has no impact on the UE. It can work together with other solutions supporting smaller applicable area of service.

- Solutions for both (Solution #11, #21 and #22)

The rationale of these solutions is the same, i.e. let UE be aware of the validity timer of the slice or applicable area of the service. Solution #11 proposes to solve both issues by the AMF while Solution #21 and #22 are both URSP based solutions. Similar as Solution #10 in Category 1), it let the UE be aware of the validity timer (in Conditional NSSAI). The UE needs to understand the new parameter. Solution #21 and #22 propose UE to be aware of the validity timer via the existing parameter. Similar logic on the applicable area of the service.

Based on the above discussion, it is not necessary to introduce new parameter on the validity timer of the slice or applicable area of the service to the UE. URSP could be reused. Then other more complicated mechanism can also be abandoned.  It is proposed to adopt URSP based solution in normative phase.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change (all new text) * * * *

7.3	Evaluation for KI#3

The solutions can be divided into 3 categories:

- Solutions only for temporary slice (Solution #8, #10 and #24)

Solution #8 proposes UDM will indicate validity timer to the AMF, then AMF could release the PDU Session gracefully. However, there is no difference from UE perspective if OAM has removed the slice. (The AMF also releases the PDU Session accordingly.) So, it cannot release the PDU Session gracefully as there can be an potential traffic initiated by the UE.

Solution #10 proposes to further send the validity timer to the UE. Then the UE could finish its service gracefully. But the UE needs to understand the new parameter.

Solution #24 is similar as Solution #10 but further proposes periodically available S-NSSAI. But not sure it is the scenario mentioned by the KI.

- Solutions only for smaller applicable area of service  (Solution #9/29, #23, #26, #44 and #45)

Solution #9 proposes multiple-level TA, which does not break the unified TA assumption but has big impact on the RAN and UE.  Solution #29 is part of Solution #9 on the aspect of RA determination/handling. But the detailed relationship between them needs to be further clarified.

Solution #23 claims it could resolve K#3 but there is no description on a finer applicable area of the service.

Solution #26 supports reducing resources for cells outside service area by the network configuration. For the cells outside the service area, but within the TA, the network slice is available but cannot support the KPIs as agreed to be available within the service area. However, it is unclear how the network enforce the smaller areas of service.

Solution #44 supports per cell level granularity per network slice for Service Area Restrictions, which breaks the unified TA assumption. The NG-RAN and UE support extension of Service Area Restrictions per cell level granularity per network slice.

Solution #45 proposes the constrained service area with cell granularity per S-NSSAI is configured in RAN to improve radio resource usage and avoid TA reallocation. In the condition that other ongoing services are unaffected, the RAN optimizes handover to prevent the UE from entering into the constrained service area (or leaving the service area) if any active PDU session relates to the S-NSSAI. Solution #45 has no impact on the UE and can work together with other solutions supporting smaller applicable area of service.

- Solutions for both (Solution #11, #21 and #22)

The rationale of these solutions is the same, i.e. let UE be aware of the validity timer of the slice or applicable area of the service. Solution #11 proposes to solve both issues by the AMF while Solution #21 and #22 are both URSP based solutions. Similar as Solution #10 in Category 1), it let the UE be aware of the validity timer (in Conditional NSSAI). The UE needs to understand the new parameter. Solution #21 and #22 propose UE to be aware of the validity timer via the existing parameter. Similar logic on the applicable area of the service.

# * * * * Second change (all new text)* * * *

8.3	Conclusions for Key Issue #3

It is agreed to adopt following principles of URSP based solutions (i.e. #21 and #22) for legacy UEs in normative phase.

-  The PCF should generate URSP based on Network Slice Area of Service for services not mapping to existing TAs boundaries and Temporary network slices related information stored in the UDR.

-  UE can be aware of the validity timer of the slice or applicable area of the service in the validation criteria in URSP and manage the PDU Sessions for the related S-NSSAI accordingly based on the current design as described in TS 23.503 [12], clause 6.6.2.3.

It is agreed to consider in the normative phase that handover optimization as supplementary to prevent the UE from leaving the service area (or entering into the constrained service area) when other ongoing services are unaffected.

# * * * * End of changes * * * *