SA WG2 Meeting #S2-153E	S2-2208445

10 – 17 October. 2022, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	MediaTek Inc.

Title:	KI#5: Evaluation and interim conclusions on UE for accessing the localized services in a registered Hosting network

Document for:	Approval

Agenda Item: 	9.4.1

Work Item / Release:	FS_eNPN_Ph2/Rel-18

Abstract of the contribution: This paper proposes the evaluation and interim conclusions on KI#5

1	Proposal

It is proposed to adopt the following proposal into TR 23.700-08.

* * * * 1st Change * * * *

7	Evaluation

7.5	Key Issue #5: Enabling access to localized services via a specific hosting network

Table 7.5-1: Summary of solutions for KI#5

| Solution #7 | The solution provides the high level entities and procedures for providing access to localized services, accessing localized/home services, and charging information for the home network operator. |
| --- | --- |
| Solution #11 | Solution is reusing existing roaming architecture and LADN mechanism. The UE uses the local breakout session for accessing specific services from hosting network and home routed session for accessing services from the home network. Only PNI-NPN as Hosting Network is considered. |
| Solution #13 | The UE will be configured with the N3IWF address of the home network, wherein for accessing the home network services. The UE uses the hosting network as an underlay network. |
| Solution #15 | The solution focuses more on KI#4. |
| Solution #18 | Solution proposes to use the existing mechanism i.e. Steering of Roaming to allow the service provider (PLMN or 3rd Party service provider) to update the UE with required information on availability of hosting networks along with the localized services they are providing. Steering of Roaming information container would carry the "operator defined hosting network selector list", which will assist the UE to switch to hosting network for obtaining localized services. |
| Solution #27 | Solution is reusing N3IWF overlay architecture and LADN mechanism, i.e. 5G System architecture with access to SNPN using credentials from Credentials Holder and 5G System architecture accessing overlay network (home network) via underlay network (hosting network). The solution is built on top of solution #11. |
| Solution #28 | The UE (User) obtains the time restricted credentials from the LSP via the home network and uses that information for selection of hosting network as well as to access localized services. N3IWF node used to access home network services. |
| Solution #29 | Proposes hosting network to notify serving network when the hosting network is ready for access, which is related to the key issue #5 aspect for how to determine the service availability of hosting network. Details on how hosting network to notify its availability to serving network is needed for a complete evaluation. |
| Solution #35 | The home network provides URSP which maps local services to its S-NSSAI and DNN. The registration of the S-NSSAI related to local services causes the cell/network reselection. |
| Solution #36 | In one of the options presented, home network and UE coordinate, wherein home network would either use the UE location to get information about target suitable hosting network performance information via target hosting network (e.g. SNPN) NEF or home network would instruct the UE to measure and report the signal reception status of the potential target hosting network(s). Based on the information received from the UE, the home network would send SoR (Steering of Roaming) instructions to steer the UE toward the target hosting network (e.g. SNPN). |
| Solution #37 | The solution defines a new network function LSF (Localized Service Function) in the home network and hosting network that specifically handles information related to localized services. The new network function will provide information to AMF about all available localized services at the UE location and this information about localized services will be shared with the UE via NAS signalling. |
| Solution #45 | Similar to LADN mechanism. The SMF determines whether the UE meets the validity information of the localized services. If the validity information is set to the location, the SMF subscribes UE mobility event notification to AMF. If the validity information is set to time, the SMF monitors the duration that the UE can access the localized services |
| Solution #46 | Reusing the Route Selection Validation Criteria to the UE for access the available localized services in the registered Hosting network. Furthermore, it also describes the order if the UE has multiple URSP rules from HPLMN or from the subscribed SNPN. |

For the above, the condition is assumed that UE has registered to a Hosting network and the UE wants to establish the PDU Session for access the localized services available in the currently registered Hosting network.

In several solutions (e.g. #18, #36, etc.) , existing SoR mechanism with some enhancement is proposed to be used for the home network to steer the UE to the hosting network. This approach addresses the KI well and has limited impacts on the UE and network.

In the solutions, the typical triggers for steering the UE to a specific hosting network are:

-	The time and location of UE has met the validity condition of a local service (e.g. #18).

-	The performance of the hosting networks (e.g. #36).

For the aspect of enabling access to both home network services and local services. Several solutions propose to use existing overlay/underlay architecture to access the target network through N3IWF. This approach addresses the KI well and has limited impacts on the UE and network.

8	Conclusion

8.5	Key Issue #5: Enabling access to localized services via a specific hosting network

The following conclusions are agreed for normative work:

-	The existing SoR procedure is enhanced as follows:

-	The SoR contains a prioritized list of hosting networks. The list is determined by the home network and may be associated with validity conditions, including time and location conditions.

-	The existing overlay/underlay architecture is reused for the UE to access both home network services and localized services via N3IWF.

Editor's note:	It is FFS whether to support only a single or both models of the overlay/underlay architecture to access the localized services, i.e. (A) home network services are accessed via underlay network and the localized services via overlay network via N3IWF, and (B) the localized services are accessed via underlay network and the home network services via overlay network via N3IWF.

Editor's note:	The scenario of accessing to localized services via N3GPP network is for further consideration.

-	The existing URSP rules or LADN feature can be re-used for a UE to access the Localized services after the UE has registered to a Hosting network.

* * * * End of Changes * * * *