SA WG2 Meeting #153e	S2-2208505

# October 10th – October 17th, Elbonia               		     (Revision of S2-220xxxx)

Source: 	China Mobile

Title: 	Update to evaluations and conclusions on KI#1

Document for: 	Approval

Agenda Item: 	9.25.1

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: Update to evaluations and conclusions on KI#1.

1 Discussion

Solution #20 proposed in SA2#152e includes UPF NF profile information registered in NRF.

Solution #20 proposes that an UPF supporting NAT functionality may register Public IP address pool information per IP Domain in NRF. The Public IP address pool information may be on per DNN and S-NSSAI basis.

There is one Editor’s Notes on clause 8.1 that should be resolved:

Editor's note:	If the NF profile for UPF shall have an indirect/direct support indication as per solution 12 for the event exposure service is FFS.

For this EN, whether indirect or direct subscription is used for a consumer NF is concluded in KI#2. So there is no need to have an indirect/direct indication registered in the NRF as the NF profile for UPF. Whether to use direct subscription or indirect subscription is the inner logic of the consumer NF. The UPF only needs to show the supporting of UPF event service exposure service.

2 Proposal

It is proposed to update TR 23.700-62 v1.0.0 as follows.

First change

7.1	Overall Evaluation of solutions for Key Issue #1

For KI#1:

-	How to support UPF event exposure service(s) registration/deregistration on NRF, and what parameters to be registered in the NF profile of UPF.

-	How to support UPF service discovery via the NRF, and what parameters that can be used for discovery.

Solution #1 (clauses 6.1.2.1-6.1.2.4) can be seen as the basis to solve these aspects of KI#1. It is also considered and enhanced by Solution #12, Solution #20 and Solution #1 (clause 6.1.2.5.3) further.

Solution #12 proposes direct subscription and indirect subscription for NWDAF collecting data from UPF. The impact of Solution #12 on KI #1 is that it proposes to include direct indication and/or indirect indication in UPF NF profile when UPF registers to NRF.

Solution #20 proposes that an UPF supporting NAT functionality may register Public IP address pool information per IP Domain in NRF. The Public IP address pool information may be on per DNN and S-NSSAI basis.

For KI#1:

-	How to support UPF selection for a UPF event exposure service request targeting a specific UE or a specific PDU session.

Solution #1 (clauses 6.1.2.5.1 and 6.1.2.5.2) can also be considered to solve that aspect of KI#1:

-	It can be used for UPF selection with information of UE IP address, as described in clause 6.1.2.5.1.

-	It can be used for UPF selection with information of SUPI, S-NSSAI and DNN, as in clause 6.1.2.5.2.

-	It can be used also for UPF selection with information of UE IP address, by first applying existing functionality for IP address translation into SUPI using BSF service.

-	It can be used for UPF selection with information of Group Identifier by first applying existing functionality for Group Identifier translation into SUPI list using UDM service.

By specifying in normative phase the enhancements proposed in clause 6.1.2.1-6.1.2.4 from solution #1, clause 6.12.3.2 from solution #12, and clause 6.20.3.1 from solution #20, the following issues from KI#1 are covered:

-	How to support UPF event exposure service(s) registration/deregistration on NRF, and what parameters to be registered in the NF profile of UPF.

-	How to support UPF service discovery via the NRF, and what parameters that can be used for discovery.

The following issue from KI#1 is not covered:

-	How to support UPF selection for a UPF event exposure service request targeting a specific UE or a specific PDU session.

Second change

8.1	Conclusions for Key Issue #1

UPF(s) register their Event Exposure service(s), supported event ID(s) onto NRF. Procedures defined in the present TR, clauses 6.1.2.1 to 6.1.2.4 and clause 6.20.3.1 are endorsed as a baseline for normative specifications.

The conclusion for the not covered issue in KI#1 will be concluded as part of KI#2.

End of change