3GPP TSG-SA WG2 Meeting #153-e	S2-2208514

# Electronic meeting, 10 – 17 October 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0108 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cast Type for Discovery message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In the incoming LS from RAN2 (S2-2208106/R2-2206391), it has pointed out that: AS-layer specification requires Tx-UE to carry cast-type-indicator in SCI, based on which the delivered packet is to be filtered by MAC layer by Rx-UE. But RAN2 noticed that there is no cast type indicator provided to lower layer for discovery message transmission based on the current TS 23.304, so AS-layer cannot know the cast type (unicast/groupcast/broadcast) of the destination L2 ID, and thus the AS layer cannot set the cast type of the discovery message based on the destination L2 ID. To meet the AS layer’s requirement on the cast type, it is proposed that the ProSe layer will provide discovery type to the AS layer. Otherwise the AS layer will treat all the discovery message as the broadcast type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ProSe layer provides the discovery type to the lower layer when performing discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AS layer will treat all the discovery message as the broadcast type. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

5.2.4	5G ProSe Direct Discovery Characteristics

5G ProSe Direct Discovery over the PC5 reference point has the following characteristics:

-	PC5 communication channel is used to carry the discovery message over PC5. The discovery message over PC5 is differentiated with other PC5 messages by AS layer.

-	ProSe layer shall indicate to AS layer whether the signalling is discovery message or PC5-S signalling.

-	ProSe layer shall indicate to AS layer whether the discovery message is unicast, broadcast or groupcast discovery. If the UE performs the Response procedure for Model B discovery, then the ProSe layer indicates to the AS layer that the discovery message is a unicast discovery. If the UE performs the groupcast discovery, then the ProSe layer indicates to AS layer that the discovery message is a groupcast discovery. Otherwise the ProSe layer indicates to AS layer that the discovery message is a broadcast discovery.

NOTE 1:	The discovery message format is defined in stage 3.

Group discovery/management to support on demand-based group communication for commercial services has the following characteristics:

-	The group discovery/formation/management can be carried out in the Application layer in coordination with the Application Server.

-	Application layer signalling between the UE and the Application Server is out of scope of this specification.

5G ProSe Direct Discovery with 5G DDNMF has the following characteristics:

-	5G DDNMF in the 5GS is used for 5G ProSe Discovery Direct Code management (including allocation and resolution). The 5G DDNMF gets the PDUID from the PCF and subscribes to notifications on Change of PDUID.

-	5G DDNMF is defined in clause 4.3.2 and the detail procedure for 5G ProSe Direct Discovery with 5G DDNMF is defined in clause 6.3.

Group discovery/management to support public safety has the following characteristics:

-	Pre-configured or provisioned information can be used for the 5G ProSe Direct Discovery procedure as defined in clause 5.1.2.

The information elements included in the 5G ProSe Direct Discovery messages are described in clause 5.8.1 and clause 6.3.2.

NOTE 2:	Based on UE implementation, the application layer discovery messages are exchanged either as user traffic over PC5 or alternatively as part of metadata in PC5 Direct Discovery message as specified in clause 6.3.2.1 and clause 6.4.2. In the latter case, the PC5 Direct Discovery message can contain additional field carrying application layer metadata information, e.g., the Application layer discovery messages for group discovery. The format and contents of this additional field is out of scope of 3GPP. Performance of the PC5 Direct Discovery message including the application layer information will be affected if the resulted PC5 Direct Discovery message size is too big, e.g. longer delay and lower reliability.

* * * * End of Changes * * * *