3GPP TSG-SA WG2 Meeting # 153E	S2-2208528

# Electronic, October 10 - 17

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3572 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF event exposure service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The followings are agreed upon in Rel-18 in accordance with the conclusions for KI#2 in TR 23.700-62: - the only defined consumers of UPF event SUBSCRIBE are SMF, and NWDAF. - the only defined consumers of UPF event notifications are AF/NEF, TSNAF/TSCTSF and NWDAF. Based on conclusions of TR 23.700-62, 3GPP TS 23.502 should be updated to support UPEAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UPF service operarations are updated and defined to enable the UPF event exposure service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UPF event exposure service is not supported by 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.2.26	UPF Services

5.2.26.1	General

The following table shows the UPF Services and UPF Service Operations.

Table 5.2.8.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_EventExposure | Subscribe | Subscribe/Notify | SMF, NWDAF |
|  | Unsubscribe | Subscribe/Notify | SMF, NWDAF |
|  | Notify | Subscribe/Notify | NEF, AF, TSNAF, TSCTSF, NWDAF |

Editor's note:	Whether AF/NEF can directly subscribe to UPF is FFS.

5.2.26.2	Nupf_EventExposure Service

5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There are operations for this service:

-	Nupf_EventExposure_Subscribe.

-	Nupf_EventExposure_UnSubscribe.

-	Nupf_EventExposure_Notify.

The following events can be notified to a NF consumer:

-	QoS Monitoring for URLLC: the event notification may contain the QoS Monitoring report as described in clause 5.33.3.2 of TS 23.501 [2].

The event notification may contain following information:

-	QoS monitoring result e.g. end to end delay for specific QoS flow or for specific PDU session.

-	TSC management information (UMIC, PMIC, NW-TT port number) as defined in clause 5.8.2.11.14 of TS 23.501 [2].

5.2.26.2.2	Nupf_EventExposure_Notify service operation

Service operation name: Nupf_EventExposure_Notify

Description: This service operation reports the event and information to the consumer that has subscribed implicitly.

Input Required: Event ID, UE address (i.e. IP address or MAC address) or UE ID, Notification Correlation Information.

Editor's note:	Whether to use UE ID is FFS.

Input, Optional: Event specific parameter as described in clause 5.2.26.2.1.

Output Required: Result Indication.

Output, Optional: None.

5.2.26.2.3	Nupf_EventExposure_Subscribe service operation

Service operation name: Nupf_EventExposure_Subscribe.

Description: The consumer NF uses this service operation to subscribe to event reporting.

Input, Required: NF ID, Target of Event Reporting: UE(s) ID, ((set of) Event ID(s) defined in clause 5.2.26.2.1, Notification Target Address (+ Notification Correlation ID))s, Event Reporting Information defined in clause 5.2.26.2.1.

Input, Optional:

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 5.2.26.2.1).

The NF consumer subscribes to the event notification by invoking Nupf_EventExposure to the UPF. The UPF allocates a Subscription Correlation ID for the subscription and responds to the consumer NF with the Subscription Correlation ID.

The ((set of) Event ID(s), Notification Target Address (+ Notification Correlation ID)) helps the Event Receiving NF to co-relate a notification against a corresponding event subscription for the indicated Event ID.

In the case that the SMF subscribes to the UPF on behalf of other NF, the NF consumer include the Notification Target Address(+Notification Correlation ID) of other NF for the Event ID which is to be notified to other NF directly and the Notification Target Address(+Notification Correlation ID) of itself for the Subscription Correlation ID change event. Each Notification Target Address(+ Notification Correlation ID) is associated with the related (set of) Event ID(s).

Event receiving NF ID identifies the NF that shall receive the event reporting.

When the consumer NF needs to modify an existing subscription previously created by itself in the UPF, it invokes Nupf_EventExposure_Subscribe service operation which contains the Subscription Correlation ID with Event ID to the UPF.

5.2.26.2.4	Nupf_EventExposure_UnSubscribe service operation

Service operation name: Nupf_EventExposure_UnSubscribe.

Description: The NF consumer uses this service operation to unsubscribe for a specific event.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

The NF consumer or the SMF on behalf of other NF unsubscribes the event notification by invoking Nupf_EventExposure_Unsubscribe (Subscription Correlation ID) to the UPF.

* * * End Of Changes * * *