SA WG2 Meeting #153E (E-meeting)	S2-2208530

October 10 – 17, 2022, Elbonia	(revision of S2-220xxxx)

Source:	Deutsche Telekom

Title:	Discussion related to IMS Data Channel and 3GPP PS Data Off

Document for:	Discussion

Agenda Item:	9.17.1

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract of the contribution: Discuss the impact for IMS Data Channel when the user deactivates mobile data.

# Background

When the user deactivates "mobile data" or "mobile data when roaming" it prevents transport via PDN connections in 3GPP access of all IP packets, "IMS Data Channel" would not be available for the user, to be consistent between what the user expects and what the user may be billed for.

This has impact on the new feature "IMS Data Channel". To enable the use of some features also for the case mobile data are deactivated, 3GPP PS Data Off service was defined in TS 22.011.

Among other services, MMTel Voice and MMTel Video were defined as 3GPP PS Data Off Exempt Services. This way the operator can configure if such services should be allowed or blocked when the user deactivates "mobile data" or "mobile data when roaming".

# Discussion

The study in TR 23.700-88 does not describe the impact on IMS Data Channel when the user deactivates "mobile data" or "mobile data when roaming". The latest draft of GSMA PRD NG.134 describes that IMS Data Channel is a 3GPP PS Data Off Exempt Service, but this is not in line with current 3GPP specifications.

Possible scenarios are therefore:

- IMS Data Channel is not a 3GPP PS Data Off Exempt Service

- IMS Data Channel is a 3GPP PS Data Off Exempt Service

- User activates / deactivates "mobile data" or "mobile data when roaming" during an ongoing call

Here, coordination with GSMA UPG is necessary.

# Proposal

It is proposed to add a new chapter or an Editors's note in clause "conclusions" for Key Issues 1. There it can be described that interworking with feature "3GPP PS Data Off service" will be coordinated with GSMA and the solution will be developped in normative phase of NG_RTC.