SA WG2 Meeting #153e	S2-2208540

10 – 17 October 2022, e-Meeting	(revision of S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	KI#1, KI#2: Conclusions

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract of the contribution: Proposes conclusions for KI#1 and KI#2.

# 1. Introduction

Conclusions for the solutions in TR 23.700-28 are proposed here.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-28.

*** First Change (all new text) ***

# 8	Conclusions

## 8.x	Type of Satellite Coverage Data and Transfer to Consumers

The following aspects are concluded for normative support.

-	Satellite coverage data is supported in the form of a coverage map for one or more fixed locations and times as described for Solution #15.

-	Satellite coverage data may be optionally supported for a known or predicted UE trajectory or mobility pattern as described for Solutions #17, #19 and #21.

-	Satellite coverage data may be transferred to a UE from an external server or an AF (internal or external to PLMN) as described for Solutions #15 and #22.

-	Whether UP or NAS via an AMF/MME (CP) will be used for the transfer is FFS and will depend on feedback from CT1.

-	If satellite coverage data is needed by an AMF, MME, the transfer may be supported by O&M.

## 8.y	Determination of Satellite Coverage Gaps

The following aspects are concluded for normative support.

-	Determination of satellite coverage gaps may be supported by UEs based on satellite coverage data received according to clause 8.x. For E-UTRAN UEs, the additional data defined in clause 8.x for determination of satellite coverage gaps will be supported together with what is already defined in Rel.17 TS 36.331 [16]. Whether NG-RAN will support ephemeris data and coverage parameters to the UE in RRC for NR will be decided by RAN WGs.

-	UEs will indicate an impending satellite coverage gap, when known, to an AMF or MME as described for Solution #6 and Solution #16 and will notify the AMF or MME when the satellite coverage gap has ended as described for Solution #16.

-	An MME, AMF may verify an impending satellite coverage gap provided by a UE using satellite coverage data (e.g. provided by O&M) as defined in clause 8.x.

-	An AMF or MME uses an indication of an impending satellite coverage gap provided by a UE or determined by the AMF or MME to avoid paging the UE during the coverage gap and initiate procedures to buffer DL data.

-	An MME or AMF may adjust power saving parameters for a UE (for eDRX, MICO or PSM) to reduce UE power consumption during a satellite coverage gap as described for Solution #1, Solution #2, Solution #5.

*** End of Changes ***