3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208543

Elbonia, October 10 – 17, 2022

Source:	Tencent, Tencent Cloud

Title:	KI#6: Conclusion Update

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM/ Rel-18

Abstract: This paper proposes conclusion update for KI #6: Uplink-downlink transmission coordination to meet Round-Trip latency requirements regarding to two ENs.

1. Discussion

This paper proposes to update conclusion for supporting uplink-downlink transmission coordination to meet Round-Trip latency requirements focusing on EN resolving.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

8.1	Conclusions for Key issue#6: Uplink-downlink transmission coordination to meet Round-Trip latency requirements

The following aspects are concluded as principles for the normative work:

-	The RT latency can be preconfigured in PCF. Alternatively, the RT latency requirement can be provided by the AF. The RT latency requirement from the AF can be a requested 5GS delay (as defined in clause 6.1.3.22 of TS 23.503 [4]) together with a requested RT latency indication. The requested RT latency indication indicates the service data flow needs to meet that the RT delay overhead doesn't exceed the doubling of the 5GS delay. The AF provides the RT latency requirement to the PCF during AF session with required QoS procedure.

-	Based on RT latency requirement, the 5GS shall split the RT latency into an UL PDB and a DL PDB. The UL PDB and DL PDB can be unequal but their sum shall not exceed the RT latency.

-	If the PCF split the RT latency into an UL PDB and a DL PDB, the PCF shall generate two PCC rules for the media stream, one for the UL and one for the DL direction, and assigns the 5QIs according to the PDBs respectively.

-  If the PCF splits RT latency, UL/DL PDB can be tracked independently.

-	Different 5QIs shall apply to different QoS Flows as defined in current QoS model.

-  If RAN split UL and DL PDBs and perform radio resource scheduling to meet RT latency requirement, RT latency can be tracked by 5GS.

Editor's note: Conclusions on procedures regarding to RAN splitting of RT latency requires coordination with RAN WGs.

# * * * * End of changes * * * *