3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208564

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Update on Conclusion for LBO case

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: Solution on remaining ENs on KI#1-LBO conclusion.

# 1. Introduction/Discussion

1.

Editor’s Note: It is FFS whether dynamic reconnect from a HR PDU session to an LBO PDU Session, described in Solution #27, should be defined in the normative phase.

In last e-meeting, it was concluded that URSP based solution is preferred to support LBO PDU Session establishment. In addition, it is agreed that normative work for this scenario will take place within the scope of a WID that is based on the conclusions of FS_eUEPO Key Issue #1, if it fulfils that PLMN ID(s) shall be sent with URSP rules. According to conclusion already drawn in FS_eUEPO, VPLMN ID is provided to UE along the VPLMN specific URSP, which can fulfil this Key Issue. It means no extra impact will be introduced compared to FS_eUEPO operating URSP based solution, therefore Solution #27 is not necessary.

2.

Editor’s Note: The agreement that the normative work for this KI#1/LBO will take place within the WID for FS_eUEPO depends on whether the conclusion for KI#1 FS_eUEPO fulfils the conclusion listed in this clause. If this is not the case, the normative work to fulfil this conclusion will have to take place within the WID for EDGE_Ph2.

This editor’s note describes the conditional dependency on FS_eUEPO conclusion, and should be turned to a NOTE.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48.

# * * * * First change * * * *

## 8.X Interim Conclusion for Key Issue #1: LBO case

### 8.1.1	Interim conclusion for scenario 1 (via LBO PDU Session)

To support the establishment of an LBO PDU Session towards the correct DNN/S-NSSAI to access an EHE in the VPLMN, URSP based solution is preferred. Key Issue 1 of TS 23.700-85 [10] is studying URSP rule provisioning and updating procedures in roaming scenarios. Therefore, the conclusions of Key Issue 1 of TS 23.700-85 [10] will take in account the conclusions of this clause.

In order to support scenarios where the EHE is accessed via an LBO PDU Session, it is concluded that URSP related procedures should be enhanced as follows:

-	PLMN ID(s) should be sent with URSP rules. How the PLMN ID(s) are included is defined in the conclusion of Key Issue #1 of TR 23.700-85 [10].

-	Whether the UE can be sent URSP rules that is applicable in the VPLMN before the UE is served by the VPLMN will be defined in the conclusion of Key Issue #1 of TR 23.700-85 [10].

NOTE 1:	Normative work for the bullets above will take place within the scope of a WID that is based on the conclusions of FS_eUEPO Key Issue #1. It will be decided in FS_eUEPO Key Issue #1 whether a new trigger for URSP re-evaluation is needed.

NOTE 2:	The agreement that the normative work for this KI#1/LBO will take place within the WID for FS_eUEPO depends on whether the conclusion for KI#1 FS_eUEPO fulfils the conclusion listed in this clause. If this is not the case, the normative work to fulfil this conclusion will have to take place within the WID for EDGE_Ph2.

# * * * * End of changes * * * *