3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208569

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion update for KI#6

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Conclusion update for KI#6 on how to dynamically track of the UL&DL latency and adjust the UL&DL QoS polices accordingly.

# 1. Introduction

In SA2#152e meeting, KI#6 is partially concluded with the EN:

Editor's note: if the RT latency will be tracked by 5GS or only the UL/DL PDB will be tracked independently (e.g. using the current PER for a given 5QI) is FFS.

This paper proposes to add details about the RT latency tracking and the dynamic adjustments of the UL/DL PDBs to resolve this EN.

Basically 3 methods were proposed in last meeting:

Option 1. Two QoS Flows for UL/DL traffic separately. PCF adjusts 5QI/PDB of each QoS Flow optionally based on QoS monitoring results.

Option 2. Two QoS Flows for UL/DL traffic separately. PCF adjusts 5QI/PDB of each QoS Flow based on AQP of both QoS Flows reported by RAN in case PDB of one QoS Flow cannot be met.

Option 3. Single QoS Flow for UL/DL traffics, but UL/DL can use different PDBs. A 5QI is associated with a round trip delay or two PDBs for UL/DL separately. RAN adjusts UL/DL PDB by itself.

We believe option3 may have the following issues:

1.	A single QoS Flow associated with multiple 5QIs or different PDBs needs to be mapped to different DRBs in the RAN side. This impacts 5GS QoS design.

2.	RAN dynamically adjusts the QoS profile on its own will cause PCF losing the control of the resources used by RAN for a service.

Between option 1 and 2, NG-RAN in option 2 can provide more accurate available PDBs for both the uplink and downlink QoS Flows for PCF adjustment, while in option 1, PCF don’t really understand whether the new PDB can be realized by RAN or not, thus the adjustment may cause a failure.

Based on above, Option 2 is proposed as conclusion.

Besides, it’s proposed to remove the following EN as the main impact of the current conclusion is on the Core Network side and there is no need to confirm with RAN WG.

Editor's note: Conclusions on procedures impacting RAN are FFS and require coordination with as well as feedback from RAN WGs.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change * * * *

## 8.2	Conclusions for Key issue#6: Uplink-downlink transmission coordination to meet Round-Trip latency requirements

The following aspects are concluded as principles for the normative work:

-	The RT latency can be preconfigured in PCF. Alternatively, the RT latency requirement can be provided by the AF. The RT latency requirement from the AF can be a requested 5GS delay (as defined in clause 6.1.3.22 of TS 23.503) together with a requested RT latency indication. The requested RT latency indication indicates the service data flow needs to meet that the RT delay overhead doesn’t exceed the doubling of the 5GS delay. The AF provides the RT latency requirement to the PCF during AF session with required QoS procedure.

-	Based on RT latency requirement, the 5GS shall split the RT latency into an UL PDB and a DL PDB. The UL PDB and DL PDB can be unequal but their sum shall not exceed the RT latency.

-	If the PCF split the RT latency into an UL PDB and a DL PDB, the PCF shall generate two PCC rules for the media stream, one for the UL and one for the DL direction, and assigns the 5QIs according to the PDBs respectively. The alternative QoS Parameter Sets for UL/DL QoS Flows may be also derived based on the RT latency requirements and further added in the two PCC rules by PCF. Then SMF generates alternative QoS Profiles for both the UL and DL QoS Flows and sends them to RAN together with the RT latency requirement and the QoS correlation indication.

-	Different 5QIs shall apply to different QoS Flows as defined in current QoS model.

-	The NG-RAN keeps track of RT latency and locally checks current fulfils to match appropriate QoS Profiles for the UL and DL QoS Flows to meet the RT latency requirements. If the NG-RAN determines that the PDB of one of the QoS Flows cannot be fulfilled, NG-RAN shall perform Notification control and send PDBs of both UL/DL QoS Flows that RAN currently fulfils to the SMF then to the PCF.

# * * * * Second change * * * *

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *