3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208579

Elbonia, October 10 – 14, 2022

Source:	China Mobile

Title:	KI#5: conclusion for support of edge relocation based on GSMA OPG impacts and improvements

Document for:	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2

Abstract: update conclusion for KI #5.

1. Discussion

2. Text Proposal

It is proposed to capture the following aspects in TR 23.700-48.

# * * * * First change * * * *

8.5	Interim conclusion for KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

To enable the discovery of an EAS deployed in another PLMN than the one serving the UE (e.g. an EAS deployed in a shared EHE).

# EAS Deployment Provision

-	The serving PLMN is provided with the information needed to support UE's to discover and connect to the shared EHE.

Editor's note:	It is FFS based on what assumption and what information related to the edge application in a shared EHE (the supported FQDNs and/or the AF related information for solution #23, and/or the EASDF information for solutions #22 and #40, and/or the DNS servers and/or the DNAI list for solution #38, and/or the EAS IP ranges for each PLMN for solution #39) is provided to the serving PLMN in line with the GSMA OPG requirement of topology hiding.

Editor's note:	It is FFS which NF performs the translation of specific information (e.g. DNAI) from different PLMNs if required.

# EAS Discovery Procedure

-	EAS discovery procedure defined in TS 23.548 [3] will be reused.

Editor's note:	It is FFS whether all the methods defined in TS 23.548 [3] will be reused in this GSMA OPG scenario and/or whether extensions to the methods are needed.

# Traffic transmission between two PLMNs

-	When the traffic transmitted between two PLMNs, the UPF in serving PLMN should support mechanisms to facilitate low latency transmission.

# Edge Relocation

For edge relocation in GSMA OPG scenario, the EAS reselection methods should be the same as the initial selection.

Edge relocation within the same hosting PLMN’s EHE is supported. EAS relocation between different PLMN’s EHE is depend on GSMA’s feedback.

# * * * * End of changes * * * *