SA WG2 Meeting #153E eMeeting	S2-2208617

Elbonia, Oct. 10 – 14, 2022 	(revision of S2-20xxxxx)

Source:	OPPO, Oracle

Title:	TR 23.700-80: Comments for Evaluation Descriptions in Clause 7 on KI#6

Document for:	Discussions

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution is to comment on the evaluations captured in Clause 7 on KI#6.

# Comments

The intent of this proposal is to present OPPO’s comments to update the existing APs and evaluations for KI#6 as captured in Clause 7.

# Proposal

***** Start of Changes *****

## 7.6	Key Issue #6: QoS and Policy enhancements

This Evaluation is related to the following requirements described in KI#6: "In order to conclude whether the 5G system meets QoS performance requirements as specified in clause 7.10 of TS 22.261 [2], this study needs to determine whether any additional QoS and/or policy enhancements to the 5G system are required".

The criteria for evaluation of solutions is as follows:

-	Whether a solution addresses the KI description, what aspects if not all.

-	Impacts in the system, NF and NF services impacted, procedure impacted.

-	Completeness of the solution, whether open issues are identified or Editor´s Note are listed.

### 7.6.1	QoS performance measurement assistance to Application AI/ML operation

Table 7.6.1-1 lists solutions that addresses how to map performance KPIs into 5GS QoS parameters and the procedure to perform QoS Monitoring for the UE to AI/ML application traffic to perform both the AI/ML split, AI/ML download and federated learning as defined in KI#6.

# Table 7.6.1-1: Mapping performance KPIs into QoS parameters. Procedure for Monitoring QoS parameters

| Solution | Covers KI requirements | Impacts on NFs | Completeness | Open issues/Editor´s Note (NOTE 1) |
| --- | --- | --- | --- | --- |
| #1 | Covers Delay and Analytics and procedure | AF: - Request monitoring of parameters described in clause 6.1.1 related to AI/ML-related traffic via Nnef_AFsessionWithQoS for QoS monitoring and Nnef_EventExposure_Subscribe for traffic usage and session inactivity time. NEF: - Support the request for monitoring of parameters described in clause 6.1.1 related to AI/ML-related traffic. - New monitoring events required for traffic usage and session inactivity time. SMF/UPF: - Provide the monitored parameters described in clause 6.1.1 related to AI/ML-related traffic. | This solution proposes 5GS monitoring capabilities to support AI/ML-based services as well as AI/ML operations (i.e. model splitting, model sharing, and distributed/federated learning, as described in TS 22.261 [2]) over the 5GS. The solution i) leverages aspects of the QoS monitoring capabilities for URLLC services relevant to AI/ML-based services, and ii) proposes additional monitoring features, i.e. traffic volume and UE’s inactivity time | Whether the usage report of inactivity time are beneficial needs further considerations. NOTE: If the application has monitoring capabilities to accurately determine traffic volume and session inactivity, then it may not require 5GS monitoring capabilities for traffic/data volume and session inactivity time. |
| #7 | Covers Delay and Bitrate and procedure | Impacts Nnef_AFSessiowithQoS (QoS reference is mandatory) and Npcf_SMPolicyControl Impacts RAN, SMF, UPF, PCF, NEF and AF | Not complete yet. Unclear how the AF request QoS Monitoring, given that the QoS reference is mandatory parameter in Nnef_AFSessionwithQoS and the QoS parameters in the PCC rule are mandatory. | How to request QoS monitoring only without requesting QoS is performed in not defined. NOTE: an alternative way for AF to subscribe delay and bitrate monitoring is to reuse 4.16.5.2 in TS 23.502, where AF/NEF provides service information to the PCF without other mandatory parameters except UE address and identification of the application session context |
| #15 (NOTE 2) | Covers Delay, Bitrate and Reliability. Covers the procedure. | Impacts UE, UPF, SMF, AF. It extends existing PMF with new functionality to monitor and do analytics. Defines new services and service operations for PMF monitoring. | Not complete yet. New services are not described. | Editor's note: Whether and how to support new UE measurement should coordinate with RAN group. Editor's note: How to support the co-existence of ATSSS and AIMLsys with PMF needs further clarification. Open issue: Lack of details to describe how the new NF obtain the QoS monitoring requirements form AF for the target QoS Flow(s) prior to trigger the PMF for the QoS monitoring. |
| #26 | Covers latency, packet loss rate, bandwidth. | Reusing existing Network Performance Analytics, hence, no new impact to any 5GC NF. | No new service is introduced. | It does not support all the required QoS parameters (i.e. Guaranteed Bit Rate) for Application AI/ML traffic as specified by TS 22.261. |
| #40 (NOTE 2) | Covers the Procedure for QoS Monitoring | Defines a new NF. AF: - Provides FL configuration parameters to the 5GC (NEF if AF is untrusted, or AFLSF is AF is trusted) via FL assistance services. NEF: - Provides request/subscription services for FL assistance operation and consumes AFLSF services. AFLSF: - Implicit awareness of application FL operation - Provides request/subscription services for FL assistance operation - Subscribes to NWDAF analytics, AMF/SMF event exposure - Handling of AMSID validation request SMF: - Sends validation request to AFLSF for the AMSID to be used by the UE. BSF: - Discovery of PCF address is enhanced to support discovery of PCF address(es) based on the group id for UE - Discovery of PCF address is enhanced to support discovery of PCF address(es) based on a list of UE IDs as an inputUE: - Requests to join an AI/ML session for FL. | - Introducing new QoS parameter, i.e. “group QoS” to be provisioned to the UEs for specific FL operation. | QoS Monitoring is activated using PCC Rules that are provided by the PCF to SMF. Whether a new NF can also activate QoS Monitoring needs to be discussed. More information of the new “group QoS” parameter similar to those were described in clause 5.7.2 of TS 23.501 is needed. More information on how NWDAF or other existing QoS monitoring mechanisms support the measurement and the evaluation for such new “group QoS” parameter are needed. No justification on why RAN is not impacted by such new “group QoS” parameters was not explained. |
| #42 (NOTE 2) | Covers latency, packet loss rate, bandwidth. | Reuses existing QoS Monitoring procedure as part of AF request to determine if the given UE within the group satisfies the new AIML group performance information | The AIML group performance information is defined, including includes Min/Max latency, Min/Max packet loss rate in UL/DL, Duration for the requested QoS, Minimum number of UEs, Max Requested bandwidth DL/UL included in the AIML group information. | Editor's note: It is FFS whether additional parameters are sent from AF as AIML group performance information. How to monitor other parameter than latency is not described. Open Issues: The proposed solution is claimed to be triggered for each iteration of the FL operation. However, no mention of how to unsubscribe/unprovision the set of QoS flows of a group of UEs from previous UE selection cycle. |
| NOTE 1: The evaluation needs updates when the Editor´s Note or open issues are resolved. NOTE 2: These solutions need to be considered to address KI#7 on performance monitoring for a UE or a group of UEs. |  |  |  |  |

Observation#A:

- Proposed solutions that monitor non-throughput related QoS parameters (e.g. delay, packet loss etc.) on per UE basis :

- Solution#1: Leveraging existing URLLC

- Solution#15: Leveraging PMP/PMF with new 5GC NF and NEF support

NOTE 1.a.1: PMP in Solution#15 supports also throughput monitoring

NOTE 1.a.2: Unresolved ENs – solution cannot be fully evaluated

- Proposed solutions that monitor group QoS performance and defining “new” Group QoS parameter:

- Solution#40: Leveraging new 5GC NF, AFLSF, with the support of NWDAF

NOTE 2.a.1: No information on what exactly is “Group QoS” and therefore it is not measurable or able to be monitored.  The only explanation for Group QoS is “ The group QoS requirement may consist of required QoS parameters per UE or per multiple UEs. “.

- Solution#42: Extending NEF with the support of PCF

NOTE 2.b.1: Solution#42 is incomplete

NOTE:2.b.2: Unresolved EN – solution cannot be fully evaluated

- Proposed solutions that monitor bit rate performance on per UE basis:

- Solution#1: Leveraging the solution from on-going R18 study – FS_UPEAS for the traffic volume monitoring via UPF support

- Solution#7: Extending PCF and SMF as well as UPF to interface with RAN to obtain the UE’s bit rate performance

NOTE3.b.1: Unresolved EN – solution cannot be fully evaluated

- Solution#15: Leveraging PMP/PMF with new 5GC NF and NEF support

NOTE 3.c.1: PMP in Solution#15 supports also non-throughput QoS parameters (e.g. packet delay, packet loss etc.) monitoring

NOTE 3.c.2: Unresolved ENs – solution cannot be fully evaluated

After examining the evaluation table and the observation above where several solutions have outstanding issues or lack of descriptions/definitions on the objective of the specific QoS performance monitoring (e.g. Group QoS), it is difficult to further evaluate on those solutions.  Therefore, it is proposed the following interim conclusions until further clarifications are provided on those solutions:

- For the QoS monitoring on per UE basis, existing URLLC mechanism as described in Solution#1 should be reused

- For the throughput/bit rate monitoring, the conclusion from KI#1 should be applied to KI#6.

### 7.6.2	QoS Time Window Transmission Monitoring and Negotiation

Editor's note:	Evaluation for solution #38 is FFS.

In this clause, two solutions are focused in this evaluation which target on the “Time Window” QoS monitoring and negotiation, and these two solutions are Solution#10 and Solution#38.

For the evaluation of the Solution#10 which is referred as Background Data Transfer Extensions to support non-real-time Application AI/ML traffic transport, one can refer to clause 7.5 for the details.

For the evaluation of the Solution#38, i.e. time dependent QoS, the objective of this solution expects that the network resources can be reserved for prescribed periods in the 5GS in order to meet the needs of AI/ML model transfer based on the assumptions that the operation requires brief and periodic use of significant 5GS resources. Reserving resources at specific times allows more efficient multiplexing of groups of UEs that need to transfer AI/ML models.

An AF sends QoS requirements (e.g. bandwidth required, PER, 5GS Delay, GBR) with QoS timing information (e.g. Model transfer periodicity etc.) to the NEF. Time dependency information indicates time for which these requirements shall apply (e.g. required QoS1 applies for when an ML model data transfer is taking place and, as an option, required QoS2 applies for the rest of the time).

Alternatively, the AF may send traffic pattern information (e.g. model size, model transfer periodicity, start time(s), duration(s), stop time(s)), so that traffic pattern information can be translated to time-dependent QoS requirements in the 5GC.

The AF may provide the information for a single UE or for a list or group of UEs. If the AF provides a list of UEs, it may indicate different periods during which these requirements shall be valid or time of model transfer for different UEs within the list.

In order to support this “time dependent QoS”, it requires the RAN support to guarantee the resource reservation for the target time window. Furthermore, such kind of resource reservation cannot be too far in advance.  Otherwise, the longer time window down the road, the lesser reliable it could be.  Another important considerations for this solution is the direct association with the list of 5GC assistance operations that assist the AF to select the candidate UE(s) to support the Application AI/ML operation.   When comparing to Solution#10, the 5GC assistance for the UE member selection is independent of the data transfer time window negotiation and reservation.

Furthermore, this mechanism will not be applicable for non-3GPP access as there is no mechanism to provide such kind of RAN support for QoS guarantee over the non-3GPP access.

Finally, it is unclear if today RAN is capable of supporting such kind of time dependent capabilities.  If not, this would have significant dependency on RAN, especially to support high volume of UEs are involved in this operation.

Table 7.6.2-1: Comparisons between Solution#10 and Solution#38

|  |  | Solution#10 | Solution#38 |
| --- | --- | --- | --- |
| Support Multiple Time Windows negotiation |  | Yes | Yes |
| Required 5GC awareness of the Application AI/ML Operation Type, e.g FL |  | No | Yes |
| Required dedicated new 5GC NF support |  | No | Yes |
| Support various QoS profiles/policies |  | Yes | Yes |
| Solution design is tightly couple with UE member selection operation |  | No | Yes |
| Support both 3GPP and non-3GPP access | Yes | No |
| Enhancement from existing feature | Yes | No |
| Impact to RAN | No | Yes |
| Impacts to PCF/PCC rules | No | Yes |
| Impacts to SMF | No | Yes |
| Impacts to UPF | No | Yes |
| Impacts to NWDAF | Yes | Yes |
| Allow flexibility to add or to remove UE(s) from the future transmission Time Window | Yes | No |

Solution#10 allows the group data transfer Time Window negotiation and transmission activation procedures to be independent for the 5GC assistance procedures for group UE member selection.  Furthermore, Solution#10 does not have heavy dependency on RAN, SMF and UPF, and certainly no impact to the existing PCC rules.  It is proposed to merge the Solution#38 with Solution#10 by excluding the aspect of 5GC assistance for the UE member selection as described in Solution#38 and to minimize the overall system impacts.

Next Changes *****

## 8.6	Key Issue #6: QoS and Policy enhancements

For the QoS performance monitoring, after examining the evaluation table and the observation above where several solutions have outstanding issues or lack of descriptions/definitions on the objective of the specific QoS performance monitoring (e.g. Group QoS), it is difficult to further evaluate on those solutions.  Therefore, it is proposed the following “interim” conclusions until further clarifications are provided on those solutions:

- For the QoS monitoring on per UE basis, existing URLLC mechanism as described in Solution#1 should be reused

- For the throughput/bit rate monitoring, the conclusion from KI#1 should be applied to KI#6.

As for the QoS time window transmission monitoring and negotiation, after examining the evaluations as described in clause 7.6.2, Solution#10 allows the group data transfer Time Window negotiation and transmission activation procedures to be independent for the 5GC assistance procedures for group UE member selection.  Furthermore, Solution#10 does not have heavy dependency on RAN, SMF and UPF, and certainly no impact to the existing PCC rules.  It is proposed to merge the Solution#38 with Solution#10 by excluding the aspect of 5GC assistance for the UE member selection as described in Solution#38 and to minimize the overall system impacts.

***** End of Changes *****