3GPP TSG-SA WG2#153E e-meeting	S2-2208625

Elbonia, 10th -17th October 2022                              	(revision of S2-220xxxx)

Source:	vivo，Ericsson

Title:	KI#8, conclusion Update

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes to update conclusion for KI#8.

1. Introduction/Discussion

This pCR proposes to update the conclusion of KI#8 by adding the description to allow the FL server to reselect FL clients during FL procedure.

2. Text Proposal

It is proposed to update the following text proposal in TR 23.700-81.

# * * * * First change * * * *

8.8	Key Issue #8: Supporting Federated Learning in 5GC

In conclusion, KI #8 proposes the following principles:

Principle 1: NWDAF containing MTLF as FL server or FL clients register to NRF with their FL related information, including Analytics ID(s), Address information, FL capability Type (i.e. FL server or FL clients) etc.

Principle 2: NWDAF containing MTLF determines ML model requires FL based on Analytic ID, Service Area/DNAI or data not available directly from data producer NF (e.g. due to privacy reasons).

Principle 3: If NWDAF containing MTLF as FL sever determines ML model requires FL, the FL Server discovers and selects FL Client from NRF. The following criteria are used for discovering a FL Client:

-	Analytic ID of the ML model required.

-	FL client capability.

-	Service Area.

-	Data available by the FL Client.

-	Time Period of Interest.

Principle 4: If NWDAF containing MTLF without FL server capability determines ML model requires FL, the MTLF discovers and selects FL sever from NRF. The following criteria are used for discovering a FL server

-	Analytic ID of the ML model required.

-	Model filter information as defined in TS 23.288 [5].

-	FL sever capability.

-	If FL server is currently doing a FL for the Analytics ID.

-	Time Period of Interest.

Principle 5: NWDAF containing MTLF as FL server may determine the final list of NWDAF containing MTLF as FL clients via initial FL request to FL clients to determine the availability and compatibility of the FL clients. During the FL procedure, the NWDAF containing MTLF as FL server may trigger reselection of FL clients via NRF, based on local policy or status of FL clients, e.g. load, availability, capability, latency, etc.

Principle 6: NWDAF containing MTLF as FL Server exchanges FL training related information with NWDAF containing MTLF as FL Client. The FL Server provides to the FL client the initial ML model information to train the local ML Model. Regarding the details information that FL server provides to the FL client, the conclusion should be aligned with KI #5, when it comes to the model sharing and interoperability between FL server and FL clients.

Editor's note:	The ML model information provided from the FL Server to the FL client is for FFS.

Principle 7: The services to enable the FL based ML model training should be generic enough for all ML model training mechanisms which require service provider trains the ML model provided by the service consumer.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

# * * * * End of changes * * * *