3GPP TSG-WG SA2 Meeting #153E e-meeting  	S2-2208655

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

# Title:	[DRAFT] LS on conclusion of FS_REDCAP_Ph2

# Response to:	LS (R3-225119, R2-2208117) on FS_REDCAP_Ph2 option feasibility from RAN3 and RAN2

# Release:	Release 18

# Work Item:	FS_REDCAP_Ph2, REDCAP_Ph2

Source:	[Huawei to be] SA2

To:	RAN2, RAN3

Cc:

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven DOT wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 would like to thank RAN2 and RAN3 for their feedback about FS_REDCAP_Ph2 and provide an update on the conclusions of the study.

FS_REDCAP_Ph2 study has completed with the following conclusions:

-	It is agreed to support MT data and signalling handling within the CN when the UE is unreachable due to long extended DRX in RRC inactive.

-	The gNB sends an indication to the CN and provides unreachability information (e.g., eDRX values negotiated between UE and gNB for RRC_INACTIVE state) when the UE has entered RRC_INACTIVE. This allows the CN to apply the HLcom functionality (e.g. data buffering, notifications to other network and application functions etc) based on the unreachability information the gNB provided (as described in solution #6 of TR 23.700-68).

-	If the gNB has indicated the UE has entered RRC_INACTIVE to the CN, the gNB also notifies the CN about the RRC State transition back to RRC_CONNECTED (as described in solution #6 of TR 23.700-68).

When the gNB sends the indication to the AMF is up to the gNB implementation.

If the indication of UE transition to RRC_INACTIVE is not sent (or sent after UE has entered RRC_INACTIVE) by the gNB then:

-	Until CN receives it, the CN cannot apply HLcom functionality and other NFs will not be aware of the UE reachability, and certain HLcom related services provided to the AF via NEF would not be available.

-	Downlink data transmitted from the UPF to RAN might be discarded and not delivered to the UE.

2. Actions:

To RAN2 and RAN3 group.

ACTION: 	SA2 asks RAN2 and RAN3 to take the above conclusion into consideration and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #154		Nov 14 – 18, 2022				Toulouse, FR

TSG-SA2 Meeting Adhoc   	Jan 16 – 20, 2023				Elbonia