3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208669

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract: A conclusion is proposed.

# 1. Introduction/Discussion

This pCR provides a structure for the conclusions and conclusions on some aspects.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-28.

# * * * * First change * * * *

# 8	Conclusions

## 8.1	Conclusion on general mobility management and/or power saving

The following are conclusions on mobility management and/or power saving:

-	The AMF/MME obtains satellite related information (e.g. satellite ephemeris, satellite footprint) from OAM or a 3rd party server.

-	The AMF/MME obtains the coverage information for the UE, based on the UE location information, the satellite related information, and optionally UE mobility information from an AF/UE and/or NWDAF (only apply to 5GC).

-	For the case when UE mobility is known or predicable:

-	The UE initiates Mobility Registration Update procedure/TAU procedure using the existing mechanism defined in TS 23.502 [3] and TS 23.401 [5].

-	The AMF/MME configures the UEs mobility management and power saving parameters, e.g. MRU/TAU timer, active time, eDRX, based on the coverage information, to make sure the UE in power saving mode out of network coverage, to avoid the network de-registering or detaching the UE, or attempting to page the UE during this time. The UE may use this information to help determine when it can access a network.

-	For the case when UE mobility is not known or predictable:

-	The UE notifies the AMF/MME when it is about to leave coverage.

-	The AMF/MME sets an implicit detach timer based on the coverage information to avoid de-registering or detaching the UE, or attempting to page the UE when it is in discontinuous coverage.

## 8.2	Conclusion on overload impacts to a target RAT/PLMN

The following are conclusions for this overload impacts to a target RAT/PLMN:

Editor’s Note: The conclusion is FFS.

## 8.3	Conclusion on alternative RAT/PLMN selection

The following are conclusions for alternative RAT/PLMN selection:

Editor’s Note: The conclusion is FFS and may not be required according to the discussion in Rel-17.

# * * * * End of changes * * * *