3GPP TSG-WG SA2 Meeting #153E e-meeting  	S2-2208719

# Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0138 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on MBS user service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LS from SA4 (S2-2205450) for MBS User Service was discussed in SA2 and one comment from the company was that currently there was no clear use case for providing MB2 in 5GS, and the company requested to postpone the discussion to SA#153. The company further analyzed the use cases and scenarios for using, and the related discussion paper can be found in S2-2208718. As a result, it is proposed to update TS 23.247 to reflect the analyses in the discussion paper, and avoid confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update the note in clause 5.2 to clarify the case for provding MB2 in 5GS. 2. Clarified in Annex C the use cases for invoking MB2 interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear Stage 2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2	General architecture for interworking with EPS

Interworking between MBS and eMBMS at service layer functionality applies in cases where the same Multicast/Broadcast service is provided via eMBMS and MBS. Figure 5.2-1 depicts the system architecture for interworking between E-UTRAN/EPC eMBMS and MBS at service layer, with collocated BM-SC and MBSF/MBSTF functionalities.

Figure 5.2-1: MBS-eMBMS interworking system architecture at service layer

The BM-SC+MBSF/MBSTF exposes common Nmb5/Nmb10/xMB-C/MB2-C and Nmb8/xMB-U/MB2-U reference points to the NEF and/or AF/AS. A common TMGI is used towards the AF/AS. The TMGI is also used as identifier for transport over E-UTRAN/EPC.

NOTE:	MB2-C/U are both legacy reference points and 5GS reference points. In case of 5GS providing MB2-C/U, it is used for the GCS/AS only supporting GC1 and MB2 interfaces, as defined TS 23.468 [10].

# * * * * Second change * * * *

Annex C (normative):
Interworking at reference points MB2 and xMB

To allow the MBS System to interwork with a GCS AS supporting the MB2 interfaces defined in TS 23.468 [10]:

-	In addition to supporting the Nmbsf service-based API at Nmb10 (as defined in clause 5.1) the MBSF shall support interfaces MB2 C.

-	In addition to supporting content ingest interfaces defined in TS 26.502 [18] at Nmb8 (as defined in clause 5.1) the MBSTF shall support interfaces MB2 U.

To allow the MBS System to interwork with a Content Provider supporting the xMB interfaces defined in TS 26.348 [11]:

-	In addition to supporting the Nmbsf service-based API at Nmb10 (as defined in clause 5.1) the MBSF shall support interface xMB-C.

-	In addition to supporting content ingest interfaces defined in TS 26.502 [18] at Nmb8 (as defined in clause 5.1) the MBSTF shall support interface xMB-U.

Figure C-1: Interworking with GCS AS supporting MB2 interfaces and with Content Provider supporting xMB interfaces

MBSF/MBSTF supports MB2-C and MB2-U to enable GCS/AS to establish the broadcast MBS session in 5GS, and the GCS/AS then uses GC1 interface accordingly to interact with the associated UEs, as defined in TS 23.468 [10].

# * * * * End of changes * * * *