SA WG2 Meeting #S2-153e(e-meeting)	S2-2208737

10 – 17 October 2022, Electronic Meeting

Source:	CATT

Title:	KI#4, #5, Sol#13: Update to add one precondition for procedure 6.13.3.1

Document for:	Approval

Agenda Item:	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper aims to update to add one precondition for procedure 6.13.3.1

1. Discussion

For the Procedure “enabling hosting network to provide access to localized services” in clause 6.13.3.1, as suggested by the comments from last SA2#152 e-meeting, there should be one precondition as follows:

And there should be another precondition for this procedure that UE with target localized service has registered to the hosting AMF and setup a PDU session with the hosting SMF, and the related AM/SM policy association has been established between hosting AMF/SMF with hosting PCF.

2. Proposal

Add the following update to solution #13 in TR 23.700-08.

*** BEGIN CHANGES ***

6.13.3	Procedures

6.13.3.1	Procedure for enabling hosting network to provide access to localized services

Figure 6.13.3.1-1: Procedure for the interaction between hosting network and localized service provider to enable UE to access the localized services via hosting network

The precondition for this procedure is the existence of a SLA between the Hosting network and the provider of localized service to enable the interaction between the AF and the Hosting Network as required. And there should be another precondition for this procedure that UE with target localized service has registered to the hosting AMF and setup a PDU session with the hosting SMF, and the related AM/SM policy association have been established between the hosting AMF/SMF with the hosting PCF.

-	Step 1: AF(localized service provider) may initiate or update the request to the NEF of the hosting network for providing access to localized services with the characteristics of the localized service, which including:

-	list of the identification of the localized services;

-	validity restriction for each localized service, e.g. the validity of time or location;

-	QoS requirements for each localized service.

-	Step 2: NEF/PCF of the hosting network determines its network rules for the localized services based on the request in Step1 and the agreement with the localized service provider. The network rules are used to reserve the network resources and meet the QoS requirement with the specific validity restriction for providing access to the localized services.

-	Step 3: NEF/PCF of the hosting network responses the result for providing access to localized services to the AF (localized services provider), the result corresponds to the request in the Step1, which including:

-	list of the identification of the localized services, which can be provided by the hosting network;

-	validity restriction for each localized service provided by the hosting network, e.g. the validity of time or location;

-	QoS requirements for each localized service provided by the hosting network.

-	Step 4: PCF of hosting network updates the parameters of the AMF and SMF in order to enforce its network rules which are determined in step 3 to provide access to localized services.

-	PCF will invoke Npcf_AMPolicyControl_UpdateNotify request to AMF as defined in clause 4.16.2.2 of TS 23.502 [4], which includes the list of the identification of the localized services, the modified service area restrictions (e.g. the validity of location), and the modified QoS (such as UE-AMBR, UE-Slice-MBR) parameters to notify AMF the access and mobility related policy for localized services has been updated.

-	PCF will invoke Npcf_SMPolicyControl_UpdateNotify request to SMF as defined in clause 4.16.5.2 of TS 23.502 [4], which includes the modified QoS (e.g. Authorized MBR, Authorized session-AMBR), the Revalidation time limit, the Time Condition, etc. parameters to notify SMF the PDU session related policy information for localized services has been updated.

-	Step 5: hosting network advertise the localized services to the UE according to the network rules (e.g. at the specific time and location).

-	When AMF received the modified access and mobility related policy from PCF in step 4, AMF will deploy and store this updated policy information, then provisioning the Service Area Restrictions (e.g. the validity of location), the list of the identification of the localized services (such as DNN and/or S-NSSAIs) to the UE, and provisioning the UE-AMBR, Service Area Restrictions to the NG-RAN.

*** END CHANGES ***