3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208758

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#5 update evaluation and conclusion

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2/ Rel-18

Abstract: Updates conclusions for KI#5.

1. Introduction/Discussion

1. EAS Deployement information is to be transferred between PLMNs for discoverying EAS in shared EHE.

2. To support traffic transmitted between two PLMNs, N6 routing information could be included in EDI for routing specific edge traffic (identified by FQDN in EDI) from serving PLMN to EHE with low latency transmission.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48-40.

# * * * * First change * * * *

8.5	Interim conclusion for KI#5: GSMA OPG impacts and improvements for EHE operated by separate party

To enable the discovery of an EAS deployed in another PLMN than the one serving the UE (e.g. an EAS deployed in a shared EHE).

EAS Deployment Provision

-	The serving PLMN is provided with the information needed to support UE's to discover and connect to the shared EHE.

- 	the EDI may transferred to the serving PLMNs via interaction with the NEF;

- 	the PLMN specific informaiton in EDI could be translated by NEF;

-	the PLMN specific informaiton in EDI could be preconfigured at SMF in serving PLMN.

-	PLMN ID could be included in EDI for indicating the corresponding PLMN of EDI.

EAS Discovery Procedure

-	EAS discovery procedure defined in TS 23.548 [3] will be reused.

Traffic transmission between two PLMNs

-	When the traffic transmitted between two PLMNs, the UPF in serving PLMN should support mechanisms to facilitate low latency transmission. N6 routing information could be included in EDI for routing specific edge traffic (identified by FQDN in EDI) from serving PLMN to EHE with low latency.

# * * * * End of changes * * * *