SA WG2 Meeting #S2-153E	S2-2208771

10 - 17 Oct, 2022, E-Meeting

Source: 	Ericsson, Qualcomm

Title: 	KI#6: Evaluation and conclusion of Key Issue #6

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes an evaluation and a conclusion of Key Issue #6.

# Discussion

There are 10 solutions in TR 23.700 v1.1.0 addressing key issue #6. These are: Solution #7, #8, #9, #10, #13, #17, #25, #38, #39 and #47. In this contribution an evaluation of these solutions is proposed. The aims and aspects of key issue #6 are as follows:

This key issue aims to study whether the existing mechanisms for overload control in the network can support all the requirements in clause 6.41 of TS 22.261 [2] "Providing Access to Local Services" and whether any enhancements or additional mechanisms need to be defined. The following aspects will be considered:

-	How to mitigate user plane and control plane overload caused by a high number of UEs returning from a temporary local access of a hosting network to their home network in a very short period of time.

-	How to minimize the impact on the UE's communication e.g. to prevent user plane and control plane outages when returning to a home network together with other high number of UEs in a very short period of time, after terminating their temporary local access to a hosting network.

NOTE:	The solution for this KI may need to consider mechanism developed for KI#5 "Enabling access to localized services via a specific hosting network".

The solutions are divided in two groups, one called "The avoidance-based method", and another called "The runtime-based method". The methods and solutions are described and evaluated, and a conclusion is proposed.

# Proposal

Add the following evaluation and conclusion to TR 23.700-08.

*** BEGIN CHANGES ***

# 7	Evaluation

Editor's note:	This clause provides an evaluation of the solutions.

## 7.6	Key Issue #6: Support for returning to home network

The solutions for KI#6 are proposed to address the UEs returning from the Hosting Network to their Home Network (i.e. HPLMN/VPLMN, SNPN) when the Localized Services in the Hosting Network are completed. Under the condition, it may cause many UEs returning back to e.g. their Home Network and this may result in signalling overload in their Home Network.

NOTE:	Home Network is used, but the network selected by the UE depends on the UE's network selection and what networks are available e.g. UE can return to VPLMN different from the PLMN the UE was registered to before selecting the Hosting Network.

When considering solution proposals for KI#6, there are two categories for handling signalling overload resulting from the UEs returning to their Home Network:

-	The avoidance-based method: the UEs is (pre)-configured with a timer/waiting time based on the Validity Information of the Localized Services e.g. by the Home Network or by the Hosting Network.

-	The runtime-based method: the UEs directly return to their Home network when the Localized Services terminate and the Home Network will use the existing mechanisms to handle the signalling overload if the congestion happens.

For avoidance-based method (i.e. Solutions#9, 10, 13, 38, 47), the Home Network may:

-	negotiate with Hosting Network the number of UEs in a time unit that can trigger PLMN selection (to potentially return to Home Network); or

-	negotiate with Hosting Network or configure UE the timer/waiting time for the UEs before triggering PLMN selection.

For runtime-based method (i.e. Solutions#8, 17, 25, 39), the Home Network applies the existing congestion control and overload control to the UEs for returning from the Hosting Network but cause congestion overload. The UEs registers to the Home Network by indicating they are back from the Hosting Network. If the Home Network is in a congestion condition, the Home Network can reject the UE with a cause and a back-off timer.

To compare the avoidance-based and runtime-based methods, the differences are below:

-	The avoidance-based method can allow the UE to return to their Home Network in a distributed or stagged manner to prevent the congestion or overload conditions happen in their Home Network.

-	The runtime-based method directly applies the existing mechanism optionally with adding the indication that the UE is returning from the Hosting Network as an indication to the Home Network.

The avoidance-based method potentially delays the UEs return to the Home Network also for the cases when the Home Network is not overloaded/congested. It is also expected that the user itself can decide to stop using the Localized Service i.e. in these cases the UE would return back to the Home Network disregarding the spreading applied for the UE's return.

The runtime handling has allowed the UEs to initiate the Registration procedure as soon as wanted and then if the Home Network is congested rejected the UE with a back-off timer in which the Home Network has been through the signalling congestion from those returning UEs from the Hosting Networks (unless Home Network is so congested that e.g. UAC is used). Additionally, it is not necessary for those returning UEs to indicates it returns from the Hosting Network. Those returning UEs has no higher priority than others UEs which are not returning from the Hosting Network.

As stated above, a UE returning from a Hosting Network after using a Localized Service would not have higher priority than any other UEs, i.e. the prioritization among UEs would better follow existing mechanisms and therefore there is no need to add a separate indication that the UE is returning from the Hosting Network as an indication to the Home Network. However, as the avoidance-based method potentially delays the UEs return to the Home Network also for the cases when the Home Network is not overloaded/congested it is better to allow the UE return as soon as the Localized Service ends or whenever the UE (e.g. user) decides to stop using the Localized Service and therefore leaves the Hosting Network before the end of the Localized Service. The end time of the Localized Service can be adapted on application layer by existing means i.e. without 3GPP impacts.

Table 7.6-1: Summary of solutions for KI#6

| Solution #7 High level flow for localized service support | The solution provides the high level entities and procedures for providing access to localized services. This solution does not cover the impact on different entities, services, and interface, will be covered by other specific solutions. |
| --- | --- |
| Solution #8 Reuse existing mechanisms for Control Plane Load Control, Congestion and Overload Control | The proposed solution is to reuse existing mechanisms for Control Plane Load Control, Congestion and Overload Control, to mitigate signalling overload when large number of UEs return to their home network. As the solution proposes to reuse the existing mechanisms, it is noted that there is no impact on services, entities and interfaces. |
| Solution #9 Prevention of overload build up at home network using AMF based congestion control when local service is over | This solution proposes to deregister UEs accessed to localized services in a staggered manner before the termination of local hosting network. In order to achieve this, two mechanisms, namely, usage of network availability timers and specific cause code to trigger a controlled deregistration are proposed. The solution assumes that the hosting network is able to determine the exact timing when the local service stops and UEs need to leave, which may not be the case for most scenarios. The solution has an impact on UE and AMF. For the UE, the impact is to support new cause codes, service availability timer and related implementation. For the AMF, the impact is to handle new timer for UEs accessing to local services and to support new cause code in network-initiated de-registration request. The solution prevents any overload condition cause by simultaneous registration request coming from all the returning UEs. |
| Solution #10 Solution for discovery and selection of NPN hosting network and localized services | Although this solution mainly addresses KI#4, it also provides a solution for KI#6. It is proposed that either the UE applies a random delay before initiating a PLMN selection or HPLMN (or Credentials Holder) configures UEs with slightly different end time to the validity time period. The end time of validity condition may not aways be the same as the real end time of the service. It can't be assumed that UE's returning to the home network is triggered by the time validity condition. |
| Solution #13 Exposure enhancements to support providing access to localized services | This solution addresses a scenario in which the UE accesses its home NW via the hosting NW, using the NWu (for PLMN) reference point in the hosting NW. (See TS 23.501 Annex D.3). Typically, this could be used for accessing IMS (via the home NW) when registered with an SNPN. The solution offers a mechanism to "ensure service continuity" when the UE returns from the hosting NW to the home NW. The UE reports Nwu interface status + HPLMN ID to the AMF in the hosting NW. The AMF in the hosting NW groups UEs into 1 of 4 possible groups, and provides back-off timer to the UEs, to be used when they connect to their home NW. The solution has an impact on UE and AMF. The UE, when receiving the end indication of the localized service from the hosting network AMF, checks that all local services are ended, and reports Nwu interface status and related home network identity (e.g. PLMN ID) to the AMF. The hosting network AMF groups the UEs and send back-off-timer to the UEs. |
| Solution #17 UE Group specific NAS level congestion control | The solution proposes to use group specific NAS level congestion control mechanism to mitigate user plane and control plane overload. The home network associates UEs temporarily accessing localized services to an Internal-Group Identifier, which can be specific to each local hosting network and/or service and applies UE group-specific NAS level congestion during the return of UEs. The solution has an impact on UE/AMF to support localized service related de-registration information, NEF to support exposure from local hosting networks, UDM to support local hosting network/service information in Internal-Group identifier and AMF/SMF to apply Group Specific NAS level congestion control. This solution requires UE to request registration in the home network and then the AMF of the home network applies UE group-specific NAS level congestion control. Naturally, this solution does not prevent an overload condition as the RAN and the AMF will get registration request from all the returning UE and this may lead to increased load in the system. |
| Solution #25 Temporary network reselection for localized service support | The home network authorizes the UE to use Temporary Network Reselection under certain criteria, and when criteria is no longer met the UE deactivates the Temporary Network Reselection and selects a new network based on the existing network selection procedures defined in TS 23.122. The criteria to use Temporary Network Reselection for a localized service can be set by the home network as to perform spreading of automatic selection. |
| Solution #38 Sequential deregistration by hosting network | This solution proposes to control the number of returning UE based on service level agreement between the home network and hosting network. The solution relies on that the hosting network can group the UEs according to their home network, but it remains unclear how it is achieved. The solution has an impact on UE and AMF. For the UE, the impact is to support finishing deregistration procedure where the termination of the localized service is indicated by AMF and to support return of the UE to its home network. For the AMF, the impact is to support indication of localized service termination as part of the Deregistration request message and to support triggering deregistration in a sequence manner in a way that the maximum number of returning UEs at a given units of time does not exceed the configured number at AMF. |
| Solution #39 Local hosting network specific back-off timer | This solution proposes a local hosting network-specific flexible back-off timer range and back-off timer assignment to spread out the registration attempts over time and limit the number of users attempting to register back to their home network simultaneously in order to avoid signalling overload and unnecessary waiting times for the returning users. The solution effectively avoids using uniformed non-optimal back-off timers for all UEs. The solution has an impact on UE and AMF. For the UE, the impact is to support sending the local network indication as part of the new registration request ("re-registration from hosting network" or "local network access indication"). For the AMF, the impact is to support new registration type/indicator related to local hosting network re-registration and assignment of a local hosting network-specific back-off timer range. This solution requires UE to request registration in the home network and then the AMF of the home network provides a back-off timer. This solution does not prevent an overload condition as the RAN and AMF will get registration request from all the returning UE and this may lead to increased load in the system. |
| Solution #47 Solution for mitigating overload at home network | In this solution the AMF in the hosting NW subscribes to a maximum re-registration rate, "maximum number of returning UEs . . . per unit time", from the AMF in the home NW. At the end of the localized service, the AMF in the hosting NW sends deregistration request messages with that rate to UEs. In the solution, this is referred to as "Adaptive deregistration procedure". The solution has an impact on UE, AMF and UDM. The UE, when receiving the AMF-initiated Deregistration Request message, shall set a random timer and wait until the random timer expires before initiating registration back to the home network or another hosting network. The home network AMF notifies the hosting network AMF of the allowed maximum number of UEs that can return to the home network per unit time. The UDM coordinates with AMFs for selection of the allowed maximum number of returning UEs from hosting network based on the notification from the home network AMF. |

*** NEXT CHANGES ***

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.6	Key Issue #6: Support for returning to home network

The following principles from the evaluation are recommended to be considered during the normative work:

-	The home/serving network can re-use existing mechanisms for Control Plane Load Control, Congestion and Overload Control described in clause 5.19 of TS 23.501 [3] when load level reaches a certain threshold and overload control mechanism are triggered. A UE returning from a Hosting Network after using a Localized Service would not have higher priority than any other UEs and therefore there is no need to impact existing mechanisms for Control Plane Load Control, Congestion and Overload Control.

-	Additional mechanisms can be implemented to ensure spreading of return of the UEs to home network without any impacts to 3GPP procedures e.g. by provisioning UEs with slightly different validity end-time of the Localized Service. This can be captured as informative text, e.g. a NOTE, in normative specification.

NOTE: 	It is assumed that the spread validity end-times provided to UEs are equal or earlier than the actual validity end-time of the localized service in a hosting network (e.g. determined by SLA as per key issue #3).

*** END CHANGES ***