SA WG2 Meeting #S2-153E	S2-2208773

10 - 17 Oct, 2022, E-Meeting

Source: 	Ericsson

Title: 	KI #4: Update evaluation of Key Issue #4

Document for: 	Agreement

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes an update of evaluation of Key Issue #4.

# Discussion

In SA2#152E, there were solution updates (Sol#10, #12, #18, #26) and new solutions (#40, #41, #42, #43, #44) added in TR 23.700-08 to address Key Issue #4. The evaluation for Key Issue#4 based on the new content should be updated accordingly.

The following two aspects in the key issue are also evaluated in this contribution:

-	How to ensure the localized services are accessed by UE according to the conditions when and where the localized services are allowed to be accessed by the UE.

-	How the UE is provisioned with credentials (if required) to access the selected localised services provided via the hosting network.

# Proposal

Add the following update to TR 23.700-08.

*** BEGIN CHANGES ***

## 7.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

Editor's note:	The evaluation in this clause has to be updated to consider the solutions agreed during the SA2#152 meeting.

### 7.4.1	General

The evaluation for KI #4 solutions is made for each component listed below:

-	The content of the localized service information that is used by UE to perform discovery/selection/access to the hosting NW for localized services.

-	From where and how UE obtains such localized service information.

-	How the localized service information is used by UE.

-	What credentials are used to access hosting network and how to obtain them if required.

-	How localized service is accessed in hosting network per agreed conditions, e.g. time/location.

### 7.4.2	Evaluation for the content of the localized service information

Construct the information on localized service basis (Sol #11, #12, #13, #15, #24, #27, #31, #32, #34, #42).

-	The information includes an element to distinguish different localized service (e.g. identifier, name of the localized service), validity conditions (time/location), one or more associated hosting networks' information, such as:

-	hosting network identifier,

-	temporary credential or credential type for accessing hosting network,

-	QoS,

-	cost,

-	information transmitted in the Hosting network's SIB

(e.g. CAG IDs, localized service ID) used by the UE to select the Hosting network.

- Other information for future extension and optimization, e.g. frequency band used by the hosting network.

NOTE:	Credential provisioning mechanisms need to be evaluated by SA WG3.

-	The benefit of this principle is to provide UE/end user with relationship between localized services and hosting networks, so that it enables the possibility for UE/end user to make request on specific localized service and for network to inform UE what are the available localized services and over which hosting networks these localized services are provided.

Construct the information based on priority list for Hosting network selection (Sol #10, #14, #18, #25, #29, #30, #33, #36, #43).

-	The information is a new priority list for Hosting network selection. Each entry on the list includes hosting network identifiers, validity conditions (time/location), etc.

-	This principle requires other solution components (e.g. Sol #12, #13, #24) to complement, since the information does not include reference to the localized services available over the different hosting networks included in the priority list and therefore it assumes that either UE or the network providing this information knows beforehand which localized service end user intends to access or what are the localized services supported by the hosting network on the priority list.

The localized service information in Sol #22 provided by hosting network to UE, is not used for selecting hosting networks, but rather for UE to know how to access a service in the hosting network which can be addressed by existing functionality, such as URSP rules.

### 7.4.3	Evaluation for from where and how UE obtains the localized service information

UE can obtain the localized service information from application server, serving network, home network, or hosting network.

UE fetches the information from an application server (Sol #14, #28, #31).

-	When a full set of new subscription/credential is provided to UE via application server, this principle has less system impact on 5GS. But on the other hand, home network subscription/credential is not utilized and the UE needs to handle co-existence of multiple subscriptions/credentials on device.

-	When the information includes a list of hosting networks and an indication that home network credential is to be used, there will be impact on network selection. Application server obtaining such information via network exposure is not necessary, since it is covered as part of the SLA between localized service provider and hosting network operator.

UE fetches the information from home network (Sol #10, #13, #14, #15, #18, #23, #24, #29, #30, #31, #33, #36, #43).

-	This principle assumes that home network is capable of managing the localized service information data.

-	The impact is about how home network is provisioned with the localized service information data, and how UE is provisioned by the home network.

-	Most of the solutions following this principle propose that the home network provides the localized service information to UE via SoR procedure in combination with the principle that the localized service information is constructed based on a priority list for (hosting) network selection as described in clause 7.4.2.

-	Some solutions assumed that the SoR-AF within the home network should be involved as the data source and the mechanism regarding how SoR-AF obtains such data should be out of 3GPP scope. Some solutions (Sol #14, #43) assumed the data source for SoR procedure is obtained from other NFs than SoR-AF.

NOTE X:	In the current specifications, SoR-AF is the only entity that creates the SoR message in case there is a need to dynamically change the content of the SoR information.

-	Since SoR procedure is mainly used for network discovery and selection, it is not suitable to be extended to provision UE with data more than required for the UE's actual network discovery and selection.

NOTE Y:	The current specifications provide the means for the serving network to provision the UE with information relevant for the serving network once the UE has registered to the hosting network e.g. network slicing information.

-	Sol #24 proposes another variant for UE to fetch the information from home network, that is via home network UDR and new UE policy. UE policy has the potential to include necessary data of the localized service, and possibility to be extended in the future. The new UE policy can be delivered to UE by PCF in home network, or PCF in serving network using N24.

UE fetches the information from serving network (Sol #11, #12, #24, #27, #32, #42).

-	This principle assumes serving network (either home network or another network which UE is accessing using home network credential) is capable of storing the localized service information data. This principle enables multiple information sources for UE to obtain localized service information.

-	In case the current serving network is not the home network, this principle also provides mechanisms for home network to authorize or determine the final data that UE will receive, where the serving network either checks a subscription flag (Sol #12, #24, #32) or passes the localized service data to home network for authorization and delivery to UE (Sol #12, #42).

-	The impact is about how serving network is provisioned with the localized service information data, and how UE is provisioned by the serving network.

-	Some solutions following this principle proposes to use AMF/SMF in serving network to store the localized service information data. In this case, the provisioning to serving network is based on local configuration which has less impact, but it requires the UE is served by the correct AMF/SMF configured with such information (e.g. UE is located in the overlapping area where the hosting network is available) or requires all the AMF/SMF in the serving network are configured with such information. The data is provisioned to UE via MM NAS or SM NAS message (it is TBD which procedures/messages are impacted).

-	Other solutions following this principle, proposes to use UDR in the serving network to store the localized service information data as application data for any UE registered to the serving network. This enables the flexibility that if the serving network is not the home network, UE can obtain localized service information data from either the serving network, or from the home network, or from both. The impact is to extend service specific parameter provisioning for the serving network to receive the data (see TS 23.502 [4] clause 4.15.6.7) and new UE policy to provide the data to the UE.

UE fetches the information from hosting network (Sol #22, #26, #34).

-	This principle assumes a hosting network is selected prior to a localized service is selected, since the localized services information arrives later from the hosting network. It can happen that the fetched information is not interesting for the end user.

-	The impact is to extend broadcast with capability for localized service, service discover policy that allows UE to query information from hosting network while in RRC-IDLE/INACTIVE with serving network, new on-demand SIB, or code for URL. Another impact is to deliver localized service information via registration reject/accept message.

NOTE:	Privacy aspect needs to be evaluated by WG SA3.

-	This principle implies that in some scenarios the UE needs to suspend the connection with or suppress the paging from the serving network when getting information from hosting network, which can cause service interruption.

There are different triggers to provision the information to UE:

-	UE request as trigger (Sol #11, #13, #15, #24, #27, #32, #34, #42).

-	UE location as trigger (Sol #11, #12, #18, #27, #32).

-	UE subscription data change via external parameter provisioning as trigger (Sol #14, #23, #43).

-	UE Steering of Roaming information update (SoR based solutions).

UE request as trigger assumes UE/end user is aware of the desired localized services. UE location as trigger requires the serving network is aware of which hosting network is available in which serving network tracking areas, and the information sent to UE is unsolicited. UE subscription data change and SoR based solutions restrict the information source to home network only.

### 7.4.4	Evaluation for how the localized service information is used by UE

New network selection mode is proposed (Sol #10, #25, #30, #33), where UE selects only hosting network when it intends for localized services and bypasses or temporary stops existing network selection procedure. This is needed in case UE is using home network credential to access hosting network, since home network and hosting network could be collocated in the same area and UE will always prioritize home network based on existing network selection mechanism. The new network selection mode can be requested by UE and authorized by home network (Sol #25).

The time and location validity condition along with other localized service information provided to UE can be used as a criteria for the UE to determine when and where to enter the new network selection mode.

Except the validity conditions, other methods for UE to determine whether to enter hosting network selection mode are also proposed, e.g. serving network to indicate to UE the presence of hosting network via either NAS or broadcast (Sol #12, #29), the availability of hosting network detected by UE regular scan (Sol #10), which requires extra system impacts.

### 7.4.X	Evaluation for what credentials are used to access hosting network and how to obtain them

If home network credentials are also used to access hosting network, existing mechanisms can be re-used for enabling access to hosting network, such as roaming architecture or Credentials Holder function.

Minor enhancements are proposed to indicate to the UE that hosting network support CH function (Sol #12), or to indicate to the UE that home network credentials can be used as credential from CH to access specific hosting network (Sol #40). Whether hosting network support of CH function can be detected by UE via existing SIB broadcast. Whether home network can act as CH can be implicitly determined by whether there are priority lists for SNPN selection configured/provisioned on UE. But if hosting network selection does not use existing CH/User controlled priority lists for SNPN selection, there will be a gap for UE to know if home network credential can be used, or it needs to obtain a new set of credentials from somewhere else. When home network credential is used to access hosting network, it requires connection between home network and hosting network as described in TS 23.501 [3] clause 5.30.2.9.

If separate credentials from the home network credentials are needed to access hosting network, the credentials can be obtained via:

-	application layer (Sol #12, #14, #28).

-	NAS layer (Sol #12, #13, #14, #15, #42).

-	enhanced onboarding procedure in SNPN type of hosting network (Sol #12, #22, #41).

-	enhanced user plane remote provisioning procedure in serving network (Sol #40).

Provisioning credentials via application layer is out of 3GPP scope.

Provisioning credentials via NAS layer needs to be coordinated and agreed with WG SA3, since there is no existing NAS layer procedure suited for credential provisioning based on previous study conclusion in TR 33.857 [X] (Rel-17 FS_eNPN_SEC).

Existing onboarding procedure in SNPN requires that the UE is pre-configured with Default UE credentials and other information, while Sol #12 proposes UE to obtain such onboarding related credentials/configuration via control plane signalling.

Onboarding without primary authentication requires coordination with WG SA3.

Minor enhancement for user plane remote provisioning enables UE to request specific hosting network related provisioning server (Sol #40), also enables the possibility for UE to obtain onboarding related credential/configuration via user plane.

### 7.4.Y	Evaluation for how localized service is accessed in hosting network per agreed conditions

The validity conditions of the localized service provided to UE can restrict the UE to access a hosting network per agreed conditions.

For hosting network supporting large areas and multiple localized services, Sol #10 proposes to divide the hosting network SNPN to smaller sub-networks via a new identity (Subnetwork ID), and then UE in hosting network can be restricted to certain areas. The access restriction is on cell level, which prevents UE from selecting/camping on the cell. Such strict access control is more suited and needed in e.g. factory production environment. There are also many existing mechanisms to divide the network to smaller parts, e.g. network slicing, forbidden area restriction, service area restriction, etc.

Sol #11, #27, #45 proposes to re-use existing LADN mechanism to restrict UE from accessing the DN when UE is out of the specific area (TAI). The access restriction is on PDU session level. Sol #45 also proposes to utilize the location report control in case the granularity of localized service is not on TAI level, but on cell level.

Sol #35, #46 proposes to re-use URSP rule to enable UE to access localized service when the RSD validity conditions are met. The access restriction is also on PDU session level.

*** END CHANGES ***