SA WG2 Meeting #153E eMeeting	S2-2208783

Elbonia, Oct. 10 – 17, 2022 	(revision of S2-20xxxxx)

Source:	NTT DOCOMO

Title:	TR 23.700-80: KI#7 – Conclusion

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes a KI#7 conclusion.

# Background

Based on the evaluation in clause 7.7 and the solutions proposed in clause 6, the following conclusions are proposed to be included in clause 8.7.

As stated in clause 5.7, the following aspects need to be addressed in KI #7:

- Assistance of 5GS to selection of UEs for FL operation

- Performance monitoring/exposure

- Assistance of 5GS to improve FL performance

Therefore, we have proposed principles for each aspect of the bullet point above.

# Proposals

***** Start of Change *****

## 8.5	Key Issue #7: 5GS Assistance to Federated Learning Operation

Following principles are proposed for normative:

NOTE 1: Security aspects (e.g., authorization and information exposure to AF) for FL operation are discussed in SA WG3.

- Assistance of 5GS to selection of UEs for FL operation:

  - 5GC provides FL assistance to AF for UE(s) selection for FL operation.

- NEF is enhanced to serve AF’s FL assistance information requests. Based on operator policy, the NEF translates the input request from AF (as described in clause 6.2.5, TS 23.501) and provides it to the 5GC NF(s).

NOTE 2: The details of the input and output parameters are discussed in normative work.

- NWDAF is enhanced to provide analytics information for UE selection FL operation.

NOTE 3: Whether the existing analysis ID(s) is enhanced or a new analysis ID is defined for FL operation is discussed in normative.

- Based on the FL assistance information from 5GC, the AF determine the final UE list for FL operation.

NOTE 4: How FL assistance information is used by AF to select UEs for FL operation is outside the scope of SA WG2.

- Performance monitoring/exposure: The outcome of KI #1 and KI #6 can be used for monitoring the resources used for FL as well as its performance, therefore, no normative work for FL purposes is expected.

- Assistance of 5GS to improve FL performance:

  - AF consumes the NWDAF analytics to improve the FL performance.

Editor Note: Whether any enhancements are required for the NWDAF analytics is FFS.

NOTE 5: It is up to the AF logic how FL performance is improved.

  - The outcome of KI #6 can be used to enhance the QoS/Policy to improve the FL operation.

***** End of Change *****