3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208791

Elbonia, October 10 – 14, 2022	(revision of S2-220xxxx)

Source:	CATT

Title:	KI#2: Update conclusion for time synchronization service enhancements

Document for:	Approval

Agenda Item:	9.7.1

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: update conclusion for KI #2: Time synchronization service enhancements in TR 23.700-25.

1. Discussion

This paper is to conclude the scenario when UE has moved outside the AF Requested Coverage Area for which the AF has requested ASTI based time distribution.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-25.

# * * * * First change * * * *

8.2	Key Issue #2: Time synchronization service enhancements

The following bullet points summarize the principles for the way forward:

-	To request a Requested Coverage Area for time synchronization services, AFs within the operator's domain formulates a spatial validity condition using a list of Tracking Area or Cell identities, but AFs outside the operator's domain use a geographical area (e.g. a civic address or shapes) instead, while NEF transforms this information into a list of Tracking Area or Cell identities.

-	The AF provides the Requested Coverage Area for time synchronization services, UE list and time synchronization services (ASTI or (g)PTP based) to TSCTSF, optional via NEF.

-	The TSCTSF need to query with UDM to check whether ASTI or (g)PTP based time synchronization is allowed to be requested by AF for the UE.

-	The TSCTSF is responsible of activate/deactivate time synchronization services (ASTI or (g)PTP based) considering the spatial validity condition provided by the AF.

-	The TSCTSF discovers the AMF(s) serving the list of TA(s)/Cell(s) that comprise the spatial validity condition using the NRF.

-	The TSCTSF subscribes to UE's presence in Area of Interest at the discovered AMF(s).

-	The TSCTSF determines (based on notifications from the discovered AMF(s)) whether the targeted UE(s) are inside or outside the AF Requested Coverage Area.

Editor's Note: Whether and then how to solve the Coverage Area on cell-level granularity is FFS.

-	The TSCTSF activates time synchronization services for UE(s) that are inside the Requested Coverage Area. If the AF has requested (g)PTP based time distribution, then TSCTSF creates the PTP port in DS-TT and adds it to the PTP instance as described in clause 4.15.9.3.2 in TS 23.502 [3].

-	TSCTSF notifies the AF with the indication of 5G access stratum time distribution (enabled, disabled) for the targeted UE(s) for which the AF has requested ASTI-based time distribution that are inside or move inside or outside of the Requested Coverage Area.

-	If the TSCTSF has determined (e.g. notified) that the UE has moved outside the AF Requested Coverage Area for which the AF has requested (g)PTP based time distribution, then TSCTSF temporarily removes the UE/DS-TT from the PTP instance:

-	If the DS-TT is configured to send Sync, Follow_Up and Announce messages for the related PTP instance, then TSCTSF deactivates the Grandmaster functionality in the DS-TT using PMIC (see also clause K.2.2.4 of TS 23.501 [2]) and removes the DS-TT from the PTP instance (see also clause K.2.2.1 of TS 23.501 [2]).

-	If NW-TT is configured to send Sync, Follow_Up and Announce messages on behalf of the DS-TT, then TSCTSF deactivates the Grandmaster functionality on behalf of the DS-TT in NW-TT using UMIC (see also clause K.2.2.4 of TS 23.501 [2]) and removes the DS-TT from the PTP instance (see also clause K.2.2.1 of TS 23.501 [2]).

-	The TSCTSF informs the AF for the impacted UE by indicating the PTP port state as Inactive for the related DS-TT PTP port.

-	If TSCTSF has determined (e.g. notified) that the UE has moved inside the AF Requested Coverage Area for which the AF has requested (g)PTP based time distribution, then TSCTSF adds the DS-TT PTP port to the PTP instance and also (re-)activates the Grandmaster functionality. The TSCTSF informs the AF for the impacted UE by indicating the PTP port state as Active for the related DS-TT PTP port.

-	If the TSCTSF has determined (e.g. notified) that the UE has moved outside the AF Requested Coverage Area for which the AF has requested ASTI based time distribution, then TSCTSF temporarily deactivate ASTI time synchronization service by disabling the indication of 5G access stratum time distribution for the targeted UE(s).

-  If TSCTSF has determined (e.g. notified) that the UE has moved inside the AF Requested Coverage Area for which the AF has requested ASTI based time distribution, then TSCTSF enables the indication of 5G access stratum time distribution for the targeted UE(s) based on the AF-requested requirement of time synchronization service.

# * * * * End of changes * * * *