SA WG2 Meeting #SA2-153E 	S2-2208829

Oct 10 – 17, 2022, Electronic meeting

Source:	Xiaomi

Title:	XRM_KI#3Sol#47 the Update of Measurement Description

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This document proposes the update of measurement for KI#3

- Discussion

During the last meeting, the measurement of the QoS parameters e.g. data rate, delay difference and round-trip delay is discussed in the KI#3 conclusion. The exist mechanism of the QoS Monitoring can be reused for the measurement. However, it is not clarified clearly in the description of related solutions e.g. Sol#47.

- Proposal

It is proposed to capture the following update in TR 23.700-60.

# * * * * Start of the change * * * *

6.47.2	Description

This solution proposes that AF can subscribe delay difference and two way delay of data flows to 5GS. Based on the notification and report of the delay status information, the AF can provide the coordinated service requirement to the 5GS for the tactile and multi-modal flows, during the AF session setup procedures.

The delay difference follows the description in Clause 6.1 of TR 23.700-60, Solution #1: Tactile and multi-modality traffic flows coordination transmission for one UE. It is assumed that the data flows of delay difference measurement belongs to a single UE.

The two way delay is defined as the sum of UL and DL Packets Delay, which is described in Clause 6.29 of TR 23.700-60, Solution #29: Two Way Delay Budget. It is assumed that the NG-RAN and PSA UPF are time synchronised, then the one way packet delay monitoring between NG-RAN and PSA UPF can be supported.

Then mechanism of the QoS Monitoring applied for the real time packet delay measurement for flows can be reused. The SMF may send a QoS Monitoring request to the PSA UPF and NG-RAN to request the QoS monitoring between PSA UPF and NG-RAN, to activate the UL/DL packet delay measurement between UE and PSA UPF for a QoS Flow during the PDU Session Establishment or Modification procedure, as specified in Clause 5.33.3 of TS 23.501 [2]. The QoS Monitoring on UL/DL packet delay between NG-RAN and PSA UPF can be performed on different levels of granularities, i.e. per QoS Flow per UE level, or per GTP-U path level, subject to the operators' configuration, and/or 3rd party application request, and/or PCF policy control for the XRM services.

# * * * *end of change * * * *