SA WG2 Meeting #SA2-153E 	S2-2208831

Oct 10 – 17, 2022, Electronic meeting

Source:	Xiaomi, China Mobile

Title:	Conclusion for KI#9

Document for:	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This document proposes conclusion for KI#9

- Discussion

Solution#34, #35, #61, #74 and #75 are related with KI#9: Trade-off of QoE and Power Saving Requirements.

This paper proposes text for the Key Issue #9 conclusion section.

- Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

8.X	Conclusions for Key issue#9: Trade-off of QoE and Power Saving Requirements.

For Key Issue #9 "Trade-off of QoE and Power Saving Requirements", the following principles are recommended as conclusions for the normative work:

The UE

- provides the status information (i.e. Power Mode) to PCF for the policy determination and adjustment. The status information can be provided to the PCF via NAS or AF.

Editor’s Note:	Whether the Power Mode is provided to the PCF via NAS or AF is FFS.

The PCF

- Get the media codec information from the AF, and generate or update the PCC rules considering the application information (i.e. media codec information).

- Get the status information (i.e. power mode) via NAS or from AF, and generate or update the PCC rules considering the UE status information (i.e. Power Mode).

The RAN

- Alternative 1: Update the QoS parameters or QoS profiles considering the Overheating information, which can be provided to RAN by the UE already.

- Alternative 2: Get the UE status information (i.e. power mode, memory status) from AF, and trigger the QoS parameters or QoS profiles update considering the power mode or memory status.

Editor’s Note:	The above information may be updated based on the discussion. E.g. whether the UE status information (i.e. power mode, memory status) or the indication based on the UE internal status information is provided to the network.

# * * * *end of change * * * *