3GPP TSG-SA WG2 Meeting #153e 	S2-2208836

# Elbonia, 10-17 October, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3583 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF event exposure service to NWDAF via the SMF in TS 23.502 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 9.25.2 |  |  |  |  |  | Date: |  |  | 2022-09-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This clause contains the detailed description and the procedure for UPF event exposure service to NWDAF via the SMF. The NWDAF can subscribe to the UPF data through the SMF, and the UPF will directly send the collected UPF data to NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the detailed description and the procedures for UPF event exposure service to NWDAF via the SMF |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for UPF event exposure service to NWDAF via the SMF will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.X(All new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 23.288 CR 0551 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE(All new text)

4.15.X	UPF event exposure service to NWDAF via the SMF

4.15.X.1	General

This clause contains the detailed description and the procedures for UPF event exposure service to NWDAF.

The NWDAF can subscribe to the UPF data via the SMF, and the UPF will directly send the collected UPF data to NWDAF.

4.15.X.2	Information flow for UPF event exposure service to NWDAF

Figure 4.15.X.2-1: UPF Information Exposure to NWDAF

For case 1, where the NWDAF requests UPF information Exposure for a certain UE or a group of UEs, step1 and 2 are as follows:

1.	The NWDAF invokes Nudm_UECM_Get service operation to retrieve the appropriate SMF by providing UE ID and NF type.

2.	The UDM provides a Nudm_UECM_Get response to the NWDAF with the corresponding SMF.

For case 2, where the NWDAF requests UPF information Exposure for a certain AOI, steps 1 and 2 are as below:

1.	The NWDAF discovers the SMF instances by invoking Nnrf_NFDiscovery request with UPF Event Exposure Service towards NRF, including Area of Interest, to get the SMF which has the capability to subscribe to UPF on behalf of NWDAF for UPF information reporting.

NOTE X:	For UPF, "Event Exposure Service Name" indicates that the UPF supports information reporting. For SMF, "Event Exposure Service Name" indicates that the SMF supports to subscribe to UPF on behalf of NWDAF for UPF information reporting.

2.	The NRF responses with SMF ID and SMF IP address to NWDAF.

3.	The NWDAF sends the request to the SMF to subscribe UPF data, including the following information:

-	Notification Target Address (NWDAF address).

-	Indication of UPF Event Exposure Service.

-	Event Filter Information: S-NSSAI, Application Id, Area of Interest.

-	Target of Event Reporting: a UE or a group of UEs or any UE.

-	Subscription Information:

-	UL/DL Throughput, UL/DL packets or number of connections.

-	N3 delay, N6 delay, E2E delay, UL/DL packet loss, or UL/DL packet retransmission.

-	UL/DL Data Volume.

4a.	For case 2, the SMF discovers the UPF instance by invoking Nnrf_NFDiscovery request with UPF Event Exposure Service towards NRF, including Area of Interest, to get the UPF which supports user plane data reporting. And the NRF responses with UPF IP address to the SMF.

4b.	For case 2, the SMF subscribes to AMF by invoking Namf_EventExposure_Subscribe with Event "Number of UEs present in a geographical area" to get the UE list 1 in the AOI.

4c.	SMF locally determines the UE list 2 for user plane data reporting which is included in the Target of Event Reporting sending to UPF.

If the Target of Event Reporting from NWDAF is a UE or a group of UEs, the SMF determines UE list 2 for user plane data reporting which locate(s) in the AOI by matching the UE or group of UEs from Target of Event Reporting with the UE list 1 from AMF in step 4b.

If the Target of Event Reporting from NWDAF is any UE, the UE list 2 is the UE list 1 from AMF in step 4b.

4.	The SMF sends the request to the UPF over Session Establishment Request/Response message including the NWDAF address, Event Filter Information, Target of Event Reporting, and Target Subscription Information. For case 1, Target of Event Reporting is the certain UE or the group of UEs same as step 1, For case 2, Target of Event Reporting is the UE list 2 from step 4c.

NOTE X:	It is up to the conclusion of UPEAS SID on whether SBI and/or PFCP (TS 29.244 [8]) is used for interaction between the SMF and UPF.

5.	The UPF responds with the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the NWDAF.

6a.	For case 2, the SMF keeps monitoring to update UE list 1 by invoking Namf_EventExposure_Subscribe with Event "UE moving in or out of Area of Interest".

6b.	The SMF determines the updated UE list 2 for user plane date reporting same as step 4c with the result from AMF from step 6a.

6c.	The SMF sends the request to the UPF over Session Modification Request/Response message including the Target of Event Reporting which is the updated UE list from step 6b. And the UPF responds to SMF same as step 5.

END of CHANGES