3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208849

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	CATT

Title:	KI#1, Conclusion

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract: A conclusion for KI#1 is proposed.

1. Introduction/Discussion

This pCR provides a structure for the conclusions.

In RAN3#117e, the following agreement was reached :

There is no need to provide the ephemeris info over S1.

Therefore, it is recommended that the CN (AMF/MME) obtains satellite coverage information (e.g. satellite ephemeris) from AF (or a 3rd party server via NEF) or OAM.

From the perspective of minimizing the impact on RAN and CN, the following conclusions are proposed:

- AMF/MME obtains satellite coverage information (e.g. satellite ephemeris) from AF (or a 3rd party server via NEF) or OAM.

- UE obtains satellite ephemeris data relies on broadcast in a SIB as defined in TS 36.331.

- AMF/MME determines the UEs mobility management based on the obtained UE information (e.g. location, mobility) and coverage information (e.g. ephemeris data)

- Reuse the disaster roaming wait range configuration of the MINT work item to reduce the impact to target RAT/PLMN.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-28.

# * * * * First change * * * *

8	Conclusions

8.x	Conclusion on general mobility management with discontinuous satellite coverage

Following conclusions are proposed:

- 	AMF/MME obtains satellite coverage information (e.g. satellite ephemeris) from AF (or a 3rd party server via NEF) or OAM.

-	UE obtains satellite ephemeris data relies on broadcast in a SIB as defined in TS 36.331.

-	AMF/MME determines the UEs mobility management based on the obtained UE information (e.g. location, mobility) and coverage information (e.g. ephemeris data)

-	Reuse the disaster roaming wait range configuration of the MINT work item to reduce the impact to target RAT/PLMN.

Note:	Whether the UE can be in CM-connected with RRC inactive and the AN release procedure is needed when there is no satellite coverage can be discussed during the normative phase.

# * * * * End of changes * * * *