3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208850

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	CATT

Title:	KI#2, Conclusion

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract: A conclusion for KI#2 is proposed.

1. Introduction/Discussion

This pCR provides a structure for the conclusions.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-28.

# * * * * First change * * * *

8	Conclusions

8.x	Conclusion on Power saving enhancement for UE in discontinuous coverage

Following conclusions are proposed:

- AMF/MME determines the power saving parameters based on the obtained UE information (e.g. location, mobility) and coverage information (e.g. ephemeris data) for the UEs

- AMF updates the power saving parameters to the UE during the UCU procedure

- During the unavailability period, the AMF or MME maintains the UE context in CM-IDLE or ECM-IDLE.

FFS: Whether the UE can be in the in-active state can be discussion in the future.

# * * * * End of changes * * * *