3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208881

Elbonia, October 10 – 17, 2022	(revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation and conclusion for KI#5: Improved support of RAs including TAs supporting Rejected S-NSSAIs

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This contribution proposes the evaluation and conclusion of KI#5 Improved support of RAs

# 1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change  * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.X	Evaluation for KI#5

The related solutions are #11, 23, 25, 26, 27, 28, 29, 30, 31.

Category 1 is based on allowed S-NSSAI that is not fully supported in RA (hence Partially allowed), where AMF provides conditional allowed S-NSSAI with its support TA to UE (solution 11, 23, 25, 26, 29). This does not require the UE to register with the AMF the Partially allowed S-NSSAI when the UE enters one of the TAIs where the partially rejected s-NSSAI is supported

Category 2 is based on rejected S-NSSAI that however are supported in some TAI of the RA (hence Partially Rejected), where AMF provides a partially rejected S-NSSAI with TA(s) where it is supported in the RA to UE (solution 25, 27, 28, 29, 30, 31). This requires the UE to register with the AMF the Partially Rejected S-NSSAI if the UE is still interested in the s-NSSAI when it enters one of the TAIs where the partially rejected s-NSSAI is supported.

Table 7.X-1: Evaluation of KI#5 related principles

|  | Category 1: | Category 2: |
| --- | --- | --- |
| Pros | - Does not require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA - PDU sessions need not be re-established and disconnected (see solution 29) - In MO case the UE can avoid causing MO access attempts for the slices that are not working in current TA. . | - Reuse most of the current procedures. (only Registration procedure is impacted) - Aligned with the current slice management features, where the network determines and updates UE on the Allowed NSSAI, rejected S-NSSAIs, target NSSAI and so on. |
| Cons | - NSAC, will be inaccurate - Has impact on Homogenous slice support in RA. AMF in connected mode can detect when the connectivity of a slice is available or not, but in idle mode the MT services cannot assume where in the RA the UE is so the AMF would page in the RA. | - require UE to perform registration update for using a requested S-NSSAI not supported in the previous TA. This may be heavy if the NSSAA applies. - requires releasing and re-establishing sessions at mobility in an out the areas where the partially rejected slcie slice is supported. this may be heavy if secondary DN authentication is needed. - when entering the area of support of partially rejected S-NSSAI, the RA size may need to be reduced to just the TAIs supporting the Rejected S-NSSAI in the previous RA. |

# * * * * Second change* * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.X	Conclusions for Key Issue #5

The following principle is proposed to be the conclusion for normative work:

-	AMF provides information related to the supported or not supported TA(s) regarding a rejected S-NSSAI in the RA to UE in the registration procedure and UE configuration procedure if the UE indicates it supports this feature. The UE shall be able to request a rejected S-NSSAI in a supported TA based on the supported/not supported TA information. A UE receiving this information shall trigger registration update when it requests a rejected S-NSSAI in a supported TA. The current concept of allowed NSSAI and uniform support of it in UE’s RA is not impacted by this feature based on indication of where in RA certain rejected S-NSSAIs are supported/not supported. In this approach, the Allowed NSSAI is still uniformly supported in the RA.

-	if NSAC does not apply, or if the operator is happy to use this feature despite NSAC includes UEs or sessions for UEs that are outside the area where a certain S-NSSAI is supported while still in the RA (so maybe NSAC can be inaccurate), it should be possible for the AMF (for supporting UEs) to indicate to the UE that some S-NSSAIs are partially allowed in the RA by indicating the TAs where these are supported or not supported. If so, the UE assumes it can use the connectivity for the slices in the TAs where it is indicated to be supported. The connected mode behaviour can be based on sol.29 principle. Whether the partially allowed NSSAI can be used for redirection or Handover of the UE is to be based on RAN feedback. In this approach, the Allowed NSSAI is still uniformly supported in the RA but the Partially Allowed S-NSSAI is not uniformly supported in the RA.

-	whether to apply one of the two options is a per S-NSSAI decision based on AMF policy, both options can be supported in a PLMN and can be applied simultaneously for one UE for different S-NSSAIs.

# * * * * End of changes * * * *