3GPP TSG-WG SA2 Meeting #153E e-meeting 	S2-2208884

Elbonia, Oct 10th – 14th, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI#10 Evaluation and Conclusion

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes evaluation and conclusion for KI#10.

1. Background

This paper proposes evaluation and conclusion on the key aspects of the Interactions with MDAS/MDAF discussed in solutions adopted in the TR for Key Issue 10.

2. Text Proposal

It is proposed to adopt the following text within the TR23.700-81.

# * * * * First change * * * *

7	Overall Evaluation

7.10	Interactions with MDAS/MDAF

Solutions #60 and solutions #72-#74 are proposed for Key Issue #10 "Interactions with MDAS/MDAF".

All proposed solutions can be categorized in the following 2 categories:

-	Use cases for NWDAF to leverage MDAS/MDAF output (Sol #60, Sol #72).

-	Procedure for MDAF/MDAS discovery and subscription (Sol #73, Sol #74).

In the following each category will be evaluated separately.

Solution #60 introduces two use cases for NWDAF to leverage MDAS/MDAF output as extra input to produce the NWDAF analytics, the observed service experience analytics by collecting additional service experience and energy saving analysis information from MDAS/MDAF, and the Redundant Transmission Experience analytics by collecting additional E2E latency analysis from MDAS/MDAF.

Solution #72 introduces the use case for NWDAF to improve the analytics accuracy or re-train ML model by leveraging the output from MDAS/MDAF fault management predictions/statistics, and also introduces the procedures for AnLF and MTLF subscribing from the MDAS/MDAF for this case.

Solution #73 and Solution #74 introduce interactions between NWDAF and MDAS/MDAF, including both discovery and subscription procedures. The NWDAF discovers the MDA Management Function from the MnS Discovery service producer by sending a MnS producer discovery service operation, and the NWDAF subscribes to MDAS/MDAF for analysis by MDA Request message. In Sol #74, the definition of attributes in the subscription request are also introduced, and also introduce when consumer issues a analytics request, how the NWDAF collects information from both MDAS/MDAF and other data sources.

# * * * * Next change * * * *

8	Conclusions

8.10	Interactions with MDAS/MDAF

- NWDAF uses MDAS/MDAF analysis as extra input for generating analytics, e.g., Oberseved Service experience analytics and Redundant Transmission Experience analytics, improving accuracy or re-training ML model, Solutions #60 and #72 are adopted as baseline for normative work.

- NWDAF, as an MDA MnS Consumer, reuse the MDA functional overview and service framework as defined in TS 28.104 [15]. NWDAF discover MDAS/MDAF by MnS Discovery service producer, and subscribes to MDAS/MDAF by MDA Request service operation, Solutions #74 is adopted as baseline for normative work.

# * * * * End of changes * * * *